|  | ANGLO-AUSTRALIAN OBSERVATORY P.O. BOX 296 EPPING N.S.W. 1710 AUSTRALIA Telephone: +61 2 9372 4800 Fax: +61 2 9372 4880 |
| --- | --- |

- FLAMES FIBRE POSITIONER

| Document Number: | INS-SPE-AUS-13721-0071 |
| --- | --- |
| Issue Number: | 1.1 |
| Issue Date: | 08 May 2001 |

Astrometric Model and Associated Software

| Prepared: | Tony Farrell, Jeremy Bailey | 08 May 2001 |  |
| --- | --- | --- | --- |
|  | Name | Date | Signature |
| Approved: | Tony Farrell | 21-Feb-2003 |  |
|  | Name | Date | Signature |
| Released: | Chris Evans | 21-Feb-2003 |  |
|  | Name | Date | Signature |

- Change Record

| Issue / Revision | Date | Section / Page affected | Reason / Remarks |
| --- | --- | --- | --- |
| 1.0 | 09/04/2001 | All | First Issue |
| 1.1 | 08/05/2001 | 5 | Add updated routine descriptions |
| 1.2 | 21/02/2003 | Cover | TJF. Signed Document. Note – as this was a planing document it does not reflect the current software status. This document should be used as an introduction only to the ideas, rather then the current specification. |

- Fibre Positioner

- Astrometric Model and Associated Software

1	The Astrometric Model	6

1.1	Fixed Parameters	6

1.2	Parameters to be determined for each observation	6

1.3	Parameters to be determined during commissioning	7

1.4	Positioner Effects	7

2	Coordinate Systems	7

3	Using the fpmdl Routines	8

3.1	Model Parameters	9

3.2	SDS files	9

3.3	The Position Angle	10

4	Notes on Implementation of the Astrometric Model	10

5	fpmdl Routines	11

5.1	fpmdlCreateModel	11

5.2	fpmdlCvtInit	12

5.3	fpmdlFree	13

5.4	fpmdlGetModel	13

5.5	fpmdlPos2Xy	14

5.6	fpmdlQuickAppToObs	15

5.7	fpmdlRd2Xy	16

5.8	fpmdlSetModel	17

5.9	fpmdlTransAccess	18

5.10	fpmdlWriteModel	19

5.11	fpmdlXy2Pos	20

- Fibre Positioner

- Astrometric Model and Associated Software

# The Astrometric Model

The astrometric model allows the coordinates on the field plate to be determined given the RA and Dec of a source and the RA and Dec of the field centre. This model has to be used in various places in the OzPos software system.

- In the Fibre Positioner Software.

- In the Configuration Software.

- In the Calibration software used to refine the model during commissioning.

The astrometric model is implemented in the C module fpmdl.c and associated include file fpmdl.h.

The model requires a number of parameters. Some of these will be fixed and should be known in advance, some will need to have their precise values established during commissioning, and some will vary for each observation. Examples of each type are as follows:

## Fixed Parameters

These will include observatory and telescope parameters such as the latitude, longitude and height above sea level of the telescope. The software obtains these values using the slalib routine slaObs using the telescope name “VLT2”.

## Parameters to be determined for each observation

These are the parameters needed for refraction calculations and include the time of observation, atmospheric temperature, pressure, humidity and observing wavelength (note that there may be different observing wavelengths for different types of fibres). Strictly speaking the temperature etc. should be predicted values for the mid-point of an observation. In practice, however, the sensitivity to these value is such that the difficulty of making an accurate prediction should not be a problem.

The values of these parameters are set by a call to fpmdlCvtInit.

## Parameters to be determined during commissioning

Parameters of this type are those that determine the geometric relationship of the field plate and sky, such as image scale, zero point offsets and distortion model parameters. We should have reasonably good starting values for these but these will be refined during commissioning.

These parameters can be set in two different ways. Firstly using the routine fpmdlTransAccess which obtains the information from an SDS file. This method is used in the configuration software. Alternatively the routine fpmdlCreateModel can be used to set the parameter values explicitly. The Positioner software uses this method as it keeps the parameter values in its own database.

## Positioner Effects

It is not the job of the astrometric model to correct for deficiencies in the positioner itself. These could be effects due to non-linearities in the encoders, geometrical misalignments in the positioner robot, thermal expansion of the field plate or robot components. It is assumed that all such effects are calibrated and corrected by internal tests within the positioner, which do not require observing stars.

# Coordinate Systems

The Astrometric Model converts RA and Dec to the positioner XY coordinate system. X and Y are related to the R axes of the positioner by:

X = R cos and   Y = R sin 

This is illustrated in the following diagram:

View of the field plate looking from M3 towards OzPos. The +Y axis is straight upwards as the plate is attached to the VLT rotator at its home position. The VLT rotator will then be used to position the +Y axis at a required position angle for the observation (ypa). If ypa = 0 then North is in the +Y direction, East is in the –X direction.

# Using the fpmdl Routines

The fpmdl package implements an object of type FpilModelType which should only ever be accessed through the fpmdl routines (or other routines which call fpmdl such as the FpilModel routines).

The routines should be used in the following sequence:

- Create a new model using either fpmdlTransAccess which creates a model based on an sds file, or fpmdlCreateModel which creates a model using explicit parameter values.

- Use the routine fpmdlCvtInit to initialise the model to do conversions for a particular time, wavelength, atmospheric parameters and field plate position angle.

- Use the routines fpmdlRd2Xy and fpmdlXy2Pos to do conversions from  Ra,Dec to XY, or the routines fpmdlPos2Xy and fpmdlXy2Rd to do the inverse transformation.

- Repeat steps 2 and 3 to do a new set of transformations for a new set of parameters.

- When finished used fpmdlFree to delete the model structure.

There are two additional routines which are needed to support the calibration software. A model can be written to an sds file (for subsequent use by fpmdlTransAccess) using fpmdlWriteModel. The parameter values for a model can be accessed using fpmdlGetModel.

## Model Parameters

There are currently seven parameters of the model in the pars array which is passed to fpmdlCreateModel or returned by fpmdlGetModel. The first six consist of a six parameter linear model which incorporates small corrections to the zero point, scale and rotation in the form:

x’  =  a + bx + cy

y’  =  d + ex + fy

Thus  pars[0] = a,  pars[1] = b etc.

The seventh parameter (pars[6]) is the radial distortion coefficient as used in the slalib routine slaPcd – i.e.  the corefficient c in  r’ = r(1 + cr2).

It is these parameters which will need to be refined during commissioning to accurately reflect the actual system. If more parameters prove to be necessary they can easily be added without changing the calling sequence of the routines.

Note that there should be a set of these seven parameters for each field plate.

## SDS files

If fpmdlTransAccess is used the above parameters are read from an sds file. There will be one of these files for each field plate. This file has the name  OzPosModel1.sds for field plate 1 or OzPosModel2.sds for field plate 2.

The directory to obtain this file from is specified using the dirOverride and dirFallback arguments.

## The Position Angle

The software must be given the sky position angle at which the rotator is going to place the positioner y axis (the value ypa as specified in section 2).

This value in radians is given in the params[0] argument of fpmdlCvtInit. This is the only component of params used for OzPos.

# Notes on Implementation of the Astrometric Model

The astrometric model is implemented in the following way using routines from Pat Wallace’s slalib package:

- Given a J2000 position of a source convert this to an apparent position for the time of observation using slaMap.  The parameters required for this step are the TDB modified Julian date of observation (UTC is probably an adequate approximation), and the proper motions if available. This stage is not done by the fpmdl routines which work from apparent positions.

- Convert the apparent position to an observed position using slaAopqk. This gives the observed (i.e. position as altered by refraction) RA and Dec of the source, use the same two steps to get the observed RA and Dec of the field centre as well.

- Obtain the tangent plane coordinates (eta, xi) using the routine slaDs2tp.

- Flip the sign of xi to give the east negative convention required for the positioner.

- Rotate xi and eta through ypa.

- Correct for distortion – if a simple radial distortion model is adequate this can be done with the routine slaPcd.

- Multiply by the focal length (in mm) to get x,y coordinates at the focal plane in mm.

- Apply a linear model using routine slaPxy to allow for zero point, scale and rotation corrections.

Notes

For an instrument with a smaller field it would be sufficient to use slaDs2tp directly on the J2000 RA and Dec. This dramatically decreases the number of parameters required. However, this would not allow for differential refraction effects and so is not suitable for the wide field of OzPos.

2dF uses a more complicated procedure which takes account of the field rotation consequences of the misalignment terms in the telescope pointing model. I don’t think this should be necessary for OzPos as these effects will be incorporated into the positioning of the rotator by the telescope control system and so don’t need to be introduced into the positioning of the fibres..

The fpmdl routines implement these operations in two stages. fpmdlRd2Xy implements steps 2 to 7. fpmdlXy2Pos implements step 8 which should only involve the small corrections which will be determined during commissioning. The splitting of the operation into these two stages exists largely for historical reasons. In practice a call to fpmdlRd2Xy will be immediately followed by a call to fpmdlXy2Pos.

# fpmdl Routines

## fpmdlCreateModel

Function:Create an OzPos transformation model from explicit parameters.

Description:This routine initialises access to the OzPos transformation details. It is used by the OzPos positioner software in place of fpmdlTransAccess to allow explicitly setting the model parameters from values in the OzPos database.

This routine should access the model and malloc and return the address of a structure of type FpilModelType.  By actually mallocing a different structure (of type OzPosModelType) which contains as its first item a structure of type FpilModelType, the OzPos telescope model implementation routines can keep there own information in this structure.  It just casts to and from FpilModelType as required (this can be considered deriving from FpilModelType, in the O-O sense).

Language:C

Call:

(void) = fpmdlCreateModel(npars, pars, field, model,  status)

Parameters:(“>” input, “!” modified, “W” workspace, “<” output)

(>)	npars	(int)	Number of parameters

(>)	pars	(double *)	An array of parameters

(>)	field	(unsigned int)	The field plate number.

(<)	model	(FpilModelType **)	The address of the model is returned here.

(!)	status	(StatusType *)	modified status.

Include files:fpmdl.h

## fpmdlCvtInit

Function:Initialise for conversions at a specified time etc.

Description:This routine initialises the telescope model conversion routines for a particular time, wavelength and telescope model details.  This allows a sequence of transformation for one field to be done efficently.

This routine is called by FpilModelCvtInit() and must meet the prototype given by the type defintion FpilModelCvtInitRoutineType.

Language:C

Call:

(void) = fpmdlCvtInit(model,  mjd, dut, temp, pres, humid, wave, tel, status)

Parameters:(“>” input, “!” modified, “W” workspace, “<” output)

(>)	model	(FpilModelType *model)	Contains the telescope model details obtained by fpmdlTransAccess.  This routine may or may not have been previously invoked on this model.

(>)	mjd	(double)	UTC data and time as a modified julian date.

(>)	dut	(double)	Delta UT (UT1 - UTC) seconds.

(>)	temp	(double)	Atmospheric temperature (K).

(>)	press	(double)	Atmospheric pressure (mB).

(>)	humid	(double)	Atmospheric humidity (0 to 1.0).

(>)	wave	(double)	Wavelength of observation (micons).

(>)	params	(double *)	Model parameters. For OzPos only params[0] is used and is the sky position angle of the positioner y axis in radians.

(!)	status	(StatusType *)	modified status.

Returned value:

Include files:fpmdl.h

## fpmdlFree

Function:Free the OzPos positioner transformation model structure.

Description:Free and tidy up after a call to fpmdlTransAccess(3).

Language:C

Call:

(void) = fpmdl(model,status)

Parameters:(“>” input, “!” modified, “W” workspace, “<” output)

(>)	model	(FpilModelType *model)	Contains the telescope model details obtained by fpmdlTransAccess(3).  The fpmdlCvtInit routine may or may not have been previously invoked on this model.

(!)	status	(StatusType *)	modified status.

Include files:fpmdl.h

## fpmdlGetModel

Function:Return the parameters of a model

Description:This routine returns an array containing the parameters of a model in the same format as that given to fpmdlCreateModel.

Language:C

Call:

(void) = fpmdlGetModel(model, npars, pars, status)

Parameters:(“>” input, “!” modified, “W” workspace, “<” output)

(>)	model	(FpilModelType *model)	Contains the telescope model details obtained by fpmdlTransAccess or fpmdlCreateModel.

(>)	npars	(int)	Size of pars array.

(<)	pars	(double *)	Array to receive model parameters.

(!)	status	(StatusType *)	modified status.

Returned value:

Include files:fpmdl.h

## fpmdlPos2Xy

Function:Convert positioner coorindates to field plate coordinates.

Description:Convert the x,y coorindates of a star on the coorindate system used by the OzPos positioner task to the x,y coordinates on the field plate as used by fpmdlXy2Rd.

This scheme of two x/y coorindate systems allows the robot to use a coordinate system which makes sense mechanically, whilst the values output by fpmdlRd2Xy make more sense from an astromical point of view.  This scheme also takes account of non-linearities etc. in the positioner system.

This routine is called by FpilModelPos2Xy() and must meet the prototype given by the type defintion FpilModelPos2XyRoutineType.

Language:C

Call:

(void) = fpmdlPos2Xy(model,  xp, yp, x, y, status)

Parameters:(“>” input, “!” modified, “W” workspace, “<” output)

(>)	model	(const FpilModelType *model)	Contains the telescope model details  obtained by fpmdlTransAccess(3).

(>)	xp	(double)	Positioenr X coordinate of object (microns).

(>)	yp	(double)	Positioenr Y coordinate of object (microns).

(>)	*x	(double)	Field plate x coordinate of object (microns).

(>)	*y	(double)	Field plate y coordinate of object (microns).

(!)	status	(StatusType *)	modified status.

Include files:fpmdl.h

## fpmdlQuickAppToObs

Function:Perform a quick apparent to observered place conversion.

Description:Perform a quick apparent to observered place conversion.

Language:C

Call:

(void) = fpmdlQuickAppToObs(model, ra, dec, aob, zob, hob, dob, rob, status)

Parameters:(“>” input, “!” modified, “W” workspace, “<” output)

(>)	model	(const FpilModelType *model)	Contains the telescope model details obtained by FpilModelAccess(3) and on which FpilModelCvtInit(3) has been invoked.

(>)	ra	(double)	Apparent center RA in radians.

(>)	dec	(double)	Apparent Center Declination in radians.

(<)	aob	(double *)	Observed azimuth (radians, N=0, E = 90degrees)

(<)	zob	(double *)	Observed zenith distance (radians)

(<)	hob	(double *)	Observed Hour Angle (radians)

(<)	dob	(double *)	Observed Declination (radians)

(<)	rob	(double *)	Observed Right Ascension (radians)

(!)	status	(StatusType *)	modified status.

Include files:fpil.h

## fpmdlRd2Xy

Function:Determine OzPos x/y from apparent RA and Dec.

Description:Determines the x,y coordinates in the field plate, from the apparent RA and Dec of a source and the field center.

This routine is called by FpilModelRd2Xy() and must meet the prototype given by the type defintion FpilModelRd2XyRoutineType.

Language:C

Call:

(void) = fpmdlRd2Xy(model, cra, cdec, ra, dec, mjd, x, y, status)

Parameters:(“>” input, “!” modified, “W” workspace, “<” output)

(>)	model	(const FpilModelType *model)	Contains the telescope model details  obtained by fpmdlTransAccess(3). It Can be assumed that the fpmdlCvtInit routine has been  invoked on this.

(>)	cra	(double)	Apparent RA of field center.

(>)	cdec	(double)	Apparent DEC of field center.

(>)	ra	(double)	Apparent RA of source.

(>)	dec	(double)	Apparent DEC of source.

(>)	mjd	(double)	UTC time and date expressed as Modified Julian Date

(<)	x		(double *)	X position of source on field plate (microns).

(<)	y		(double *)	Y position of source on field plate (microns).

(!)	status	(StatusType *)	modified status.

Include files:fpmdl.h

## fpmdlSetModel

Function:Set the parameters of a model

Description:This routine is given an array containing the parameters of a model in the same format as that given to fpmdlCreateModel.

Language:C

Call:

(void) = fpmdlSetModel(model, npars, pars, status)

Parameters:(“>” input, “!” modified, “W” workspace, “<” output)

(>)	model	(FpilModelType *model)	Contains the telescope model details obtained by fpmdlTransAccess or fpmdlCreateModel.

(>)	npars	(int)	Size of pars array.

(>)	pars	(double *)	Array containing model parameters.

(!)	status	(StatusType *)	modified status.

Returned value:

Include files:fpmdl.h

## fpmdlTransAccess

Function:Access the OzPos transformation details.

Description:This routine initialises access to the OzPos transformation details.  It is called by FpilModelAccess() and must fit  the prototype type defintion FpilModelAccessRoutineType

This routine should access the model and malloc and return the address of a structure of type FpilModelType.  By actually mallocing a different structure (of type OzPosModelType) which contains as its first item a structure of type FpilModelType, the OzPos telescope model implementation routines can keep there own information in this structure.  It just casts to and from FpilModelType as required (this can be considered deriving from FpilModelType, in the O-O sense).

See FpilModelAccess for details on the usage of the directory releated arguments.

Language:C

Call:

(void) = fpmdlTransAccesss(clientData, dirOverride, dirFallback, RptRoutine, RptArg,  field, model,  status)

Parameters:(“>” input, “!” modified, “W” workspace, “<” output)

(>)	clientData	(void *)	Was passed to FpilInit.

(>)	dirOverride	(const char *)	If non-null, a directory to search for the distortion details file.

(>)	dirFallback	(const char *)	If non-null, a fall back directory to search for the distortion details file.

(>)	RptRoutine	(FpilRptRoutineType)	A routine to be invoked to report which file has been opened.

(>)	RptArg	(void *)	Argument passed to RptRoutine.

(>)	field	(unsigned int)	The field plate number.

(<)	model	(FpilModelType **)	The address of the model is returned here.

(!)	status	(StatusType *)	modified status.

Include files:fpmdl.h

## fpmdlWriteModel

Function:Write model to an SDS file

Description:This routine writes the model to an SDS file of suitable format for picking up by fpmdlTransAccess. For OzPos this is a file of name OzPosModeln.sds for field plate n.

Language:C

Call:

(void) = fpmdlWriteModel(model, dirname, status)

Parameters:(“>” input, “!” modified, “W” workspace, “<” output)

(>)	model	(FpilModelType *model)	Contains the telescope model details obtained by fpmdlTransAccess or fpmdlCreateModel.

(>)	dirname	(char*)	Name of directory into which file will be written.

(!)	status	(StatusType *)	modified status.

Returned value:

Include files:fpmdl.h

## fpmdlXy2Pos

Function:Convert OzPos field plate coordinates to positioner coordinates.

Description:Convert the x/y coordinates of a star on the field plate, as output by fpmdlRd2Xy to the coordinates required for input to the OzPos robot positioner system.

This scheme of two x/y coorindate systems allows the robot to use a coordinate system which makes sense mechanically, whilst the values output by fpmdlRd2Xy make more sense from an astromical point of view.  This scheme also takes account of non-linearities etc. in the positioner system).

This routine is called by FpilModelXy2Ps() and must meet the prototype given by the type defintion FpilModelXy2PosRoutineType.

Language:C

Call:

(void) = fpmdlXy2Pos(model, x, y, xp, yp, status)

Parameters:(“>” input, “!” modified, “W” workspace, “<” output)

(>)	model	(const FpilModelType *model)	Contains the telescope model details  obtained by fpmdlTransAccess(3). It Can be assumed that the fpmdlCvtInit routine has been invoked on this.

(>)	x		(double)	Field plate x coordinate of object (microns).

(>)	y		(double)	Field plate y coordinate of object (microns).

(>)	*xp	(double)	Positioner X coordinate of object (microns).

(>)	*yp	(double)	Positioner Y coordinate of object (microns).

(!)	status	(StatusType *)	modified status.

Include files:fpmdl.h

A.12 fpmdlXy2Rd

Function:Determine apparent RA and Dec from OzPos x and y.

Description:Determine the apparent RA an Dec of a source from the x,y coorindate in the field plate and the field center.

This routine is called by FpilModelXy2Rd() and must meet the prototype given by the type defintion FpilModelXy2RdRoutineType.

Language:C

Call:

(void) = fpmdlXy2Rd(model, cra, cdec, x, y, mjd, ra, dec, status)

Parameters:(“>” input, “!” modified, “W” workspace, “<” output)

(>)	model	(const FpilModelType *model)	Contains the telescope model details  obtained by fpmdlTransAccess(3). It Can be assumed that the fpmdlCvtInit routine has been invoked on this.

(>)	cra	(double)	Apparent RA of field center.

(>)	cdec	(double)	Apparent DEC of field center.

(>)	x		(double)	X position of source on field plate (microns).

(>)	y		(double)	Y position of source on field plate (microns).

(>)	mjd	(double)	UTC time and date expressed as Modified Julian Date

(<)	ra		(double *)	Apparent RA of source.

(<)	dec	(double *)	Apparent DEC of source.

(!)	status	(StatusType *)	modified status.

Include files:fpmdl.h

___oOo___