|  | ANGLO-AUSTRALIAN OBSERVATORY AAO 2dF Software Report 28 Version 2.0.20.4 |
| --- | --- |

Keith Shortridge, Tony Farrell

19 April, 200025 February, 2000

Fibre Instrument Configuration Algorithm Routines

Contents

1	Introduction	4

2	External Interface Specification	5

2.1	Reporting Progress	5

2.2	Initialisation	5

2.3	Closing Down	6

2.4	Setting up a target field	6

2.5	Performing a complete configuration	6

2.6	Controlling allocation options	6

2.6.1	Config library parameters.	7

2.7	Field validity checks.	7

2.8	Adding and selecting new allocation algorithms.	8

2.9	Determine which pivots can be allocated	8

2.10	ImportMode stuff.	8

2.11	Detailed external interface specifications	8

2.11.1	ConfigInitialise()	8

2.11.2	ConfigClose()	9

2.11.3	ReportFunction()	10

2.11.4	AllocRptFunction()	12

2.11.5	InfoRoutine()	13

2.11.6	ConfigSetTargets()	14

2.11.7	ConfigAllocate()	15

2.11.8	ConfigSetParameter()	16

2.11.9	ConfigNumberParameters()	16

2.11.10	ConfigParameterDetails()	17

2.11.11	ConfigGetParameter()	18

2.11.12	ConfigSetImportFile()	18

2.11.13	ConfigSetImportOnlyMode()	19

2.11.14	ConfigRegisterMethod()	19

2.11.15	ConfigSelectMethod()	20

2.11.16	ConfigNumMethods()	20

2.11.17	ConfigGetMethodName()	21

2.11.18	ConfigFieldCheck()	22

2.11.19	ConfigSetPivotsFlag()	22

2.11.20	ConfigSetSpecPivotsFlag()	23

2.12	Using ConfigParameterDetails()	23

3	Writing a new allocation algorithm	25

3.1	Overview	26

3.2	Structures and types used	28

3.2.1	The CONFIG_MethodDetails structure	29

3.2.2	The CONFIG_MethodInfo structure	29

3.2.3	The CONFIG_PivotInfo structure	31

3.2.4	The CONFIG_TargetInfo structure	32

3.2.5	The CONFIG_AllocInfo structure	34

3.3	Algorithm-specific routine specifications	36

3.3.1	GlobalInit()	36

3.3.2	ReleaseMem()	36

3.3.3	Init()	37

3.3.4	Allocate()	38

3.3.5	NumberParams()	39

3.3.6	ParamDetails()	39

3.3.7	SetParam()	40

3.3.8	GetParam()	41

3.3.9	SetImportFile()	42

3.4	Utility routines	43

3.4.1	MaxExtension()	43

3.4.2	GetVirtPiv()	44

3.4.3	ThetaFib()	44

3.4.4	CheckAlpha()	45

3.4.5	CheckAllocation()	46

3.4.6	FieldCheck()	47

3.4.7	NoteAllocation()	48

3.4.8	TentativeFix()	49

3.4.9	NoteDeallocation()	50

3.4.10	NoteDeallocationBroken()	51

3.4.11	ButtonClear()	51

3.4.12	FibreClear()	52

3.4.13	MaxButAngle()	52

3.4.14	MaxPivAngle()	53

3.4.15	SafeClearance()	54

3.4.16	ColFibFib()	54

3.4.17	ColButFib()	55

3.4.18	ColButBut()	56

3.4.19	Collision()	57

3.4.20	ColInvPos()	58

3.4.21	ParkMayCollide()	58

4	Detailed description of the problem	59

4.1	The physical parts of the 2dF	59

4.2	Covering the angles	60

4.3	Allocation constraints	61

5	The default algorithm	63

5.1	Tweaking the current algorithm	64

5.2	Gavin’s modifications	64

Revisions:

24-Feb-2000	TJF - Update styles etc. for word 97/98.  No change in contents.

25-Feb-2000	TJF – Reformat using the AAO Software Template.

03-Mar-2000	TJF – Add previously undocumented routines ConfigSetImportFile(), ConfigSetImportOnlyMode(), ConfigMethodSetImportFile().  Document name change of Config.h to tdFconfig.h.

03-Mar-2000	TJF – Add ConfigRegisterMethod() and ConfigSelectMethod().  Move ConfigFieldCheck() into external interface section.

06-Mar-2000	TJF – Revamp section 3 to provide for calling of all routines via pointer such that the algorithm method does not need to be linked with routines of specified names

07-Mar-2000	TJF – Add NoteDeallocationBoroken() routine..Add InfoRoutine to ConfigInitialise and to MethodInfo structure.  Add collision detection routines.

07-Mar-2000	TJF – Now supports generic fibre instruments.  Document revamped accordingly.  Remove ConfigAllocateFibre(), ConfigDeallocateFibre() and ConfigVerify(), they have never been implemented and are not currently used.

10-Mar-2000	TJF – Add section on Gavin’s algorithm based on details from the 2dF web page.

27-Mar-2000	TJF – Add ConfigSetPivotsFlag() and ConfigSetSpecPivotsFlag() functions and AllowAllocChange item to PivotInfo structure.  Add AllocReport function argument to ConfigAllocate().

31-Mar-2000	TJF – Add TentativeFix callback routine and support tentative allocations.

19-Apr-2000	TJF – Add ParkX and ParkY to PivotInfo structure.  Add ConfigCollisionRoutine to MethodInfo structure.

# Introduction

This document describes routines written to perform the automatic allocation of fibres to targets for a fibre positioner instrument, such as the AAO’s 2dF and 6dF instruments. This operation is generally referred to as ‘configuration’.

This library started out as a library specific to the AAO’s 2dF instrument.  It has, as of about March 2000, been revamped to support any number of instruments and specific support is currently provided for the AAO’s 2dF and 6dF instruments.  Some parts of this document and the underlying software may sometimes still refer to 2dF, but unless it is obvious from the context that in that case 2dF is being considered separately, you should read such instances as referring to any fibre instrument.

This library is written to allow the easy generation of different user interfaces to the configuration scheme, largely for historical reasons.  The code that calls these routines is referred to in this document as the ‘external code’ has to be provided separately and this document not concerned with its details (a current, and so far the only, example of this is the AAO’s “configure” program).

Since the question of the best actual allocation algorithm to use is still moot and may change depending on the particular requirements of the astronomer, this library does not provide the allocation algorithm itself.  Instead it provides ‘housekeeping’ routines interfacing between the ‘external code and the algorithm, allowing the appropriate algorithm to be selected as desired.  (It does through provide a default algorithm, which most people use.).

The idea is that the ‘algorithm specific’ routines  have a straightforward interface. This allows anyone who wants to try to modify the algorithm used or provide an entirely different algorithm can do so without having to worry about how the pivot positions are defined, the details of the way the targets are presented to the program, etc.  Depending on the design of the ‘external code, the ‘algorithm specific code’ may be stored in shared libraries and loaded dynamically as desired.  You can switch between ‘algorithms’ at run time.  The library includes a default allocation algorithm.  The only structural difference between the default algorithm and any other algorithm is that the default is loaded when the library is initialised, where as other algorithm’s must be purpose loaded.

This may become clearer if we look at the code including the default algorithm. The default algorithm has a set of routines that maintain of a matrix of all possible fibre-target combinations, in which the elements show whether or not the corresponding allocation of fibre to pivot is possible given the allocations that have already been made. So this is a (Number of Targets) by (Number of Pivots) matrix and it has to be updated each time an allocation is made. At the heart of this method-specific code is a routine that looks at this matrix and, roughly speaking, allocates the pivot that can reach the fewest targets to one of the targets it can reach. In this case, it obviously helps to be able to count the number of targets a pivot can reach by just counting along the rows of a matrix. Other algorithms may not find this matrix a useful concept. This algorithm clearly has a definite overall structure (based on constructing the matrix to make it easy to locate the pivot that can reach the fewest targets) and some details of implementation (such as how it decides which target to pick once it has decided which pivot to allocate next). Changing the details would be simple, and may involve little more than a few well-chosen changes to one routine. Changing the overall approach might well mean abandoning all the algorithm-specific code. However, in neither case should it be necessary to modify the ‘housekeeping’ routines.

This document first describes the overall interface to the allocation subroutine library (which is all calls to routines in the ‘housekeeping and external interface’ layer). Then it describes the interface between that layer and the method-specific routines. So this document is a full-specification of the configuration library as it appears to the external code, and should be used by anyone writing a program that is to call the configuration routines. It is also a full specification that should be used by anyone wanting to write a completely new set of algorithm-specific routines. It also describes the standard allocation algorithm used, to try to make it easier for people to make minor changes to the way it works.

# External Interface Specification

This section describes the various routines that are intended to be called from outside the allocation package. An overview of the various functions that have to be performed is followed by detailed specifications for each of the routines involved.

## Reporting Progress

In many cases, the allocation interface routines have a ReportFunction argument. This is supplied in all cases where the operation performed by the routine in question can be time consuming. In such cases, a report function allows the progress of the operation to be reported, for example to a user-interface, and also provides a means for the operation to be cancelled if required.

The same calling sequence is used for all the reporting functions called by the allocation routines. The reporting function should be defined as a function of type CONFIG_ReportFunction. Such a function is shown in the detailed specification for ReportFunction().

The CONFIG_ReportFunction type is defined in tdFconfig.h as a function returning an int. In all cases, a null address may be passed for a feedback function, in which case no feedback calls will be made (and the operation in question cannot be cancelled).

Additionally, the ConfigAllocate() functions has a AllocReport argument.  This is the address of a routine invoked to indicate allocation and deallocation of fibres.  It has a similar calling sequence to ReportFunction, but, for historical reasons, allocation and deallocation reports to ReportFunction are unreliable and it should not be used to maintain details of allocations.  Instead AllocReport should be used to allow the “external” routines to keep track of which allocations “ConfigAllocate()” has made.  The type for this function is CONFIG_AllocRptFunction and full details can be found in the details specifications for AllocReport().

## Initialisation

The routine ConfigInitialise() should be called to initialise the allocation algorithms.

You pass to this function a description of the fibre positioner instrument you wish to configure for by passing an FpilType variable prevsiouly initialised using FpilInit().  The FPIL library allows access to details of instrument including what is known as the “constants” details – which fibres are broken, where the park positions are etc.  Whist ConfigInitialise() could use FPIL routines to access these constants details directly, it is easier if the caller does this and passes in another variable of type FpilConstantsType with the information.  This allows the caller to select where to find the constants file.  You must also pass in the address of a routine to be invoked to report informational messages to the user.  Error reports are reported using the DRAMA ERS package.  See the ERS library for details about catching error reports.

## Closing Down

Any program that has called ConfigInitialise() to initialise the allocation library routines should call ConfigClose() to close them down. This releases any resources (memory allocated, etc.) that have been obtained by the allocation routines. It is possible to call ConfigInitialise() again to re-initialise the allocation routines (and then call ConfigClose() again to close them down later) and this is the only way to change the fibre positioner instrument you are configuring for.

## Setting up a target field

A new target field is passed to the allocation routines using ConfigSetTargets().  The field must be passed using an SDS structure as described in the document ‘2dF Field Configuration File Structure Definition’. (Note, intentional reference to 2dF here, the 2dF structure format is currently used for all instruments) This structure contains a sub-structure describing the field itself (names, astrometric information, etc) which is ignored by the allocation routines.  It also contains a sub-structure describing the allocated objects in the field, so a target field can be passed to the allocation routines with some fibres already allocated. There are two sub-structures that contain details of the targets as yet unallocated, one for guide objects, one for target objects. As for the pivot information, the allocation routines can accept an SDS identifier that indicates the top level of such a structure, already opened, or can accept the name of a file that contains such a structure.

A call to this routine sets up a target field for subsequent allocation through a call to ConfigAllocate(), but there is no need to make such a call. ConfigSetTargets() defines a new target field for the allocation routines, automatically replacing any previous target field. There is no corresponding close down routine that has to be called before a new target field can be passed to the allocation routines through another call to ConfigSetTargets().

## Performing a complete configuration

Once the allocation routines have been initialised using ConfigInitialise() so that all the necessary pivot details are known, and once ConfigSetTargets() has been called to describe the target objects, a complete allocation is performed through a single call to ConfigAllocate().

## Controlling allocation options

Most allocation algorithms will be parameterized in one way or another. For example, one may have a number of different ways of choosing between possible targets (‘avoid crossing fibres’, ‘minimise fibre lengths’, etc).  It may even have the option of extra steps that can be used to improve allocation coverage at the expense of time taken (‘twist fibres’, ‘deallocate and retry’, etc.). These will be quite different for different algorithms, so a general-purpose interface must be provided. The one supplied here is based on having named parameters with associated values.  Additionally, the “housekeeping” layer” has a number of parameters of its own.

Parameters may have either logical (‘true’ or ‘false’) values, numeric values (integer or floating point) or may have character strings as values. Two types of character string parameters are provided: ones that may be set to any character string, and ones that can only be set to one of a set of specific strings.

The routine ConfigSetParameter() is used to set the value of a named parameter.

A user interface can just assume that it knows the parameters accepted by the allocation algorithm, but this might be rash. It certainly will inhibit experimentation with the algorithm. More usually, a user interface will ask the allocation algorithm what parameters it supports and what their possible values are. This is done through the enquiry routines ConfigNumberParameters() and ConfigParameterDetails().

ConfigNumberParameters() has no arguments itself, and simply returns the number of different parameters recognised by the “Config Library” and the allocation algorithm. Further details of these can be obtained by calling ConfigParameterDetails(). This routine can be called for each parameter in turn.

ConfigGetParameter() may be used to get the current value of a parameter. If this has not been explicitly set through a call to ConfigSetParameter(), a default value will be returned.

### Config library parameters.

The following parameters are provided by the config library itself.

- Fibre clearance (microns).  This is the clearance required between a button and a fibre.  It is initialised to the default value for the particular fibre positioner instrument, which is the minimum value which can be set

- Button clearance (microns).  This is the clearance required between two buttons. It is initialised to the default value for the particular fibre positioner instrument, which is the minimum value which can be set

- Max non-radial pivot angle (degrees).  This is the maximum amount a fibre is allowed to bend when coming out of the pivot.. It is initialised to the default value for the particular fibre positioner instrument, which is the maximum value which can be set

- Max button twist angle (degrees).  This is the maximum amount a fibre is allowed to bend due to a twist on the button.. It is initialised to the default value for the particular fibre positioner instrument, which is the maximum value which can be set.  This is not settable on some instruments.

Field validity checks.

Since the “external” program may allocate fibres manually before a call to ConfigSetTargets() (by manipulating the structures) and ConfigAllocate().  Alternatively, it may be specifying an previously allocated field.  It either case it should probably check the resultant field for validity.  It can do this by using ConfigFieldCheck() between the calls to ConfigSetTargets() and ConfigAllocate().  This uses the same checking code the algorithm layer will use.

## Adding and selecting new allocation algorithms.

The implementation provides a default allocation algorithm.  The external layer may add new allocation algorithms using ConfigRegisterMethod().  It may select from the various methods using ConfigSelectMethod().  It may determine the number of methods using ConfigNumMethods() and fetch the name of a method using ConfigGetMethodName().  These later routines allow a user interface to manage methods without knowing any details about them.

## Determine which pivots can be allocated

It is possible to indicate that some pivots may not be allocated/deallocated by the ConfigAllocate() function.  The ConfigSetPivotFlag() can be used to indicate if a particular pivot can be allocated, whilst ConfigSetSpecPivotFlag() can be used to indicate the pivots associated with a particular spectrograph may or may not be allocated.

## ImportMode stuff.

TBD or deleted

## Detailed external interface specifications

This section contains formal specifications for all the routines introduced in the previous sections.  To access these routines, include the file ‘tdFconfig.h’.

### ConfigInitialise()

Function: Initialisation of the configuration algorithm routines.

Description: This routine is the first of the configuration algorithm routines that should be called. It performs any necessary initialisation required by the routines.  Note that whilst you can specify a routine for reporting information messages here, you can’t specify a routine for reporting error messages.  The Config library uses the DRAMA ERS package to report error messages and you should see that library for details on how to intercept such messages.

Call:  ConfigInitialise (Instrument, InstConstDetails, InfoRoutine, FunctionArg, Status);

Parameters:

(>) Instrument	(FpilType) The description of the fibre instrument to configure for.  This variable should have been returned by FpilInit() when it was fully initialised (fully – the optional modelRoutines must have been supplied).  It describes the instrument we are configuring for.

(>) InstConstDetails	(FpilConstantsType)  The value returned by FpilConsAccess() when invoked with the value of Instrument for the inst argument.  By allowing the caller to provide this instead of ConfigInitialise() calling FpilConsAccess(), the caller can decide where to read the files from.

(>) InfoRoutine	(ConfigInfoRoutine) A routine used to report informational messages about the configuration to stdout or the equivalent.

(>) FunctionArg 	(void *) is an argument provided for the convenience of the writer of the info routine and will be passed to the routine when it is called.

(!) Status 	(StatusType *) is a conventional inherited status variable.

Prior requirements: None of the configuration algorithm routines should have been called prior to a call to this routine, unless a complete sequence beginning with a call to this routine and ending with a call to ConfigClose() has been run through.

Internal declaration:

void ConfigInitialise (

FpilType Instrument,   /* (>) Details of the fibre instrument */

FpilConstantsType InstConstDetails, /* (>) Constants details  */

ConfigInfoRoutine InfoRoutine /* (>) informational message reporting routine */

void *FunctionArg      /* (>) Argument for InfoRoutine */

StatusType *Status)    /* (!) Inherited status */

### ConfigClose()

Function: Closes down the configuration algorithm routines.

Description: This routine is the last of the configuration algorithm routines that should be called. It performs any necessary shutdown required by the routines.

Call:  ConfigClose (Status);

Parameters:

(!) Status 	(StatusType *) is a conventional inherited status variable.

Prior requirements: ConfigInitialise() should have been called to initialise the configuration routines..

Internal declaration:

void ConfigClose (

StatusType *Status)       /* Inherited status */

### ReportFunction()

Function: Provided by the external routines to monitor the configuration process.

Description: This routine is not one provided by the configuration algorithm library. It should be provided by the external code that calls the allocation routines as a feed back routine that they can call to report their progress.

NOTE:  Historically, this function was used by the “external program” to get reports of the allocation and deallocation of a pivot, to it could update its database of allocations.  It should no longer be used for this purpose, instead it should just be used for progress updates and to allow cancellation of the allocation.  If used to note allocation and deallocations of fibres, there is no certinatiy that all such events will be notified.  Use the AllocReport argument to ConfigAllocate() get such information in a reliable maner.

Call:  Cancel =  ReportFunction (FunctionArg, Type, Description, PivotIndex, IsGuide, TargetIndex, TargetX, TargetY, Theta, PercentDone);

Parameters:

(>) FunctionArg 	(void *) is an argument passed to the original routine along with the address of the feedback routine. It can be used for any purpose the original caller finds convenient.

(>) Type 	(char) is a single character, one of ‘A’ to indicate an allocation, ‘D’ to indicate a deallocation, or ‘P’ to indicate a more general progress update. PivotIndex,  IsGuide, TargetIndex, TargetX and TargetY will be set if Type is ‘A’ or ‘D’, ie where an allocation or deallocation is being reported. PercentDone is always set.

(>) Description 	(char *) is a string that describes what is going on at the moment, and a user-interface might make use of this.

(>) PivotIndex 	(int) will be the index number of the pivot being allocated or deallocated. Note that this index number starts from zero, not from one.

(>) IsGuide 	(int) will be true (ie non-zero) if the target being allocated or deallocated is a guide star, and will be set false (ie zero) if it is a program object.

(>) TargetIndex 	(int) will be the index number of the target that has been allocated. This will be an index number into the list of unallocated guide objects or the list of unallocated program objects that were in the structure passed to the allocation routines to describe the target field (see ConfigSetTarget()). Which of the two lists TargetIndex refers to will depend on the value of IsGuide.  Note that TargetIndex starts from zero, not from one.

(>) TargetX 	(long int) will be the X coordinate of the target being allocated or deallocated.

(>) TargetY 	(long int) will be the Y coordinate of the target being allocated or deallocated.

(>) Theta	(double) will be the button twist angle used for the allocation being made, in the case where Type is ‘A’.

(>) PercentDone 	(double) will be an estimate of the percentage of the task described by Description has now been completed, and will be set in all cases. Although this is a double precision floating point number, it should probably be rounded at least to the nearest integer before being output to the user. It will almost certainly not be accurate enough to warrant more. In some cases, it will not be possible for the routines to make any reasonable assessment of this value, particularly for iterative routines. In this case, it should be acceptable for a routine simply to return the percentage of fibres allocated so far - which may not be the same thing as time used so far as a percentage of the time required!

Returns:

(<) Cancel	(int) A ‘cancel’ flag. This should be non-zero if the allocation operation is to be cancelled, and should be zero if it is to be continued.

Prior requirements: The address of the feedback routine to be used should have been passed to a configuration routine by the external code that calls it.

Typedef declaration:

typedef int (*CONFIG_ReportFunction) (

/* (<) Returns true for ‘cancel’ */

void *FunctionArg,   /* (>) Caller-supplied argument */

char Type,           /* (>) Indicates the type of report */

char *Description,   /* (>) What the program is doing */

int PivotIndex,      /* (>) The index number of a pivot */

int IsGuide,         /* (>) Whether target is a guide star */

int TargetIndex,     /* (>) The index number of a target */

long int TargetX,    /* (>) The X-coordinate of a target */

long int TargetY,    /* (>) The Y-coordinate of a target */

double Theta,        /* (>) Theta value for button */

double PercentDone)  /* (>) The percentage of work done */

### AllocRptFunction()

Function: Provided by the external routines to monitor allocations and deallocations.

Description: This routine is not one provided by the configuration algorithm library. It should be provided by the external code that calls the allocation routines as a feed back routine that they can call to report allocation and deallocation of fibres.  This is the only way to find out what ConfigAllocate() does.

Call:  Cancel =  AllocRptFunction (FunctionArg, Type, PivotIndex, IsGuide, TargetIndex, TargetX, TargetY, Theta);

Parameters:

(>) FunctionArg 	(void *) is an argument passed to the ConfigAllocation() as the AllocRptArg along with the address of the feedback routine. It can be used for any purpose the original caller finds convenient.

(>) Alloc 	(int) Set true (ie non-zero) to indicate an allocation is being made.  Set false (ie zero) to indicate a deallocation is being made.

(>) PivotIndex 	(int) will be the index number of the pivot being allocated or deallocated. Note that this index number starts from zero, not from one.

(>) IsGuide 	(int) will be true (ie non-zero) if the target being allocated or deallocated is a guide star, and will be set false (ie zero) if it is a program object..

(>) TargetIndex 	(int) will be the index number of the target that has been allocated. This will be an index number into the list of unallocated guide objects or the list of unallocated program objects that were in the structure passed to the allocation routines to describe the target field (see ConfigSetTarget()). Which of the two lists TargetIndex refers to will depend on the value of IsGuide.  Note that TargetIndex starts from zero, not from one.

(>) TargetX 	(long int) will be the X coordinate of the target being allocated or deallocated.

(>) TargetY 	(long int) will be the Y coordinate of the target being allocated or deallocated.

(>) Theta	(double) will be the button twist angle used for the allocation being made, in the case where Type is ‘A’.

(>) Status 	(StatusType *) is a conventional inherited status variable.

Prior requirements: The address of the feedback routine to be used should have been passed to ConfigAllocate() by the external code that calls it.

Typedef declaration:

typedef int (*CONFIG_AllocRptFunction) (

void *FunctionArg,   /* (>) Caller-supplied argument */

int Alloc,           /* (>) True for allocation report */

int PivotIndex,      /* (>) The index number of a pivot */

int IsGuide,         /* (>) Whether target is a guide star */

int TargetIndex,     /* (>) The index number of a target */

long int TargetX,    /* (>) The X-coordinate of a target */

long int TargetY,    /* (>) The Y-coordinate of a target */

double Theta,        /* (>) Theta value for button */

StatusType *status)  /* (>) Inherited status */

### InfoRoutine()

Function: Provided by the external routines to log informational messages..

Description: This routine is not one provided by the configuration algorithm library. It should be provided by the external code that calls the allocation routines as a feed back routine that they can call to output informational messages to the user at any desired time..

Call:  void =  InfoRoutine (FunctionArg, String);

Parameters:

(>) FunctionArg 	(void *) is an argument passed to the original routine along with the address of the feedback routine. It can be used for any purpose the original caller finds convenient.

(>) String 	(const char *) is a string to be written to the equivalent of stdout.

Prior requirements: The address of the feedback routine to be used should have been passed to a configuration routine by the external code that calls it.

Typedef declaration:

typedef void (*ConfigInfoRoutine) (

void *FunctionArg,   /* (>) Caller-supplied argument */

const char *String); /* (>) String to be output      */

### ConfigSetTargets()

Function: Passes a new set of targets to the configuration routines.

Description: This routine is called to pass a new set of targets to the configuration allocation routines. It expects a target list encoded in an SDS structure that it can either be passed directly or can read form a disk file. The structure should have sub-structures specifying the unallocated guide objects, the unallocated target objects, and any allocations already made.

Call:  ConfigSetTargets (FileName, TopID, Field, ReportFunction, 									FunctionArg, Status);

Parameters:

(>) FileName 	(char *) is the name of a file containing an SDS structure as described in the document ‘2dF Field Configuration File Structure Definition’.  It is only used if TopID is zero.

(>) TopID 	(SdsIdType) is zero or the SDS identifier for an already opened structure of that type.

(>) Field 	(int) is the number of the field being set up — the pivot configuration structures passed when the allocation routines are initialised describe two fields, numbered zero and one, and since these can be subtly different, the allocation routines need to know which field is being used for this set of targets.

(>) ReportFunction (CONFIG_ReportFunction) is the address of a function that the allocation routines can call to report on their progress. Normally, this call will be handled quickly. However, if there are fibres already allocated, the routine performs a full validity check on the allocation, and if there are a large number of such fibres this can take a noticeable amount of time. The function provided should have the calling sequence described in the section on ‘Reporting Progress’. It can be null.

(>) FunctionArg 	(void *) is an argument provided for the convenience of the writer of the reporting function and will be passed to the reporting function when it is called.

(>) Status 	(StatusType *) is a conventional inherited status variable.

Prior requirements: ConfigInitialise() should have been called to initialise the configuration routines..

Internal declaration:

void ConfigSetTargets (

char *FileName,          /* (>) File containing structure  */

SdsIdType TopID,         /* (>) SDS ID of top of structure */

int Field,               /* (>) Field in question */

CONFIG_ReportFunction

ReportFunction, /* (>) Feedback routine to call */

void *FunctionArg,       /* (>) Caller-supplied argument */

StatusType *Status)      /* (!) Inherited status */

### ConfigAllocate()

Function: Performs a complete allocation.

Description: This routine performs a complete allocation, attempting to allocate as yet unallocated targets to as yet unallocated fibres. It reports its progress to the calling code via a feedback routine whose address it is passed.

Call:  ConfigAllocate (ReportFunction, FunctionArg, Status);

Parameters:

(>) ReportFunction (CONFIG_ReportFunction) is a routine that will be called by the allocation routines to report the progress of the allocation and to allow a way for the allocation to be cancelled if necessary. It has the form described in the section 2.1.

(>) FunctionArg 	(void *) is an argument provided for the convenience of the writer of the reporting function and will be passed to the reporting function when it is called.

(>) AllocReport	(CONFIG_AllocFunction) is a routine that will be called by the allocation routines to report the allocation or deallocation of a pivot.

(>) AllocRptArg	(void *) is an argument provided for the convenience of the writer of the AllocReport function and will be passed to the reporting function when it is called.

(!) Status 	(StatusType *) is a conventional inherited status variable.

Prior requirements: ConfigInitialise() should have been called to initialise the configuration routines..

Internal declaration:

void ConfigAllocate (

CONFIG_ReportFunction

ReportFunction, /* (>) Feedback routine to call */

void *FunctionArg,       /* (>) Caller-supplied argument */

StatusType *Status)      /* (!) Inherited status */

### ConfigSetParameter()

Function: Sets the value of one of the allocation algorithm parameters.

Description: The allocation algorithm code may be parameterised — its behaviour can be modified by the external code making calls to set ‘parameters’. These parameters will be quite different from implementation to implementation, so there needs to be a general-purpose way of finding out about the supported parameters of any given implementation. This routine sets the value of such a parameter.

Call:  ConfigSetParameter (Name, Value, Status);

Parameters:

(>) Name	(char *) Address of a nul-terminated string giving the name of the parameter in question. This must match exactly — names are case-sensitive.

(>) Value	(char *) Address of a nul-terminated string giving the value to which the parameter is to be set. If this is a numeric value it should be encoded as a character string. If it is a logical value it should be one of either ‘T’ (true) or ‘F’ (false).

(!) Status	(StatusType *) Inherited status.

Prior requirements: ConfigInitialise() should have been called prior to any calls to this routine.

Internal declaration:

void ConfigSetParameter ( /* Set value of a named parameter */

char *Name,              /* (>) Name of parameter */

char *Value,             /* (>) Value for parameter */

StatusType *Status);     /* (!) Inherited status */

### ConfigNumberParameters()

Function: Returns the number of allocation algorithm parameters.

Description: The allocation algorithm code may be parameterised — its behaviour can be modified by the external code making calls to set ‘parameters’. These parameters will be quite different from implementation to implementation, so there needs to be a general-purpose way of finding out about the supported parameters of any given implementation. This routine simply returns the number of such parameters supported by the algorithm.

Call:  NumberParameters = ConfigNumberParameters();

Parameters: None.

Returns:

(<) NumberParameters	(int) The number of parameters supported.

Prior requirements: ConfigInitialise() should have been called prior to any calls to this routine.

Internal declaration:

int ConfigNumberParameters (void);

/* (<) Returns number of parameters */

### ConfigParameterDetails()

Function: Returns details about one of the allocation algorithm parameters.

Description: The allocation algorithm code may be parameterised — its behaviour can be modified by the external code making calls to set ‘parameters’. These parameters will be quite different from implementation to implementation, so there needs to be a general-purpose way of finding out about the supported parameters of any given implementation. This routine returns details about one of the allocation algorithm parameters.

Call:  ConfigParameterDetails (ParameterNumber, Name, Type, Limits, NValues, Values, Status);

Parameters:

(>) ParameterNumber	(int) The number of the parameter. This should be a number in the range 1 through the value returned by ConfigNumberParameters().

(<) Name	(char **) Set to the address of a nul-terminated string giving the name of the parameter.

(<) Type	(char *) Set to a single character giving the type of the parameter: ‘I’ integer, ‘F’ floating point, ‘L’ logical, or ‘C’ character.

(<) Limits	(double [2]) The address of an array of two double precision values. The first is set to the minimum value of the parameter and the second to its maximum value. This is only used for parameters of type ‘I’ or ‘F’.

(<) NValues	(int *) The number of possible fixed values a character parameter may have. Only used for parameters of type ‘C’. If set to zero, the parameter may have any value.

(<) Values	(char ***)  For a character parameter, this is set to the address of an array of NValues pointers to char. Each of these points to a nul-terminated string giving one of the allowed values for the parameter.

(!) Status	(StatusType *) Inherited status.

Prior requirements: ConfigIntialise() should have been called prior to any calls to this routine. Normally, ConfigNumberParameters() will also have been called to see how many parameters there are. If ParameterNumber is too large, an error message will be output and bad status returned.

Internal declaration:

void ConfigParameterDetails (

/* Gets details of a given parameter */

int ParameterNumber,   /* (>) Number of parameter */

char **Name,           /* (<) Parameter name */

char *Type,            /* (<) Type code for parameter */

double Limits[2],      /* (<) Numerical range */

int *NValues,          /* (<) # of possible values */

char ***Values,        /* (<) List of possible values */

StatusType *Status);   /* (!) Inherited status */

### ConfigGetParameter()

### Function: Gets the value of one of the allocation algorithm parameters.

Description: The allocation algorithm code may be parameterised — its behaviour can be modified by the external code making calls to set ‘parameters’. These parameters will be quite different from implementation to implementation, so there needs to be a general-purpose way of finding out about the supported parameters of any given implementation. This routine gets the value of such a parameter.

Call:  ConfigGetParameter (Name, Value, VSize, Status);

Parameters:

### ConfigSetImportFile()

Function: Set the Import File name and mode.

Description: This routine directly calls the allocation layer’s SetImportFile() function.  This is not well defined at the moment but can be considered a way of importing allocation information into the allocation layer.

MAY YET BE REMOVED AND REPLACED BY A PARAMETER

Call:  ConfigSetImportFile (name,Status);

Parameters:

(>) Name 	(char *)

(!) Status 	(StatusType

Prior requirements:

Internal declaration:

void ConfigSetImportFile (

char name,               /* (>) Name of file     */

StatusType *Status)      /* (!) Inherited status */

### ConfigSetImportOnlyMode()

Function: Causes the suppression of errors when running in import only mode..

Description: This routine.sets an internal flag to indicate that errors should not be output if we attempt to allocate a broken fibre.

SHOULD THIS BE A LOGICAL PARAMETER

Call:  ConfigSetImportOnlyMode (value);

Parameters:

(>) value	(int)

Prior requirements:

Internal declaration:

Void ConfigSetImportOnlyMode (

int value)/* (>) Set true to enable import only mode, false otherwise*/

### ConfigRegisterMethod()

Function: Register a new allocation method with the Config library.

Description: This routine is supplied with details of an allocation method.  It stores those details for later use.  Currently, a maximum of 4 allocation methods may be registered, in addition to the default allocation method.

Call:  ConfigRegisterMethod (Method, Enable, Index, Status);

Parameters:

(>) Method 	(CONFIG_MethodDetails *)  A structure supplying details of an allocation method.  See section 3 for details

(>) Enable 	(int) If true, then this method becomes the default method the next time ConfigAllocate().

(<) Index	(unsigned *) The index of this method, for use with ConfigSelectMethod() and ConfigGetMethodName().

(!) Status 	(StatusType *) is a conventional inherited status variable.

Prior requirements: Should only be called after calling ConfigInitialise().

Internal declaration:

Void ConfigRegisterMethod (

CONFIG_MethodDetails *method, /* (>) Method details */

int Enable                    /* (>) Is method to be enabled */

unsigned *Index               /* (<) Index for method */

StatusType *Status);          /* (!) Inherited status */

### ConfigSelectMethod()

Function: Select the specified method for future allocations..

Description: This routine.enables an allocation method previously registered with ConfigRegisterMethod() as the method to use the next time ConfigAllocate() is invoked.

Call:  ConfigSelectMethod (Index, Status);

Parameters:

(>) Index 	(unsigned) Indicates the allocation method to select.  This value must have been previously returned by ConfigRegisterMethod(), except that a value of 0 indicates the default method.

(!) Status 	(StatusType *) is a conventional inherited status variable.

Prior requirements:

Internal declaration:

void ConfigSelectMethod (

unsigned int,            /* (>) Method to select */

StatusType *Status);     /* (!) Inherited status */

### ConfigNumMethods()

Function: Return the number of allocation methods available.

Description: This routine returns the number of allocation methods that have been registered with the Config library.

Call:  ConfigNumMethod (Number, Status);

Parameters:

(>) Number 	(unsigned *) The number of registered allocation methods is returned here.  You can use a value between 0 and this number minus one as the index in ConfigSelectMethod() and ConfigGetMethodName().

(!) Status 	(StatusType *) is a conventional inherited status variable.

Prior requirements:

Internal declaration:

void ConfigNumMethod (

unsigned int *Number,    /* (>) Method to select */

StatusType *Status);     /* (!) Inherited status */

### ConfigGetMethodName()

Function: Return the name of a method.

Description: Returns the name of a specified method..

Call:  ConfigGetMethodName (Index, Namelen, Name, Status);

Parameters:

(>) Index 	(unsigned) Indicates the allocation method to return the name of.  This value must have been previously returned by ConfigRegisterMethod(), except that a value of 0 indicates the default method.

(>) Namelen 	(unsigned) the space available in the Name argument.

(<) Name	(char *) The name of the method is written here.

(!) Status 	(StatusType *) is a conventional inherited status variable.

Prior requirements:

Internal declaration:

void ConfigGetMethodMethod (

unsigned int,            /* (>) Method to select */

unsigned Namelen,        /* (>) Output length limit */

char *Name               /* (>) Name is written here */

StatusType *Status);     /* (!) Inherited status */

### ConfigFieldCheck()

Function: Checks whether or not the current set of allocations is valid.

Description: This routine does a validity check on the current set of allocations, as specified to ConfigSetTargets() and possibly as modified by ConfigAllocate().

Call: ConfigFieldCheck (ReportFunction, FunctionArg, Status);

Parameters:

(>) ReportFunction	(CONFIG_ReportFunction) Address of a feedback routine to be called with details of the progress of any time-consuming operations performed during this call. May be NULL.

(>) FunctionArg	(void *) An argument to be passed to the feedback routine when it is called.

(!) Status	(StatusType *) Inherited status variable. .  Will be set to CONFIG__INVALID_FIELD if the field is not valid

Prior requirements: ConfigMethodInit() must have been called to initialise the method-specific code, and ConfigSetTargets() to set up the targets.

Internal declaration:

void ConfigFieldCheck (    /* Performs a full field validation */

CONFIG_ReportFunction ReportFunction,

/* (>) Feedback routine to call */

void *FunctionArg,          /* (>) Caller-supplied argument */

StatusType  *status);      /* (!) Inherited status variable */

### ConfigSetPivotsFlag()

Function: Set or clear the flag which indicates a pivot’s allocation may be changed.

Description: Each pivot has a flag associated with it that indicates if the pivot’s allocation state may be changed by a call to ConfigAllocate().  This routine can be used to change the state of the flag.

Call: ConfigSetPivotFlag (Pivot, Flag, Status);

Parameters:

(>) Pivot	(int) The number of the pivot, range 1 to maximum pivot number.

(>) Flag	(int) True if the pivot can be allocated, Flag if it cannot be allocate.

(!) Status	(StatusType *) Inherited status variable. Will be set to CONFIG__INVAL_PIVOT if the pivot number is invalid.

Prior requirements: ConfigMethodInit() must have been called to initialise the method-specific code, and ConfigSetTargets() to set up the targets.

Internal declaration:

void ConfigSetPivotFlag (/

int Pivot,                 /* (>) Pivot number */

int Flag,                  /* (>) New flag value for pivot */

StatusType  *status);      /* (!) Inherited status variable */

### ConfigSetSpecPivotsFlag()

Function: Set or clear the flag which indicates a pivot’s allocation may be changed for a range of pivots.

Description: Each pivot has a flag associated with it that indicates if the pivot’s allocation state may be changed by a call to ConfigAllocate().  This routine can be used to change the state of the flag for a range of pivots allocated to a spectrograph, or as guide fibres.

Call: ConfigSetPivotFlag (Pivot, Flag, Status);

Parameters:

(>) Spec	(int) The value minus 1 indicates all pivots.  The value 0 indicates guide pivots.  Values 1 to number of spectrographs indicates a spectrography.

(>) Flag	(int) True if the associated pivots can be allocated, Flag if it cannot be allocate.

(!) Status	(StatusType *) Inherited status variable. Will be set to CONFIG__INVAL_SPEC if the spectrograph number is invalid.

Prior requirements: ConfigMethodInit() must have been called to initialise the method-specific code, and ConfigSetTargets() to set up the targets.

Internal declaration:

void ConfigSetSpecPivotsFlag (/

int Spec,                  /* (>) Spectrograph number */

int Flag,                  /* (>) New flag value for pivot */

StatusType  *status);      /* (!) Inherited status variable */

## Using ConfigParameterDetails()

The number of levels of indirection in the parameters Name and Values for the routine ConfigParameterDetails() may appear complicated, but it is much easier to use than to explain.  The following test program may make things clearer. In the main program, Name is just a pointer to a character string, and Values is just an array of character string pointers. It’s only in the description of the subroutine interface that the additional level of redirection makes things sound awkward.

#include "DitsTypes.h"

#include "tdFconfigMethod.h"

#include <stdio.h>

int main()

{

StatusType Status;

int I,IVal,Np,NValues;

char *Name,Type,**Values;

double Limits[2];

Status = 0;

Np = ConfigNumberParameters();

for (I = 1; I <= Np; I++) {

ConfigParameterDetails (I,&Name,&Type,Limits,

&NValues,&Values,&Status);

if (Status == 0) {

printf ("Parameter '%s'",Name);

if (Type == 'L') {

printf (" is logical\n");

} else if (Type == 'I') {

printf (" is an integer between %d and %d\n",

(int) Limits[0], (int) Limits[1]);

} else if (Type == 'F') {

printf (" is a real number between %f and %f\n",

Limits[0],Limits[1]);

} else if (Type == 'C') {

printf (" is a character parameter, with ");

if (NValues == 0) {

printf (" unrestricted values\n");

} else {

printf (" the following possible values:\n");

for (IVal = 0; IVal < NValues; IVal++) {

printf ("'%s'\n",Values[IVal]);

}

}

}

}

}

return ((int) Status);

}

# Writing a new allocation algorithm

This section explains how to write a new allocation algorithm to replace the standard one supplied with the package.

Working out a good new algorithm is probably very difficult, but the intention is that implementing it should be as easy as possible. You need to understand the layering of the configuration library routines.

There is a layer called the ‘external’ code. This is the main body of the program that calls the configuration library routines, and is not part of the configuration library at all. If you are writing a new allocation algorithm you don’t have to concern yourself with this layer at all.

There is a layer called the ‘housekeeping’ routines. These handle everything except the actual allocation. That might not seem much, but in fact there is a lot of housekeeping involved in the configuration problem. If you’re writing a new algorithm, you don’t want to be concerned with details like file formats, where the pivot and target information comes from, or the user-interface. The ‘housekeeping’ routines, all of which start with ‘Config’, are called by some external code (either a stand-alone program, a complicated user-interface, or whatever), and they handle reading the pivot and target information, validating it, talking to the external code, etc. The ‘housekeeping’ layer will then call your algorithm-specific routines to do the actual allocation, generally passing them pointers to various arrays in memory where the pivot and target details can be found.

Then there’s the ‘algorithm-specific’ layer, which is the set of routines that you have to write. You have to provide a total of 9 routines.  You place pointers to these routines and other information in a structure of type CONFIG_MethodDetails and specify the address of this structure in a call to ConfigRegisterMethod().  Up to 4 allocation methods may be registered and can be selected using ConfigSelectMethod().  Normally what would happen is that the external code will ask your code to fill in a variable of type CONFIG_MethodDetails.  It will then call ConfigRegisterMethod() using that variable.

When invoked by the housekeeping layer, all of the routines you specify in this structure take as their first argument, a pointer to a structure of type CONFIG_MethodInfo.  In addition to some other information, this sturcture contains pointers to various routines supplied by the configuration library “housekeeping” layer.  These routines may be invoked as desired by the allocation routines.  For example, FieldCheck() an be used to perform a complete validity check on the allocations made so far, and guarantees to do so in the same way as the positioner code itself, so a configuration passed by it will always be accepted by the positioner. More trivial utilities include things like FibreClear() which returns the fibre clearance value you ought to be using — the interface to the external routines allows them to set more stringent clearance values, and you should always use the value provided by the utility routines rather than a value read from other sources (particularly as you won’t know which fibre instrument you are configuring for).

You should not call any of the housekeeping library routines directly from your allocation library, must instead always call them via the pointers in the CONFIG_MethodInfo structure.  This approach ensures it is easy to build shareable libraries from allocation libraries.  By using sharable allocation libraries the “external” code can dynamically load in new allocation routines.  If you call the housekeeping library routines directly, a sharable library will link in its own copy of the housekeeping routines and associated data storage, which will be inconsistent with the version used by the “external” layer.  You should not make any instrument specific assumptions in your algrothim or call the instrument FPIL library routines.  All the information you need should be available from the housekeeping routines and if it appears not to be, please contact the authors such that it can bee added.  You should not directly call routines related to the “external” interface in your allocation library as that makes you code dependent on that “external” interface.  Instead, always use the routines the addresses of which as passed to your routines.  It is safe to call system services and C RTL routines.

Of the routines you supply in CONFIG_MethodDetails, two , GlobalInit() and Init() are initialisation routines, one called just once at the start of the program and one called whenever a new field (a set of targets) is read. One, ReleaseMem() is a shutdown routine, called when the program closes down. The one that does the real work is Allocate() which is supposed to run the actual allocation algorithm. Then there are four routines, NumberParms(), ParamDetails(), SetParam() and GetParam() that are concerned with handling any parameters you may want to use to provide fine control over the allocation. You probably don’t have to write new versions of the parameter routines, since it should be possible to use the versions provided with the standard algorithm — you just have to set up some tables for them.  SetImportFile() ?

## Overview

In some of what follows, I describe what the ‘default allocation algorithm does when its implementation of the various routines is called, but this is purely for illustration and in no way implies that other algorithms should do the same thing.

Most of the work of allocating fibres is done when the interface routine ConfigAllocate() is called by the ‘external’ code (by which I mean whatever program has been written to call the allocation package interface routines defined previously). ConfigAllocate() calls two method-specific routines, Init() and Allocate().

When Init() is invoked, it can be assumed that the various arrays of information about the field contained in the MethodInfo argument are valid, in particular, the three arrays of structures, one holding the information about the pivots, one holding the information about the targets, and the third having one entry (one structured array element) for each pivot showing whether or not it is allocated yet and if so to what. . What Init() does with all this is entirely up to it, but the standard method-specific code uses the information they contain to set up a matrix with one element for each possible allocation of fibre to target. It then sets the elements of this matrix to indicate whether the corresponding allocation (sometimes called a ‘path’) is possible or not. (An ‘impossible’ path is one which violates some geometrical constraint, such as the length of the fibre, or would involve a collision with some already allocated fibre.

Allocate() is then called and is expected to perform the allocation. It can do this however it likes. It can experimentally allocate and then deallocate fibres if it likes, or it can just run through doing nothing but allocate fibres if it can see a way to do that without experimentation. When it makes an allocation (even if experimental) it is expected a) to report the new allocation to the ‘external’ code by calling a feedback routine whose address it has been given, and b) to call the housekeeping routine NoteAllocation() which updates the array of structures than contains the allocation details. Similarly, if it makes a deallocation it is expected to call the same feedback routine — this time with a flag indicating ‘deallocation’, and to call the housekeeping routine NoteDeallocation() to update the allocation details. The standard code, in essence, runs through the matrix looking for the ‘hardest’ pivot to allocate (the pivot that has the fewest targets to which it can be allocated) and allocates it to one of these, then updates the matrix to allow for collisions with the newly allocated fibre and repeats the process until it can’t allocate any more.

There is more to it than that, but not a great deal more. Obviously, a lot of additional detail is needed before you try to write a new allocation algorithm. The structures used have to be rigorously defined, and that’s probably the most important thing that needs explanation. The interface between the interface layer and the algorithm-specific layer comprises more than the two routines mentioned above, and all the additional routines that have to be provided have to be defined as well, together with all the utility routines provided by the interface layer specifically for use by the algorithm-specific layer.

In the rest of this section, I describe the rest of the routines that the algorithm-specific routines must provide. This is purely a summary; more details are in later sections.

The first ‘Config’ routine called by the external code is ConfigInitialise(). This in turn will call the allocation method specific GlobalInit() routine. This routine should do anything that might be necessary to initialise the algorithm-specific routines. The standard routine does nothing but initialise some pointers to null.

The last ‘Config’ routine called by the external code is ConfigClose(). This in turn will call allocation method specific ReleaseMem() routine. This routine should do anything that might be necessary to close down the algorithm-specific routines. The reason for the name is that in most cases all that will be necessary is the freeing of memory obtained by the algorithm-specific routines. It is possible that there will be a number of call sequences that begin with a call to GlobalInit() and end with a call to ReleaseMem(), and the algorithm-specific routines should be aware of this.

Most allocation algorithms will be parameterised in some way. It should be possible to exercise some control over the details of the algorithm. Since this is going to be so different from algorithm to algorithm, there must be ways for the external code to enquire the details of these parameters. These details need to be enough for a user-interface to be able to present the choices to the user without itself understanding anything about the meaning of the parameters themselves.

The interface provided uses named parameters that can be ‘logical’ (ie having only ‘true’ or ‘false’ values), integers or floating point numbers, or character strings that can have any value or one of a number of fixed values. The names can be obtained by the external code through calls to ConfigNumberParameters() and ConfigParameterDetails(), which do little more than call the algorithm-specific routines NumberParams() and ParamDetails(). The trick to making these intelligible to the user is to have parameter names and values that are as self-explanatory as possible. Names here are just character strings, can be of any length and can include spaces. So, for example, the standard algorithm has a parameter called ‘Target selection’ which is a character parameter with the possible values ‘by hardest target’, ‘by furthest target’, ‘by straightest target’ and ‘by closest target’. Putting these up in a user interface makes it reasonably clear what the options are.

The routines ConfigSetParameter() and ConfigGetParameter(), which do little more than call the algorithm-specific routines SetParam() and GetParam(), are used to get and set parameter values. The standard package contains implementations of these routines that are driven entirely from a set of variable declarations at the start of the program. Most new implementations should be able to keep the code unchanged and modify only the variable declarations (and the way the parameters are used!)

Structures and types used

All the types used by the configuration code are defined in the file tdFconfigMethod.h. This defines the structured types CONFIG_MethodDetails, CONFIG_MethodInfo, CONFIG_PivotInfo, CONFIG_TargetInfo and CONFIG_AllocInfo, described in detail in the rest of this section. It also defines the logical type CONFIG_Boolean and the function type CONFIG_ReportFunction used for the feedback routines.

Many of the routines are defined with a final argument of type StatusType, described as an ‘inherited status variable’. This is the standard DRAMA convention, and the type is defined in the standard DRAMA include files. The convention is that a non-zero status value indicates an error. The ‘inherited’ part is that if a routine is called with this status variable already set to a non-zero value, the routine is supposed to return immediately. This allows the code to ‘fall-through’ a series of calls to functions without needing to test status after each one. If one fails, it will set the status variable to a non-zero value and the subsequent routines will do nothing.

The key to understanding the way to code an algorithm-specific set of routines is to understand the details of the configuration as maintained in the MethodInfo structure.   A structure of type CONFIG_PivotInfo is used by the ‘housekeeping’ layer to hold information about a single pivot. The ‘housekeeping’ layer maintains an array of these, one for each pivot, and the address of the start of this array available from the MethodInfo structure..

Similarly, a structure of type CONFIG_TargetInfo holds information about a single target, and the ‘housekeeping’ layer maintains an array of these, one for each target it knows about, and the address of this array is kept in the MethodInfo structure . The contents of these structures should not be changed during the course of an allocation, either by the ‘housekeeping’ layer or by the algorithm-specific routines.

Finally, a structure of type CONFIG_AllocInfo holds information about the current allocation of a single pivot (is it allocated or not, if so to which target, etc.). The ‘housekeeping’ layer maintains an array of these, one for each pivot, and the address of this array is also kept in the MethodInfo structure. The contents of these allocation information structures does change during the course of an allocation. The algorithm-specific routines should not modify the allocation information structures directly, but should call the utility routines NoteAllocation() and NoteDeallocation() to make any necessary changes to them. In fact, the algorithm-specific routines should not directly modify any of the information in or pointed to by the MethodInfo structure and it is therefore declared as const..

### The CONFIG_MethodDetails structure

This structure defines the details of an allocation method – specifying the routines to perform the allocation as well as some auxiliary information.

typdef struct CONFIG_MethodDetails {  /* Details about a method */

int                             Version;

int                             RoutineCount;

void *                          clientData;

char                            Name[CONFIG_MTH_NAME_LEN];

ConfigMethodGlobalInitRoutine   GlobalInit;

ConfigMethodInitRoutine         Init;

ConfigMethodReleaseRoutine      ReleaseMem;

ConfigMethodAllocateRoutine     Allocate;

ConfigMethodNumParamsRoutine    NumParams;

ConfigMethodParamDetailsRoutine ParamDetails;

ConfigMethodSetParamRoutine     SetParam;

ConfigMethodGetParamRoutine     GetParam;

ConfigMethodSetImportRoutine    SetImportFile;

} CONFIG_MethodDetails;

The various routine types and the usage of the routines are defined in section 3.3.  The Version item should be set to the value CONFIG_VERSION, defined in tdFconfigMethod.h.  This allows a check between the version your library was compiled against and the version used by the “external” program.  The RoutineCount item should be set to the value CONFIG_N_ROUTINES.  This item may be used in the future to allow optional specfication of routines etc. The clientData item is simply passed through to the various routines within the CONFIG_MethodInfo structure.  It can be used for any desired purpose.by the allocation routines (for example - maintaining context across). The name item is just a simple string which gives a name to the method.  CONFIG_MTH_NAME_LEN is currently 40.

### The CONFIG_MethodInfo structure

The address of a structure of this type is passed to each of the method implementation routines and should be passed back to each housekeeping routine invoked.  It contains pointers to the housekeeping routines, the clientData item specified by the user using the CONFIG_MethodDetails structure and values bits of information needed by an allocation library.

typedef struct CONFIG_MethodInfo {

void *                          clientData;

ConfigIntValRoutine             MaxExtension;

ConfigGetVirtPivRoutine         GetVirtPiv;

ConfigThetaFibRoutine           ThetaFib;

ConfigCheckAlphaRoutine         CheckAlpha;

ConfigChkAllocationRoutine      CheckAllocation;

ConfigFieldChkRoutine           FieldCheck;

ConfigNoteAllocationRoutine     NoteAllocation;

ConfigTentativeFixRoutine       TentativeFix

ConfigNoteDeallocationRoutine   NoteDeallocation;

ConfigIntValRoutine             ButtonClear;

ConfigIntValRoutine             FibreClear;

ConfigRealValRoutine            MaxButAngle;

ConfigRealValRoutinne           MaxPivAngle;

ConfigIntValRoutine             SafeClearance;

ConfigColFibFibRoutine          ColFibFib;

ConfigColButFibRoutine          ColButFib;

ConfigColButButRoutine          ColButBut;

ConfigColInvPosRoutine          ColInvPos;

ConfigCollisionRoutine          Collision;

ConfigInfoRoutine               InfoRoutine;

void                            * InfoArgument;

ConfigInfoRoutine               ErrRepRoutine;

int                             Field,

int                             NumberPivots;

int                             NumberTargets;

const CONFIG_TargetInfo         * TargetInfo;

const CONFIG_PivotInfo          * PivotInfo;

const CONFIG_AllocInfo          *AllocInfo;

} CONFIG_MethodInfo;

Here the clientData item is the exact value that was passed in the same item in the CONFIG_MethodDetails structure in the call to ConfigRegisterMethod().  The details of the various housekeeping routines are found in section 3.4.

Field is the number of the field currently being.  NumberPivots is the number of pivots in the configuration.  NumberTargets is the number of targets in the target field.

TargetInfo is a pointer to an array of NumberTarget structures of type CONFIG_TargetInfo, as documented in section 3.2.4.

PivotInfo is a pointer to an array of NumberPivots structures of type CONFIG_PivotInfo, as documented in section 3.2.3.

AllocInfo is a pointer to an array of NumberPivots structures of type CONFIG_AllocInfo, as documented in section 3.2.5.

For all three of the above arrays and the Field, NumberPivots and NumberTargets items, the item is only valid from when the Init() routine is invoked and should not be modified by the Algrothim method code directly (hence the “const” ness of the items).

InfoRoutine is the address of a routine that can be invoked to output informational messages to the user. Its first argument must be the value given in InfoArgument.  It is documented in section 2.12.5 rather then below.  This will case a direct call to the routine supplied to ConfigInitialise().

ErrRepRoutine is the address of a routine which can be invoked to output error messages to the user.  When this is invoked, it results in the ERS library’s ErsRep() routine be invoked with the string argument.  Call it with the first argument specified as zero.

### The CONFIG_PivotInfo structure

Each fibre emerges from a ‘pivot point’.  The CONFIG_PivotInfo structure contains details about the pivot point and the fibre that emerges from it. Since there is a one-to-one relationship between pivots and fibres, the terms tend to be used interchangeably. The structure is defined in the include file tdFconfigMethod.h as:

typedef struct CONFIG_PivotInfo {  /* Details about each pivot */

long X;              /* X-ordinate of pivot, in microns */

long Y;              /* Y-ordinate of pivot, in microns */

long ParkX;          /* X-ordinate of park position, in microns*/

long ParkY;          /* Y-ordinate of park position, in microns*/

char Spect;          /* Spectrograph. Zero, 'A', to 'E' or 'F'

* where 'F' indicates fiducial and zero

* indicates broken.  Otherwise, the value

* indicates a spectrograph */

double Theta;        /* Theta value for pivot when parked*/

int AllocAllocChange;/* If true (CONFIG_TRUE), the allocate of

* this pivot can be changed. */

} CONFIG_PivotInfo;

The fields within the structure have the following meanings:

X	is the X co-ordinate of the pivot. As with all co-ordinates in the configuration software, this is a value in microns from an origin located at the centre of the ring of pivots. The pivot, of course, is a mechanical construction and not a single point. When a fibre is placed in the field, it can be treated as a straight line joining this pivot point and a conceptual point located near the end of the fibre button. (This point near the end of the button is called the ‘‘button virtual pivot point’, and is discussed in more detail in the background section describing the allocation problem.)

Y 	is the Y co-ordinate of the pivot in the same co-ordinate system.

ParkX	is the X co-ordinate of the pivot when in its parked position.

ParkY	is the Y co-ordinate of the pivot when in its parked position.

Spect is a code character describing what the fibre associated with the pivot is connected to. It will be one of: ‘A’ to ’E’,’F’ or a zero value. ‘A’ indicates that the fibre is one that feeds into spectrograph ‘A’. ‘B’ indicates that the fibre feeds into spectrograph ‘B’ etc (up to five spectrographs are supported). ‘F’ indicates that the fibre is a guide fibre (one that should normally only be assigned to guide stars), and a zero value indicates that the fibre in question is broken and should not be assigned to anything.

Theta is a double precision value giving the angle of the fibre when it is parked. This angle, like all ‘theta’ angles in the configuration code, is the angle between the vertical through the pivot and the fibre, measured anti-clockwise in radians, with zero being straight up. Since fibres are parked all pointing to the centre of the field, this is also the ‘theta’ value made by the line joining the pivot and the centre of the field, and can be used to simplify the calculation of the non-radial pivot angle (alpha) for the fibre when it is allocated. (The non-radial pivot angle is the angle between the allocated fibre and the line joining the pivot and the field centre, and so is a measure of how much the fibre is being bent as it leaves the pivot; there is a limit to how large this angle can be in a valid allocation.)

AllowAllocChange is a int used as a boolean value.  If set true (CONFIG_TRUE) it indicates the allocation of a pivot can be changed.  If set false (CONFIG_FALSE) it indicates the allocation cannot be changed and NoteAllocation()/NoteDeallocation() will return errors if you try to change the allocation.  Such a pivot may or may not already be allocated.  Typical uses are to ensure only a particular spectrograph is allocated or to allow fixed allocation of particular pivots to particular targets.

The ‘housekeeping’ software creates an array of such structures, one for each pivot, when it is first initialised with the details of the current pivot configuration (when ConfigInitialise() is called). Pivots are numbered from one up, the pivot number increasing in an anti-clockwise direction, pivot one being the pivot closest to the ‘straight up’ position. The first element of the array refers to pivot one, so pivot number N will be described by the structure whose index is N-1. It is the address of this array of structures that is passed using the MethodInfo structure.. This whole array must remain unchanged throughout an allocation, and so should not be modified by the algorithm-specific code.

### The CONFIG_TargetInfo structure

Each target in the field is described by a structure of type CONFIG_TargetInfo. This structure is defined in tdFconfigMethod.h as:

typedef struct CONFIG_TargetInfo {/* Details about each target */

long X;               /* X-coordinate of target, in microns */

long Y;               /* Y-coordinate of target, in microns */

int OriginalIndex;    /* Index in original target list */

CONFIG_Boolean PreAllocated;

/* True if explicitly allocated */

char Type;            /* Type of object. 'P' = program object,

* 'S' = sky, 'F' = fiducial (guide) */

char Priority;        /* Priority value, 0..9, 9 = highest. */

char Spect;           /* Spectrograph to use. 'A', or 'B' or

* zero for don't care. */

} CONFIG_TargetInfo;

The fields within the structure have the following meanings:

X 	is the X co-ordinate of the pivot. As with all co-ordinates in the configuration software, this is a value in microns from an origin located at the centre of the ring of pivots. The target is treated as being a single point at the actual target centre, and this is the point at which the centre of the fibre button prism  must be placed.

Y 	is the Y co-ordinate of the target in the same co-ordinate system.

OriginalIndex is a housekeeping item that will be needed when calling the feedback routines. The target lists are originally passed to the housekeeping layer as two separate lists, one for unallocated target objects and one for unallocated guide stars. The housekeeping layer merges these into one target list. The OriginalIndex field gives the index of the target in question in the original list from which it came (guide stars or targets), and this is what the external code is most interested in. In some cases, there is also a third list of targets. It is possible for fibres to be pre-allocated to targets — the allocation is already made in the field details that are passed to the configuration routines. In some cases, these allocations will be to targets in the lists of unallocated guide stars and targets (in which case the targets in those lists will be flagged as already allocated, something that the housekeeping routines check when a new field is passed to them), but it is possible for the pre-allocated targets not to appear at all in the target lists. In this case the OriginalIndex field for the target is set to -1.

PreAllocated is a flag to indicate that the target was pre-allocated. As should be obvious from the previous description of the OriginalIndex field, targets with an OriginalIndex field of -1 are all pre-allocated targets, but not all pre-allocated targets have an OriginalIndex field of -1; some have known OriginalIndex values. Generally, an allocation algorithm should not attempt to deallocate pre-allocated targets, although this might be reasonable under some circumstances; this is up to the writer of the algorithm.

Type is a single character that will be one of ‘P’, ‘S’, or ‘F’. ‘P’ indicates a program object — one whose spectrum is required as part of the observing program. ‘S’ indicates a sky object — a position chosen to contain no actual object, and required for calibration purposes. ‘F’ indicates a fiducial object — a guide star. Generally, an allocation algorithm will not distinguish between program and sky targets, but guide stars need to be allocated to guide fibres, and non-guide targets need to be allocated to non-guide fibres. So a target with a type of ‘F’ should only be allocated a pivot with a Spect field of ‘F’, and a target with a type of ‘P’ or ‘S’ should only be allocated to a pivot with a Spect field of ‘A’ or ‘B’ (ie one whose fibre feeds into one of the two spectrographs).

Priority is a single integer giving the priority assigned to this target. This is a number in the range 0 through 9, with 9 indicating the highest priority. Generally, an allocation algorithm should give priority to allocating high priority targets, but the trade-offs between this and allocating as many pivots as possible are a matter for the designed of the algorithm.  The value CONFIG_PRIO_INVALID will be placed in this field if the target is outside of the field and should not be configured (largely for historical reasons).

Spect is a single character used to indicate whether or not there is a preference for the target to be allocated to a specific spectrograph. This will be one of ‘A’, ‘B’ or zero. If zero, no preference is being indicated. If it is ‘A’, the target should only be allocated to a pivot whose Spect field is also ‘A’, and similarly for ‘B’.

The ‘housekeeping’ software creates an array of such structures, one for each target, when it is passed the details of the current target field (when ConfigSetTargets() is called). The order of targets in this array is not significant. This whole array must remain unchanged throughout an allocation, and so should not be modified by the algorithm-specific code.

### The CONFIG_AllocInfo structure

For each pivot the housekeeping routines create a structure that contains information about whether or not it is allocated and the details of any allocation. This could have been simply incorporated into the CONFIG_PivotInfo structure described earlier, but they are kept separate to emphasise that the allocation information will change during the allocation process, but the basic pivot details are fixed.

The structure is defined in tdFconfigMethod.h as follows:

typedef struct CONFIG_AllocInfo {

/* Details about pivot allocation */

int TargetIndex;    /* Allocated target, if any (index into

* the TargetInfo global array) */

long X;             /* X-coordinate of position, in microns */

long Y;             /* Y-coordinate of position, in microns */

double Theta;       /* Theta value for fibre */

char State;         /* 'U' or 'A' or zero, for 'unallocated',

* 'allocated' or broken. */

int Tentative;      /* Is the allocation Tentative */

} CONFIG_AllocInfo;

The fields have the following meanings:

TargetIndex indicates the target, if any, allocated to the pivot. This is only meaningful if the State field is ‘A’ to indicate that the pivot is allocated. In this case, it gives the index of the element in the target information array describing the target. (This element is a structure of type ConfigTargetInfo, and the address of the start of the target information array of these structures is passed as one of the elements of the MethodInfo structure.) Note that the index values start at zero.

X 	gives the X co-ordinate of the target allocated to the pivot, if any. This is only meaningful if the State field is ‘A’ to indicate that the pivot is allocated. This value will be the same as the X field of the ConfigTargetInfo structure indicated by the TargetIndex field, so there is some redundancy in the information provided here, but it is sometimes convenient to have the information in the allocation structure directly.

Y 	gives the Y co-ordinate of the target allocated to the pivot, if any. The comments above relating to the X field apply equally to the Y field.

Theta is a double precision value giving the orientation of the button.  This angle, like all ‘theta’ angles in the configuration code, is the angle between the vertical through the button and the fibre, measured anti-clockwise in radians, with zero being straight up.

State is a single character indicating whether or not the pivot has been allocated to a target. It will be one of ‘U’, ‘A’ or zero. ‘U’ indicates that the pivot has not yet been allocated, ‘A’ indicates that it has been allocated. If State is zero, the fibre associated with the pivot is broken and cannot be allocated.

Tentative is a integer used as a logical flag.  If true (non-zero) it indicates the allocation is tentative.  If it is false (zero) it indicates the allocation is not tentative.  Tentative allocations have not been notified to the “external” code using the allocation report function.  Tentative allocations can confirmed or removed using the TentativeFix() routine.

The ‘housekeeping’ software creates an array of such structures, one for each pivot, when it is first initialised with the details of the current pivot configuration (when ConfigInitialise() is called). The allocation information is filled up by the housekeeping layer when ConfigSetTargets() is called. So the housekeeping layer maintains two arrays, each with the same number of elements (one for each pivot), one an array of CONFIG_AllocInfo structures holding the allocation information and one an array of CONFIG_PivotInfo structures holding the fixed pivot details. Pivots are numbered from one up, the pivot number increasing in an anti-clockwise direction, pivot one being the pivot closest to the ‘straight up’ position. The first element of the array refers to pivot one, so pivot number N will be described by the structure whose index is N-1. The Nth element in the array of CONFIG_PivotInfo structures always refers to the same pivot as does the Nth element in the array of CONFIG_AllocInfo structures. It is the address of this array of structures that is passed using the MethodInfo structure. When a new allocation or deallocation is made by the algorithm-specific code, a call to the housekeeping routine NoteAllocation() or to the housekeeping routine NoteDeallocation() should be made. These routines will update the allocation information array, and the algorithm-specific code should not attempt to modify this array directly.

## Algorithm-specific routine specifications

This section describes the algorithm-specific routines that are called by the ‘housekeeping’ layer. An implementation of the algorithm-specific routines must provide all of these routines unless otherwise noted.  Here the routine name is the name of the item within the CONFIG_MethodDetails structure.  Since you specify pointers to routines when setting up the structure, the actual name of the routine is not relevant.  But you must provide a routine that matches the typedef specified.

### GlobalInit()

Function: Start-up initialisation of the algorithm-specific routines.

Description: This routine is called as part of the start-up sequence for the configuration routines. It allows the algorithm-specific routines to do any necessary initialisation of global variables, or indeed anything else that needs to be done at this time.

Call:  GlobalInit(MethodInfo);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) Used to supply details to the method implementation routines, including the addresses of the config library housekeeping routines.  Note that the pivot, target and allocation information is not valid when this call is made.

Prior requirements: None of the method implementation routines will  have been called prior to a call to this routine, unless a complete sequence beginning with a call to this routine and ending with a call to ReleaseMem() has been run through.

Typedef declaration:

typedef void (* ConfigMethodGlobalInitRoutine) (

const CONFIG_MethodInfo * MethodInfo);

### ReleaseMem()

Function: Release any resources used by the algorithm-specific routines.

Description: This routine is called as part of the close-down sequence for the configuration routines. It allows the algorithm-specific routines to do anything that needs to be done at this time, but this usually involves releasing resources obtained during the allocation process, particularly memory — hence the name.

Call:  ReleaseMem(MethodInfo);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) Used to supply details to the method implementation routines, including the addresses of the config library housekeeping routines..

Prior requirements: GlobalInit() must have been called. This should be the last call to any algorithm-specific routine, although a complete sequence of calls starting with a new call to GlobalInit() and ending with a call to this routine can be made as many times as required.

Typedef declaration:

typedef void (* ConfigMethodReleaseRoutine) (

const CONFIG_MethodInfo * MethodInfo);

### Init()

Function: Indicates a new set of pivot and target details are available.

Description: This routine is called to initialise the algorithm-specific routines with a new set of pivot information and a new set of target information. The pivot information is held in an array of structures of type CONFIG_PivotInfo within the MethodInfo structure.   Usually, this information will not change from one configuration to the next, but this should not be assumed. The target information is held in an array of structures of type CONFIG_TargetInfo in the MethodInfo structure. Some of the pivots may already be allocated to targets;  an array of structures of type CONFIG_AllocInfo in MethodInfo, one for each pivot, contains information about any allocations made so far.

Call:  Init(MethodInfo, ReportFunction, FunctionArg, Status);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) Used to supply details to the method implementation routines, including the addresses of the config library housekeeping routines..

(>) ReportFunction	(CONFIG_ReportFunction) Address of a feedback routine to be called with details of the progress of any time-consuming operations performed during this call. May be NULL.

(>) FunctionArg	(void *) An argument to be passed to the feedback routine when it is called.

(!) Status	(StatusType *) Inherited status variable.

Prior requirements: InitGlobal() should have been called before any call to this routine.

Note:.The information held in the PivotInfo and TargetInfo arrays will not change during the course of an allocation. The information held in the AllocInfo array will be updated constantly during the course of an allocation

Typedef declaration:

Typedef void (* ConfigMethodInitRoutine) (

const CONFIG_MethodInfo * MethodInfo,

CONFIG_ReportFunction ReportFunction,

/* (>) Feedback routine to call */

void *FunctionArg,   /* (>) Caller-supplied argument */

StatusType *Status); /* (!) Inherited status */

### Allocate()

Function: Perform an actual allocation.

Description: This routine is called to perform the actual allocation of fibres. It must have been preceded by a call to Init(), and should attempt to allocate the unallocated pivots to the unallocated targets in whatever way it chooses. As it allocates pivots (or even deallocates pivots, if it is an algorithm that can make tentative allocations and then think better of it) it should report its progress by making calls to the feedback routine whose address it is passed. Also, as it allocates and deallocates pivots it should call the utility routines NoteAllocation() and NoteDeallocation(). These modify the contents of the allocation information array whose address was passed to Init(), and this routine can make use of that or can maintain some internal record of pivot to target allocations.

Call:  Allocate(MethodInfo, ReportFunction, FunctionArg, Status);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) Used to supply details to the method implementation routines, including the addresses of the config library housekeeping routines..

(>) ReportFunction	(CONFIG_ReportFunction) Address of a feedback routine to be called with details of the progress of any time-consuming operations performed during this call. May be NULL.

(>) FunctionArg	(void *) An argument to be passed to the feedback routine when it is called.

(!) Status	(StatusType *) Inherited status variable.

Prior requirements: Init() must have been called prior to a call to this routine. Calls to explicitly allocate or deallocate individual pivots may be made between the call to Init() and the call to this routine.

Typedef declaration:

typedef void (* ConfigMethodAllocateRoutine) (

const CONFIG_MethodInfo * MethodInfo,

CONFIG_ReportFunction ReportFunction,

/* (>) Feedback routine to call */

void *FunctionArg,        /* (>) Caller-supplied argument */

StatusType *Status);      /* (!) Inherited status */

### NumberParams()

Function: Returns the number of algorithm-specific parameters.

Description: The algorithm-specific code may be parameterised — its behaviour can be modified by the external code making calls to set ‘parameters’. These parameters will be quite different from implementation to implementation, so there needs to be a general-purpose way of finding out about the supported parameters of any given implementation. This routine simply returns the number of such parameters supported by the algorithm-specific code.

Call:  NumberParameters = NumberParams(MethodInfo);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) Used to supply details to the method implementation routines, including the addresses of the config library housekeeping routines..

Returns:

(<) NumberParameters	(int) The number of parameters supported.

Prior requirements: GlobalInit() should have been called prior to any calls to this routine.

Typedef declaration:

typedef void (* ConfigMethodNumParamsRoutine) (

const CONFIG_MethodInfo * MethodInfo,

### ParamDetails()

Function: Returns details about one of the algorithm-specific parameters.

Description: The algorithm-specific code may be parameterised — its behaviour can be modified by the external code making calls to set ‘parameters’. These parameters will be quite different from implementation to implementation, so there needs to be a general-purpose way of finding out about the supported parameters of any given implementation. This routine returns details about one of the algorithm-specific parameters.

Call:  ParamDetails (MethodInfo, ParameterNumber, Name, Type, Limits, NValues, Values, Status);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) Used to supply details to the method implementation routines, including the addresses of the config library housekeeping routines..

(>) ParameterNumber	(int) The number of the parameter. This should be a number in the range 1 through the value returned by ConfigMethodNumberParams().

(<) Name	(char **) Set to the address of a nul-terminated string giving the name of the parameter.

(<) Type	(char *) Set to a single character giving the type of the parameter: ‘I’ integer, ‘F’ floating point, ‘L’ logical, or ‘C’ character.

(<) Limits	(double [2]) The address of an array of two double precision values. The first is set to the minimum value of the parameter and the second to its maximum value. This is only used for parameters of type ‘I’ or ‘F’.

(<) NValues	(int *) The number of possible fixed values a character parameter may have. Only used for parameters of type ‘C’. If set to zero, the parameter may have any value.

(<) Values	(char ***)  For a character parameter, this is set to the address of an array of NValues pointers to char. Each of these points to a nul-terminated string giving one of the allowed values for the parameter.

(!) Status	(StatusType *) Inherited status.

Prior requirements: GlobalInit() should have been called prior to any calls to this routine. Normally, NumberParams() will also have been called to see how many parameters there are. If ParameterNumber is too large, an error message will be output and bad status returned.

Typedef declaration:

typedef void (* ConfigMethodParamDetailsRoutine) (

const CONFIG_MethodInfo * MethodInfo,

int ParameterNumber,   /* (>) Number of parameter */

char **Name,           /* (<) Parameter name */

char *Type,            /* (<) Type code for parameter */

double Limits[2],      /* (<) Numerical range */

int *NValues,          /* (<) # of possible values */

char ***Values,        /* (<) List of possible values */

StatusType *Status);   /* (!) Inherited status */

### SetParam()

Function: Sets the value of one of the algorithm-specific parameters.

Description: The algorithm-specific code may be parameterised — its behaviour can be modified by the external code making calls to set ‘parameters’. These parameters will be quite different from implementation to implementation, so there needs to be a general-purpose way of finding out about the supported parameters of any given implementation. This routine sets the value of such a parameter.

Call:  SetParam (MethodInfo, Name, Value, Status);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) Used to supply details to the method implementation routines, including the addresses of the config library housekeeping routines..

(>) Name	(char *) Address of a nul-terminated string giving the name of the parameter in question. This must match exactly — names are case-sensitive.

(>) Value	(char *) Address of a nul-terminated string giving the value to which the parameter is to be set. If this is a numeric value it should be encoded as a character string. If it is a logical value it should be one of either ‘T’ (true) or ‘F’ (false).

(!) Status	(StatusType *) Inherited status.

Prior requirements: GlobalInit() should have been called prior to any calls to this routine.

Typedef declaration:

typedef void (* ConfigMethodSetParamRoutine)

const CONFIG_MethodInfo * MethodInfo,

char *Name,              /* (>) Name of parameter */

char *Value,             /* (>) Value for parameter */

StatusType *Status);     /* (!) Inherited status */

### GetParam()

Function: Gets the value of one of the algorithm-specific parameters.

Description: The algorithm-specific code may be parameterised — its behaviour can be modified by the external code making calls to set ‘parameters’. These parameters will be quite different from implementation to implementation, so there needs to be a general-purpose way of finding out about the supported parameters of any given implementation. This routine gets the value of such a parameter.

Call:  GetParam (MethodInfo, Name, Value, VSize, Status);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) Used to supply details to the method implementation routines, including the addresses of the config library housekeeping routines..

(>) Name	(char *) Address of a nul-terminated string giving the name of the parameter in question. This must match exactly — names are case-sensitive.

(>) Value	(char *) Address of a nul-terminated string that is to receive the value of the parameter. If this is a numeric value it will be encoded as a character string. If it is a logical value it will be set to one either ‘T’ (true) or ‘F’ (false).

(>) VSize	(int) The number of bytes available at the address given by Value. This routine always returns a nul-terminated string, even if this means truncating the parameter value. VSize must allow for the terminating nul - no more than VSize characters will be written by this routine, including that nul character.

(!) Status	(StatusType *) Inherited status.

Prior requirements: GlobalInit() should have been called prior to any calls to this routine.

Typedef declaration:

typedef void (* ConfigMethodGetParamRoutine)

const CONFIG_MethodInfo * MethodInfo,

char *Name,             /* (>) Name of parameter */

char *Value,            /* (>) Value for parameter */

int VSize,              /* (>) Maximum number of characters */

StatusType *Status);    /* (!) Inherited status */

### SetImportFile()

Function: Set import mode and the name of the file to import..

Description: This routine is called directly by ConfigSetImportFile().  This is not well defined at the moment but can be considered a way of importing allocation information into the allocation layer.  In the default implementation, the miport file  is actually read and imported by the Allocate() function, this call just sets the name of the and the mode is enabled using a parameter (so this function is in effect providing a way .

CONSIDER REMOVING AND USING A PARAMETER.  MAYBE WE NEED TO DEFINE FILENAME PARAMETERS.  LOOK AT HOW THIS IS USED IN CONFIGURE

Call:  SetImportFile (MethodInfo, Name,Status);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) Used to supply details to the method implementation routines, including the addresses of the config library housekeeping routines..

(>) Name 	(char *)

(!) Status 	(StatusType

Prior requirements:

Typedef declaration:

typedef void (*ConfigMethodSetImportRoutine) (

const CONFIG_MethodInfo * MethodInfo,

char name[80],           /* (>) Name of file     */

StatusType *Status)      /* (!) Inherited status */

## Utility routines

This section provides detailed specifications of the utility routines provided to the algrothim specific code by the ‘housekeeping’ layer via the MethodInfo structure. The conventions for coordinates and angles are described in section 4.2.

.

For each of the routines here, the name is the name of the item within the CONFIG_MethodInfo structure.  This item is a pointer to the routine and you should only call ‘housekeeping’ routines via the appropriate pointer to avoid a link-time dependency.  The “typedef” declaration shows how this pointer and the underlying routine are declared.

### MaxExtension()

Function: Returns the maximum fibre extension in microns.

Description: This routine returns the maximum extension of a fibre in microns. This maximum extension limits the set of targets that any given fibre can reach.

Call:  Extension = MaxExtension(MethodInfo);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) The MethodInfo variable that was passed to the calling routine and that this routine is called via.

Returns:

(<) Extension	(long int) The maximum fibre extension, in microns.

Prior requirements: None.

Typdef declaration:

typedef long (* ConfigIntValRoutine) (

const CONFIG_MethodInfo * MethodInfo);

### GetVirtPiv()

Function: Returns the coordinates of the virtual pivot point for a button.

Description: When a button is twisted, the line of the fibre can be taken as being a straight line from the pivot point at the edge of the field to a conceptual point behind the button called the ‘virtual pivot point’. The position of this point depends on the coordinates of the button ‘origin’ — the location of the centre of the prism at the end of the button, usually coincident with the target for an allocated button — and the orientation of the button, ie its ‘theta’ angle. This routine calculates the location of the virtual pivot point for a button given the origin coordinates and its theta value.

Call:  GetVirtPiv (MethodInfo, OriginX, OriginY, Theta, VirtPivX, VirtPivY);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) The MethodInfo variable that was passed to the calling routine and that this routine is called via.

(>) OriginX	(double) The x-coordinate of the centre of the button prism. In microns, from the centre of the field.

(>) OriginY	(double) The y-coordinate of the centre of the button prism. In microns, from the centre of the field.

(>) Theta	(double) The theta angle for the button. In radians, anti-clockwise from the vertical through the origin to the line through the centre of the button.

(<) VirtPivX	(double *) Returned with the x-coordinate of the virtual pivot point for the button.

(<) VirtPivY	(double *) Returned with the y-coordinate of the virtual pivot point for the button.

Prior requirements: None.

Typdef declaration:

typedef long (* ConfigGetVirtPivRoutine) (

const CONFIG_MethodInfo * MethodInfo,

double OriginX,/* (>) The x-coordinate of the button origin */

double OriginY,/* (>) The y-coordinate of the button origin */

double Theta,         /* (>) The theta angle for the button */

double *VirtPivX,

/* (<) The x-coordinate of the virtual pivot */

double *VirtPivY);

/* (<) The y-coordinate of the virtual pivot */

### ThetaFib()

Function: Calculates the radial theta fibre angle for an allocated fibre.

Description: The ‘radial theta fibre angle’ for a fibre is the ‘theta’ angle for the fibre relative to the pivot point. That is, it is the angle, measured anti-clockwise in radians, from the pivot point to the line of the fibre. This routine calculates this for an allocated fibre, using a target information structure to get the position of the fibre button (note that this assumes that the button is not ‘twisted’, ie is in line with the fibre) and a pivot information structure to get the position of the pivot. The ‘theta’ value for the button, ie the anticlockwise angle from the vertical through the button to the line through the centre of the button (again assuming that the button has a zero twist angle), is given by ThetaFib + PI if ThetaFib is less than PI, and is ThetaFib - PI if ThetaFib is greater than PI.

Call:  ThetaFib = ThetaFib (MethodInfo, TargetInfo, PivotInfo)

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) The MethodInfo variable that was passed to the calling routine and that this routine is called via.

(>) TargetInfo	(const CONFIG_TargetInfo *) Used to supply the coordinates of the origin of the button, assuming that the fibre has been allocated to that button and the button has a zero twist angle..

(>) PivotInfo	(const CONFIG_PivotInfo *) Used to supply the coordinates of the pivot.

Returns:

(<) ThetaFib	(double) The ‘theta’ angle for the fibre relative to the pivot.

Prior requirements: None.

Typdef declaration:

typedef double (* ConfigThetaFibRoutine) (

const CONFIG_MethodInfo * MethodInfo,

const CONFIG_TargetInfo *TargetInfo, /* (>) Target info structure */

const CONFIG_PivotInfo *PivotInfo);   /* (>) Pivot info structure */

### CheckAlpha()

Function: Calculates and checks the non-radial pivot angle for a fibre.

Description: The ‘non-radial pivot angle’ (alpha) for a fibre is the angle between the fibre and the radial line joining its pivot to the centre of the field. This routine calculates this angle for an allocated fibre, given information about the target and the pivot, and indicates if this is within the allowed range. This routine assumes that the button has a zero twist angle, ie that the fibre lies exactly along the line joining the pivot point and the target.

Call:  InRange = CheckAlpha (MethodInfo, TargetInfo, PivotInfo, Alpha);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) The MethodInfo variable that was passed to the calling routine and that this routine is called via.

(>) TargetInfo	(const CONFIG_TargetInfo *) Used to supply the coordinates of the origin of the button, assuming that the fibre has been allocated to that button and the button has a zero twist angle..

(>) PivotInfo	(const CONFIG_PivotInfo *) Used to supply the coordinates of the pivot.

(<) Alpha	(double *) Returned with the non-radial pivot angle for the fibre.

Returns:

(<) InRange	(CONFIG_Boolean) Returns a true value if the calculated alpha value is within the allowed range.

Prior requirements: None.

Typedef declaration:

typedef CONFIG_Boolean (* ConfigCheckAlphaRoutine) (

const CONFIG_MethodInfo * MethodInfo, */

const CONFIG_TargetInfo *TargetInfo, /* (>) Target info structure */

const CONFIG_PivotInfo *PivotInfo,    /* (>) Pivot info structure */

double *Alpha);       /* (<) Returns calculated alpha value */

### CheckAllocation()

Function: Checks whether or not a proposed allocation is possible.

Description: Given a proposed allocation of pivot to target, this routine checks whether it will be invalid because of a collision with another allocated fibre. This assumes that the allocation is physically valid — ie that the constraints of fibre length, pivot angle, etc are all OK. The pivot and target in question are identified in terms of their index numbers into the lists of pivots and targets as passed to allocation method Init() routine. If the allocation is invalid as a result of a collision with another fibre, the pivot number for the first such fibre is returned — this can later be used to help speed up deallocation processing. (But note that there may be many such colliding pivots, and this only lists the first.)

Call:  AllocationOK = CheckAllocation (MethodInfo, Pivot, Target, Theta, Colliding Pivot);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) The MethodInfo variable that was passed to the calling routine and that this routine is called via.

(>) Pivot	(int) The index number of the pivot in question. (From zero up)

(>) Target	(int) The index number of the target in question. (From zero up)

(>) Theta	(double) The proposed ‘theta’ angle for the button — the anticlockwise angle in radians between the centre of the prism for the button and the line through the centre of the button.

(<) CollidingPivot	(int *) The index number of the first allocated pivot found to cause a collision with the proposed allocation.

Returns:

(<) AllocationOK	(CONFIG_Boolean) Returns a true value if the proposed allocation is valid, and a false one if it will produce a collision with an already allocated fibre..

Prior requirements: The allocation method MethodInit() routine must have been called to provide a context for the pivot and target indices used..

Typedef declaration:

typedef CONFIG_Boolean (* ConfigCheckAllocationRoutine) (

const CONFIG_MethodInfo * MethodInfo,

int Pivot,       /* (>) The number of the pivot in question */

int Target,     /* (>) The number of the target in question */

double Theta,       /* (>) The proposed button angle, theta */

int *CollidingPivot);       /* (<) Any one colliding pivot. */

### FieldCheck()

Function: Checks whether or not the current set of allocations are valid.

Description: During the allocation process, the method-specific code reports the allocations it makes to the ‘housekeeping’ layer using NoteAllocation() and NoteDeallocation(). When this routine is called, the housekeeping layer checks the set of allocations that have been made so far, using the same algorithm that the positioner task will eventually use when presented with the configuration. This provides a safety check that the allocations made so far are in fact all valid and will be accepted as such by the positioner.

Call: FieldCheck (MethodInfo, ReportFunction, FunctionArg, Status);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) The MethodInfo variable that was passed to the calling routine and that this routine is called via.

(>) ReportFunction	(CONFIG_ReportFunction) Address of a feedback routine to be called with details of the progress of any time-consuming operations performed during this call. May be NULL.

(>) FunctionArg	(void *) An argument to be passed to the feedback routine when it is called.

(!) Status	(StatusType *) Inherited status variable.  Will be set to CONFIG__INVALID_FIELD if the field is not valid.

Prior requirements: The allocation algrothim Init() routine must have been called to initialise the method-specific code, and allocations and deallocations must all have been reported using NoteAllocation() and NoteDeallocation().

Typedef declaration:

typedef void (* ConfigFieldCheckRoutine) (

const CONFIG_MethodInfo * MethodInfo,

CONFIG_ReportFunction ReportFunction,

/* (>) Feedback routine to call */

void *FunctionArg,          /* (>) Caller-supplied argument */

StatusType  *status);      /* (!) Inherited status variable */

### NoteAllocation()

Function: Used to report the allocation of a fibre to a target.

Description: During the allocation process, the method-specific code reports the allocations it makes to the ‘housekeeping’ layer using this routine and NoteDeallocation(). The call to this routine causes the allocation information array of structures (the address of which was passed to allocation algrothim’s Init() routine to be updated to show the new allocation.

Call:  NoteAllocation (MethodInfo, Pivot, Target, Theta, Tentative, Status);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) The MethodInfo variable that was passed to the calling routine and that this routine is called via.

(>) Pivot	(int) The index number of the pivot in question. (From zero up)

(>) Target	(int) The index number of the target in question. (From zero up)

(>) Theta	(double) The proposed ‘theta’ angle for the button — the anticlockwise angle in radians between the centre of the prism for the button and the line through the centre of the button.

(>) Tentative	(int) Indicates if an allocation is tentative.  If an allocaton is tentative, the external code is not notified of the allocation using the Allocation Report function supplied to ConfigAllocate().  Otherwise the structures are set up the same.  Use can confirm or reject a tentative allocation using the TenativeFix() routine.  The main reason for tentative allocation is to avoid user interface updates during a procedure which tries various allocations before picking the required one.  It is an error for the allocation algorithm to complete with outstanding tentative allocations.

(!) Status	(StatusType *) Inherited status variable.

Prior requirements: None. Strictly, the allocation algrothim’s Init() routine must have been called to initialise the method-specific code. However, if this is being called from method-specific code, that must have been done already, since that is the first method-specific routine called.

Typedef declaration:

typedef void (* ConfigNoteAllocationRoutine) (

const CONFIG_MethodInfo * MethodInfo,/

int Pivot,                     /* (>) Pivot being allocated */

int Target,                /* (>) Target allocated to pivot */

double Theta,           /* (>) Theta button angle for fibre */

int Tentative,           /* (>) Is the allocation tentative */

StatusType *Status);       /* (!) Inherited status variable */

### TentativeFix()

Function: Used to confirm or reject a tentative allocation..

Description: This allows a tentative allocation to be confirmed or rejected.  If confirmed, then the Allocation Report function supplied to ConfigAllocate() will be invoked for this allocation.  Otherwise, the pivot is deallocated.

Call:  NoteAllocation (MethodInfo, Pivot, Confirm, Status);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) The MethodInfo variable that was passed to the calling routine and that this routine is called via.

(>) Pivot	(int) The index number of the pivot in question. (From zero up)

(>) Confirm	(int) Set true to confirm the allocation, false to reject it.

(!) Status	(StatusType *) Inherited status variable.

Prior requirements: None. Strictly, the allocation algrothim’s Init() routine must have been called to initialise the method-specific code. However, if this is being called from method-specific code, that must have been done already, since that is the first method-specific routine called.

Typedef declaration:

typedef void (* ConfigTentativeFixRoutine) (

const CONFIG_MethodInfo * MethodInfo,/

int Pivot,                     /* (>) Pivot being allocated */

int Confirm,                /* (>) Is allocation confirmed */

StatusType *Status);       /* (!) Inherited status variable */

### NoteDeallocation()

Function: Used to report the deallocation of a fibre.

Description: During the allocation process, the method-specific code reports the allocations it makes to the ‘housekeeping’ layer using this routine and NoteAllocation(). The call to this routine causes the allocation information array of structures (the address of which was passed to allocation algrothim’s Init() routine to be updated to reflect the deallocation of a previously allocated fibre.

Call:  NoteDeallocation (MethodInfo, Pivot, Status);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) The MethodInfo variable that was passed to the calling routine and that this routine is called via.

(>) Pivot	(int) The index number of the pivot in question. (From zero up)

(!) Status	(StatusType *) Inherited status variable.

Prior requirements: None. Strictly, the allocation method’s Init() routine must have been called to initialise the method-specific code. However, if this is being called from method-specific code, that must have been done already, since that is the first method-specific routine called.

Typedef declaration:

typedef void (* ConfigNoteDeallocationRoutine) (

const CONFIG_MethodInfo * MethodInfo,*/

int Pivot,                     /* (>) Pivot being allocated */

StatusType *Status);       /* (!) Inherited status variable */

### NoteDeallocationBroken()

Function: Used to report the de-allocation of a broken fibre fibre.

Description: When checking an existing allocation, the algorithm specific code may find an allocation of a broken fibre and wish to de-allocate it.  This version of  NoteDeallocation() should be invoked in that case to ensure the fibre is correctly marked as broken.

Call:  NoteDeallocationBroken (MethodInfo, Pivot, Status);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) The MethodInfo variable that was passed to the calling routine and that this routine is called via.

(>) Pivot	(int) The index number of the pivot in question. (From zero up)

(!) Status	(StatusType *) Inherited status variable.

Prior requirements: None. Strictly, the allocation method’s Init() routine must have been called to initialise the method-specific code. However, if this is being called from method-specific code, that must have been done already, since that is the first method-specific routine called.

Typedef declaration:

typedef void (* ConfigNoteDeallocationRoutine) (

const CONFIG_MethodInfo * MethodInfo,*/

int Pivot,                     /* (>) Pivot being allocated */

StatusType *Status);       /* (!) Inherited status variable */

### ButtonClear()

Function: Returns the minimum button clearance in microns.

Description: This routine returns the minimum allowed clearance around a button  The value chosen will allow for any uncertainties in button position and fibre position and non-linearity. It may also (depending on the external code, which can set the actual value) reflect some additional leeway allowed for changes in time of the field.

Call:  Clearance = ButtonClear(MethodInfo);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) The MethodInfo variable that was passed to the calling routine and that this routine is called via.

Returns:

(<) Clearance	(long int) The maximum button clearance in microns.

Prior requirements: None.

Typedef declaration:

typedef long (* ConfigIntValRoutine) (

const CONFIG_MethodInfo * MethodInfo);

### FibreClear()

Function: Returns the minimum fibre clearance in microns.

Description: This routine returns the minimum allowed clearance around a fibre  The value chosen will allow for any uncertainties in button position and fibre position and non-linearity. It may also (depending on the external code, which can set the actual value) reflect some additional leeway allowed for changes in time of the field.

Call:  Clearance = FibreClear(MethodInfo);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) The MethodInfo variable that was passed to the calling routine and that this routine is called via.

Returns:

(<) Clearance	(long int) The maximum fibre clearance in microns.

Prior requirements: None.

Typedef declaration:

typedef long (* ConfigIntValRoutine) (

const CONFIG_MethodInfo * MethodInfo);

### MaxButAngle()

Function: Returns the maximum twist angle allowed for a button.

Description: This routine returns the minimum allowed ‘twist’ angle of a button.  This is the angle between the line of the fibre leading to the button and the line through the centre of the button, and is a measure of the amount of bend in the fibre as it enters the button. The value chosen may (depending on the external code, which can set the actual value) reflect some additional leeway allowed for changes in time of the field.  For some fibre instruments (such as 6dF), this is always zero as the button rotates in the gripper jaw to make it zero.  For others (such as 2dF) is can be set by changing the button theta angle.

Call:  MaxButAngle = ConfigMaxButAngle(MethodInfo);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) The MethodInfo variable that was passed to the calling routine and that this routine is called via.

Returns:

(<) MaxButAngle	(double) The maximum button twist angle in radians.

Prior requirements: None.

Typedef declaration:

typedef double (* ConfigRealValRoutine) (

const CONFIG_MethodInfo * MethodInfo);

### MaxPivAngle()

Function: Returns the maximum non-radial pivot angle allowed for a fibre

Description: This routine returns the minimum allowed non-radial pivot angle for a fibre. This is the angle between the line of the fibre and the radial line from the pivot to the field centre and is a measure of the amount of bend in the fibre as it leaves the pivot. The value chosen may (depending on the external code, which can set the actual value) reflect some additional leeway allowed for changes in time of the field.

Call:  MaxPivAngle = MaxPivAngle(MethodInfo);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) The MethodInfo variable that was passed to the calling routine and that this routine is called via.

Returns:

(<) MaxPivAngle	(double) The maximum non-radial pivot angle in radians.

Prior requirements: None.

Typedef declaration:

typedef double (* ConfigRealValRoutine) (

const CONFIG_MethodInfo * MethodInfo);

### SafeClearance()

Function: Returns a guaranteed safe clearance around a button.

Description: This routine returns a guaranteed safe clearance around a button. This can be used in some cases to speed checking of a new possible allocation. The detailed checking for collision between a button and a fibre or another button is a complex calculation involving knowledge of the shape of a button. However, a quick check can be done using rectangular calculations and this value. Anything more than this distance from any part of the button will not collide with the button. Anything closer than that may collide with the button, or may not — for that to be determined a detailed calculation has to be performed. But using this value to see if a collision can be ruled out can eliminate a number of calls to the more detailed routines.

Call:  Clearance = SafeClearance(MethodInfo);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) The MethodInfo variable that was passed to the calling routine and that this routine is called via.

Returns:

(<) Clearance	(long int) The ‘safe’ clearance in microns.

Prior requirements: None.

Typedef declaration:

typedef long (* ConfigIntValRoutine) (

const CONFIG_MethodInfo * MethodInfo);

### ColFibFib()

Function:Checks for collisions between two fibres (a crossover).

Description: double This routine does an instrument specific collision detection check, checking for a collision between.two fibres (known as a crossover)

Call:  Collision = ColFibFib(MethodInfo, pivXA, pivAY, fvpAX, fvpAY, pivBX, pivBY, fvpBY, fvpBY);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) The MethodInfo variable that was passed to the calling routine and that this routine is called via.

(>) pivAX	(double) Fibre A X ordinate

(>) pivAY	(double) Fibre A Y ordinate

(>) fvpAX	(double) Fibre A virtual pivot X ordinate

(>) fvpAY	(double) Fibre A virtual pivot Y ordinate

(>) pivBX	(double) Fibre B X ordinate

(>) pivBY	(double) Fibre B Y ordinate

(>) fvpBX	(double) Fibre B virtual pivot X ordinate

(>) fvpBY	(double) Fibre B virtual pivot Y ordinate

Returns:

(<) Collides	(int) Returns 1 if a collision exists..

Prior requirements: None.

Typedef declaration:

typedef int (* ConfigColFibFibRoutine) (

const CONFIG_MethodInfo * MethodInfo,

double  pivAX,            /* (>) Fibre A X ordinate */

double  pivAY,            /* (>) Fibre A Y ordinate */

double  fvpAX,            /* (>) Fibre A virtual pivot X ordinate*/

double  fvpAY,            /* (>) Fibre A virtual pivot Y ordinate */

double  pivBX,            /* (>) Fibre B X ordinate */

double  pivBY,            /* (>) Fibre B Y ordinate */

double  fvpBX,            /* (>) Fibre B virtual pivot X ordinate */

double  fvpBY);           /* (>) Fibre B virtual pivot Y ordinate */

### ColButFib()

Function:Checks for collisions between fibres and buttons.

Description:This routine does an instrument specific collision detection check, checking for a collision between button and a fibre.

Call:  Collision = ColButfib(MethodInfo, butX, butY, theta, fvpX, fvpY, pivX, pivY);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) The MethodInfo variable that was passed to the calling routine and that this routine is called via.

(>) butX	(double) Button X ordinate

(>) butY	(double) Button Y ordinate

(>) theta	(double) Button theta

(>) fvpX	(double) Fibre virtual pivot X ordinate

(>) fvpY	(double) Fibre virtual pivot Y ordinate

(>) pivX	(double) Fibre pivot X ordinate

(>) pivY	(double) Fibre pivot Y ordinate*/

Returns:

(<) Collides	(int) Returns 1 if a collision exists..

Prior requirements: None.

Typedef declaration:

typedef int (* ConfigColButFibRoutine) (

const CONFIG_MethodInfo * MethodInfo,

double butX,               /* (>) Button X ordinate */

double butY,               /* (>) Button Y ordinate */

double theta,              /* (>) Button theta      */

double fvpX,               /* (>) Fibre virtual pivot X ordinate */

double fvpY,               /* (>) Fibre virtual pivot Y ordinate */

double pivX,               /* (>) Fibre pivot X ordinate */

double pivY);              /* (>) Fibre pivot Y ordinate*/

### ColButBut()

Function:Checks for collisions between two buttons.

Description:This routine does an instrument specific collision detection check, checking for a collision between.two buttons.

Call:  Collision = ColButBut(MethodInfo, butXa, butYa, thetaa, butXb, butYb, thetab);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) The MethodInfo variable that was passed to the calling routine and that this routine is called via.

(>) butXa	(double) Button A X ordinate.

(>) butYa	(double) Button A Y ordinate.

(>) thetaa	(double) Button A theta.

(>) butXb	(double) Button B X ordinate.

(>) butYb	(double) Button B Y ordinate.

(>) thetab	(double) Button B theta.

Returns:

(<) Collides	(int) Returns 1 if a collision exists..

Prior requirements: None.

Typedef declaration:

typedef int (* ConfigColButButRoutine) (

const CONFIG_MethodInfo * MethodInfo

double butXa,                 /* (>) Button A X ordinate */

double butYa,                 /* (>) Button A Y ordinate */

double thetaa,                /* (>) Button A theta      */

double butXb,                 /* (>) Button B X ordinate */

double butYb,                 /* (>) Button B Y ordinate */

double thetab);               /* (>) Button B theta      */

### Collision()

Function:Checks for collisions between two buttons and their buttons.

Description:This routine does an efficient instrument specific collision detection check, checking for a collision between two buttons, and each buttons and associated fibre.

This routine does a Button-Button collision check, and a Button-Fibre check in each direction.  The virtual pivot positions will be calculated.

Call:  Collision = Collision(MethodInfo, butX, butY, theta, fvpX, fvpY, pivX, pivY);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) The MethodInfo variable that was passed to the calling routine and that this routine is called via.

(>) butXa	(double) Button A fibre X ordinate

(>) butYa	(double) Button B fibre Y ordinate

(>) thetaa	(double) Button A theta

(>) pivXa	(double) Fibre A pivot X ordinate

(>) pivYa	(double) Fibre A pivot Y ordinate

(>) butXb	(double) Button B fibre X ordinate

(>) butYb	(double) Button B fibre Y ordinate

(>) thetab	(double) Button B theta

(>) pivXb	(double) Fibre B pivot X ordinate

(>) pivYb	(double) Fibre B pivot Y ordinate */

Returns: one of the following values.

CONFIG_NOCOL  (false) if no collision.

CONFIG_COL_BUTBUT  If the collision is between the buttons.

CONFIG_COL_BUTFIB If the collision is between the button a and fibre b.

CONFIG_COL_BUTFIB If the collision is between the button b and fibre a.

Prior requirements: None.

Typedef declaration:

typedef int (* ConfigCollisionRoutine) (

const CONFIG_MethodInfo * MethodInfo,

long int butXa,                  /* (>) Button A fibre X position */

long int butYa,                 /* (>) Button A fibre Y position */

double thetaa,                  /* (>) Button A theta angle      */

long int pivXa,                 /* (>) Button A pivot X position */

long int pivYa,                 /* (>) Button A pivot Y position */

long int butXb,                 /* (>) Button B fibre X position */

long int butYb,                 /* (>) Button B fibre Y position */

double thetab,                  /* (>) Button B theta angle      */

long int pivXb,                 /* (>) Button B pivot X position */

long int pivYb);                /* (>) Button B pivot Y position */

### ColInvPos()

Function:Checks for collisions between a button and an invalid plate position..

Description:This routine does an instrument specific collision detection check, checking for a collision between a button and an invalid plate position.

Call:  Collision = ColInvPos(MethodInfo, butx, buty, theta);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) The MethodInfo variable that was passed to the calling routine and that this routine is called via.

(>) butx	(long int) Button X ordinate.

(>) buty	(long int) Button Y ordinate.

(>) theta	(double) Button theta.

Returns:

(<) Collides	(int) Returns 1 if a collision exists..

Prior requirements: None.

Typedef declaration:

typedef int (* ConfigInvPosRoutine) (

const CONFIG_MethodInfo * MethodInfo

long int  butx,               /* (>) Button X ordinate */

long int  buty,               /* (>) Button Y ordinate */

double    theta);             /* (>) Button theta      */

);

### ParkMayCollide()

Function: Indicates if park positions can collide with other fibres...

Description: The physical design of some fibre positioner instruments is such that you can be sure that a parked fibre does not collide with any other fibres.  Knowing this allows for cheaper field checks etc.  This routine allows you to determine if this if parked fibres may collide with other fibres.

Call:  Logical = ParkMayColllide(MethodInfo);

Parameters:

(>) MethodInfo	(const CONFIG_MethodInfo *) The MethodInfo variable that was passed to the calling routine and that this routine is called via.

Returns:

(<) Logical	(int) Returns 1 if a parked fibre may collide with another fibre...

Prior requirements: None.

Typedef declaration:

typedef int (* ConfigParkMayColllide) (

const CONFIG_MethodInfo * MethodInfo

);

# Detailed description of the problem

In order to write a configuration algorithm, the problem itself has to be stated as precisely as possible. This section attempts to do that.  Note that this section talks about 2dF.  Also long as an instrument can be supported by the AAO’s FPIL library, the instrument can be used with this library.

## The physical parts of the 2dF

The 2dF has two field plates. These are flat plates that can be interchanged so that at any given time the image produced by the 2dF corrector falls onto one of them and the other is placed so that the 2dF positioner can access it. The swapping of the plates between the two positions is accomplished by a tumbler mechanism that is not relevant to the configuration problem.

Associated with each field plate are just over 400 optical fibres. At the business end of each fibre is a small, roughly rectangular, magnetic button that ends in a small prism. The centre of this prism has to be placed on the field plate at the position of one of the objects of interest in the field imaged by the 2dF corrector. The fibre then feeds the light through into one of two spectrographs. The fibre runs from the magnetic button to a retractor mechanism mounted at the edge of the field plate that keeps sufficient tension on the fibre to make it form a roughly straight line between the end of the button and a pivot point at the edge of the field. The retractors, one for each fibre, are arranged in groups of tens radially around the field plate. The pivot points for each fibre are actually the points in the middle of a fibre guide (or ‘gate’) formed from two vertical rods that the fibre passes between after it leaves the retractor. If a fibre is positioned with its prism at the centre of the field plate, and such that its button lies along the radial line joining the centre of the field and its pivot point, then the fibre will be perfectly straight. Fibres can bend, but only by a limited amount.

Fibres can cross over one another. If they do so, it complicates the process of moving from one fibre configuration to another, but a configuration with crossing fibres is still valid. Indeed, most configurations will have crossing fibres.

A fibre button does not have to lie directly along the radial line joining the pivot to the target point. It may be twisted, usually in order to avoid collision with another fibre. In this case, the fibre will no longer run in a perfectly straight line but will flex at each end. However, by limiting the angles involved (which we have to do anyway for the fibres to work properly) we can treat the fibre satisfactorily using two straight lines. This is discussed in more detail in the next section.

Unallocated fibres are placed in a ‘park’ position just in front of their pivots. The geometry of the system is such that a parked pivot cannot collide with an allocated fibre. (A fibre would have to be bent a quite unacceptable amount to collide with an adjacent parked fibre, so this will be caught by the limits on non-radial pivot angle described below.)

## Covering the angles

The diagram shows some of the angles that have to be considered as part of the configuration process, particularly when fibre buttons are twisted

A 2dF button showing the various angles involved

The button is shown as a small box for the prism and a larger box for the body of the button. A third box is drawn from the end of the button to a point called the ‘virtual pivot point’. The button is shown with a non-zero ‘twist angle’ — that is, the line through the centre of the button does not lie in the same direction as does the fibre. The fibre is shown leading from the pivot point to the virtual pivot point associated with the button.

The box shown between the button and the virtual pivot point is a conceptual one. The idea of the  virtual pivot point  is that, so long as the angles involved (the non-radial pivot angle and the twist angle) are small, the path the fibre actually follows can be treated as if it were a straight line from the pivot point at the edge of the field to the virtual pivot point associated with the button. From there, the fibre will ‘snake around’ but within the confines of the box drawn between the virtual pivot point and the button itself. The way the fibre is actually attached to the button constrains it to make this approximation quite satisfactory. Obviously it is imprecise, but so long as we allow a sufficient clearance between buttons and fibres it will be good enough — and much simpler than trying to model the actual path of the fibre.

An allocation can be specified completely in terms of the pivot position for the fibre (usually specified using the index number of the pivot), the position of the target, and the angle at which the button is to be placed. All coordinates in the 2dF system are given as x,y coordinates in microns relative to a centre point for the field plate (which will approximate the actual centre of the circle of pivots, but is actually a point the positioner mechanism uses as a fiducial). The positive x-axis points to the right (3 o’clock), the positive y-axis points straight up (12 o’clock). The positions of the pivot points for the two field plates will be slightly different because of mechanical tolerances.

The 2dF documentation and code tends to refer to ‘theta’ angles for things. These angles are always defined as the angle between the vertical through some relevant origin and the thing in question, measured anti-clockwise. For example in the diagram, the angle that defines the orientation of the button, the ‘theta’ angle for the button, is the angle between the vertical through the centre of the prism of the button (this is taken as the origin) and the line through the centre of the button. This is the angle that has to be specified when an allocation of fibre to target is made.

(Other ‘theta’ angles that appear in the configuration code are what is sometimes called the ‘theta angle for the fibre’ which is the angle between the vertical through the pivot to the fibre, measured anti-clockwise — and the ‘theta angle for the button and the fibre’, which is the angle between the vertical through the virtual pivot point associated with the button and the fibre, again measured anti-clockwise. In all these cases, the line in question — the line through the centre of the button, the line formed by the fibre itself — terminates at the chosen origin point, so there is no 180 degree ambiguity in these definitions.)

## Allocation constraints

The considerations limiting the allocation of a particular fibre to a particular target point in the field are as follows:

•	The fibres have a limited length. A fibre can reach from its pivot point to a point somewhat past the centre of the field, but cannot reach all the way across the field. This constraint is expressed as a limit on the pivot to target distance for the fibre.

•	The fibres cannot be allowed to bend more than a certain amount at the pivot point. A fibre positioned directly along the radial line from its pivot point to the centre of the field is taken to be perfectly straight. The angle between this radial line and the actual line of the fibre is called the ‘non-radial pivot angle’ (sometimes ‘alpha’) and there is a limit on the size of this angle, defined as a parameter for the allocation program.

•	The fibres cannot be allowed to bend more than a certain amount as they enter the button. The angle between the line through the centre of the button and the fibre is called the ‘twist’ angle, and there is a limit on the size of this angle, again defined as a parameter for the allocation program.

•	Although fibres may cross over one another, a fibre cannot be allowed to run too close to a button. If it were to touch the button or overlay it, the fibre risks being entangled with the gripper assembly — or cut by the button itself — when the button is moved. A button-to-fibre clearance, in microns, is defined as one of the limiting parameters for the configuration. While in principle any positive clearance should be acceptable, the location of the fibre is not in fact known perfectly (partly because of the simplifying assumptions made about it when a button is allowed to twist) and so a minimum clearance is specified. (This can also be used to allow some flexibility for changes in the field with time, as explained below.)

•	Similarly, buttons cannot be allowed too close to one another. Physically, they cannot occupy the same space on the field plate, and a decent clearance must be left to allow the positioner to pick the buttons up cleanly, and also to allow for any possible magnetic interaction between buttons. A button-to-button clearance, in microns, is defined as another of the limiting parameters for the configuration.

•	Some fibres are ‘guide’ fibre bundles, rather than single fibres. These can only sensibly be allocated to guide stars in the field. Similarly, ordinary single fibres can only sensibly be allocated to ordinary target objects.

•	In some cases, the two spectrographs may be configured differently — one may be optimised for the blue, the other for the red, for example. In this case, the allocation algorithm may have to take into account the spectrograph into which each particular fibre feeds. The interfaces to the allocation algorithm allow targets to be flagged as requiring one or other of the spectrographs, or as able to be assigned to either.

•	Some objects may be regarded as being of higher priority than others. The interface to the allocation algorithm allow a numeric priority to be assigned to each target. How the algorithm handles this is up to the algorithm writer.

There are therefore five numeric parameters that apply to the configuration algorithm: the fibre length, a limit on the twist angle, a limit on the non-radial pivot angle, a button to button clearance, and a button to fibre clearance. At the time of writing, the actual safe values for these are still being determined. The writer of an allocation algorithm, however, should use the utility routines provided by the ‘housekeeping’ layer of the configuration library to get these values, rather than assuming any specific values (even those found in 2dF system include files). This is because in some cases a configuration will have to be set up with larger than usual tolerances to allow for changes in the field as the orientation of the telescope changes; a configuration that has fibres at the maximum allowed pivot angles, for example, will become invalid if used at a time different to that for which it was set up.

# The default algorithm

The default algorithm-specific code supplied with the configuration library at present uses an algorithm described as ‘Keith Taylor’s algorithm’ after he championed it in the original discussions at AAO about the best algorithm to use. The algorithm as supplied differs slightly from Keith Taylor’s original, and is the result of its being recoded in C by Keith Shortridge. It is essentially an ‘allocate the hardest pivot first’ algorithm, with the hardest pivot being defined as the one that can be allocated to the fewest targets.  The default algorithm has been modified extensively by Gavin Dalton since these notes were written and they may not reflect the current status of the algorithm.  See section 5.2 for a bit more information on this.

Briefly, the algorithm maintains a matrix whose dimensions are given by ‘number of pivots’ and ‘number of targets’. Each matrix element therefore represents a possible allocation of target to pivot. Such a possible allocation is sometimes called a ‘path’ in the code.

In the initialisation stage, this matrix is initialised allowing for allocations that are impossible on physical grounds: maximum fibre length exceeded, maximum non-radial pivot angle exceeded, incompatible on grounds of fibre type (guide fibre or ordinary fibre) or on grounds of spectrograph required.

Given this matrix, it is very quick to count the number of targets to which any given pivot can be allocated.

A new allocation is made simply by counting the possible allocations for each unallocated pivot,, and deciding on that basis which pivot to allocate next (the one with the fewest possible targets to which it can be allocated.) A choice is then made between the possible targets for the pivot, and as a result a target to pivot allocation is made.

Given this allocation, the algorithm then runs through the matrix, re-examining — in the light of this new allocation —all the allocation ‘paths’ still flagged as possible. Collisions with the newly allocated pivot will rule out a number of previously possible ‘paths’.

With the matrix updated, the process of selecting a pivot is repeated. Then the matrix is updated again, and so on until there are no possible allocations left. At this point, the algorithm usually quits.

Stated like this, the algorithm is very simple and has almost no possible variations. This is not entirely the case. There is some room to manoeuvre in the question of which of the possible targets should be allocated to the selected ‘hardest’ pivot. The current code allows this to be controlled by a parameter which can be used to select one of:

•	The ‘hardest’ target — that is, the target that can be reached by the fewest pivots.

•	The closest target — the one physically closest to the pivot.

•	The furthest target — the one physically furthest from the pivot.

•	The straightest target — the one with the smallest non-radial pivot angle.

All of these choices have slightly different characteristics in terms of overall coverage achieved, number of crossovers (fibres that have to cross other fibres, which can complicate moving to another configuration afterwards), and radial distribution of targets allocated.

An additional option made available by the existing code is a ‘tentative deallocation’ step. If this is tried, each fibre is deallocated in turn and when it is deallocated the system then repeats its normal allocation loop, allocating fibres until no possible paths are left. Following the deallocation of one fibre one new allocation can always be made — the one that has just been deallocated — so no coverage can be lost by this step. However, sometimes more than one fibre can be allocated once one has been deallocated, so there is the occasional gain. Also, often the first allocation made after the deallocation is not the fibre just deallocated, so the process does affect the characteristics of the configuration (either for the good or the bad, depending on what criteria are being used to judge the allocation).

## Tweaking the current algorithm

Anyone who wants to tinker with the current algorithm, instead of writing a new one completely from scratch, will find the routine ConfigMethodPickNext() a good place to start. This embodies the heart of the algorithm, since this is the routine that decides on the next allocation to be made. It has access to the matrix of possible paths, and all the pivot and target information. Almost any algorithm that did not rely on being able to deallocate fibres could in principle be implemented just by changing this routine, although algorithms that did not make use of the path matrix would hardly be well-served by the structure of the current code.

The overall structure of the algorithm can be seen by looking at the code for ConfigMethodAllocate(), which will be seen to consist mainly of a loop that calls ConfigMethodPickNext(), updates the matrix according to the newly allocated fibre, and then calls ConfigMethodPickNext() again until it reports that it can find no possible allocations left to make

## Gavin’s modifications

As mentioned in the introduction to this section, the default algorithm has been modified extensively by Gavin Dalton (University of Oxford).  It is unclear to this author (TJF) how the modifications he has made relate to the above description, but this is the standard description of Gavin’s algrohtim as of 10-Mar-2000.

The algorithm currently used is very optimized and has proved to give the best results for “typical” fields. After an initial allocation pass it searches down a tree of multiple fibre swaps, looking for swaps which give increased allocations. The algorithm is not unlike a chess program. The tree search terminates at a depth of ten swaps where it is has been found, empirically, that the expenditure of CPU time required to deepen the search is not rewarded by increased allocations.

The algorithm handles objects of different priorities by trying to allocate the highest priority objects first. During the swapping process it will continue to search until it becomes possible to allocate a fibre which was previously parked, or it becomes possible to promote an allocated fibre to a higher priority object.

After the swapping phase there is a final `uncrossing' pass which looks at all pairs of fibres which cross to see if they can be reversed. This is important, as reducing the number of fibre crossings in the final configuration provides a significant reduction in the field-field setup time by reducing the numbers of fibres that must be parked in transit, but this reduction is provided without constraining the allocation itself.