|  | ANGLO-AUSTRALIAN OBSERVATORY P.O. BOX 296 EPPING N.S.W. 2121 AUSTRALIA Telephone: +61 2 9372 4800 Fax: +61 2 9372 4880 |
| --- | --- |

FLAMES FIBRE POSITIONER

| Document Number: | INS-MAN-AUS-13721-0080 |
| --- | --- |
| Issue Number: | 1.5 |
| Issue Date: | 3 Dec 2002 |

FPOSS Internals

| Prepared: | Keith Shortridge | 3 Dec 2002 |  |
| --- | --- | --- | --- |
|  | Name | Date | Signature |
| Approved: | Chris Evans | 3 Dec 2002 |  |
|  | Name | Date | Signature |
| Released: | Denis Whittard | 3 Dec 2002 |  |
|  | Name | Date | Signature |

- Change Record

| Issue / Revision | Date | Section / Page affected | Reason / Remarks |
| --- | --- | --- | --- |
| 1.0 | 26/9/01 | All | First Issue |
| 1.1 | 03/12/01 | 5,7.7,9.1 | Minor updates and formatting changes |
| 1.2 | 14/02/02 | Most | Expert mode description added, numerous minor changes. |
| 1.3 | 04/06/02 | 8.6 | Updated tolerance values in table |
| 1.4 | 22/11/02 | 8.14,9 | Added section on keeping FPOSS and FP in sync. |
| 1.5 | 03/12/02 | 8.3, 8.4 | Expert mode menu contents modified.. |

FPOSS Internals

- Contents

1	Introduction	7

1.1	Scope	8

1.2	Applicable Documents	8

1.3	Reference Documents	8

1.4	Abbreviations and Acronyms	9

1.5	Glossary	9

1.6	Nomenclature	9

1.7	Stylistic Conventions	10

2	What Does FPOSS Do?	11

3	Language issues	12

4	Building FPOSS	13

5	Program Structure and Data Structures	14

5.1	Pivot Data	14

5.2	Configuration Data	15

5.2.1	The fieldData sub-structure	17

5.2.2	The unallocGuide sub-structure	18

5.2.3	The unallocObj sub-structure	19

5.2.4	The objects sub-structure	19

6	Sub-systems	21

7	A Walk through the code	23

7.1	Startup	23

7.2	Tcl/Tk startup	24

7.3	Basic sequence – Magnitude Filter	25

7.4	Basic sequence – Open File	25

7.5	Basic sequence – Select Guide Star	27

7.6	Basic sequence – Fibre combination	27

7.7	Basic sequence – Allocate	28

7.8	Basic sequence – Write PAF	29

8	Aspects of the Code	30

8.1	Tcl/Tk ancillary files	30

8.2	Error reporting	32

8.2.1	Errors generated by Tcl	33

8.2.2	Errors detected and handled entirely in C.	34

8.2.3	Errors detected by Tcl code	34

8.3	Use of canvas tags in the mimic display	36

8.4	What Routines Does FPIL call?	38

8.5	Calling Tcl from C	38

8.6	Allocation tolerances	39

8.7	Makefiles	40

8.7.1	Targets	40

8.7.2	Sub-directories	40

8.7.3	System-dependent Makefiles	41

8.7.4	RCS version number	41

8.8	Changing Tcl and Tk versions	41

Source files	43

8.9	DRAMA files	43

8.9.1	General-purpose files	43

8.9.2	Ers files	44

8.9.3	Mess files	44

8.10	SDS Files	44

8.11	SLALIB files	45

8.12	FPCOL files	46

8.13	Configure (FPOSS) files	47

8.13.1	FPIL files	48

8.13.2	Allocation algorithm files	49

8.14	Additional files	49

8.15	Documentation files	50

8.16	Makefiles	50

9	Keeping FP and FPOSS in sync	51

10	Expert Mode	53

10.1	Invoking expert mode	53

10.2	Menus in expert mode	53

10.2.1	The  File menu in expert mode	53

10.2.2	The View menu in expert mode	54

10.2.3	The Options menu in expert mode	55

10.2.4	The Commands menu in expert mode	56

10.3	Additional target type codes	58

10.4	The Allocation Dialogue in Expert mode	58

Figure 1 Overall FPOSS structure

FPOSS Internals

# Introduction

This document describes the internals of the fibre configuration program, FPOSS. It is intended to be read by software people who need to know their way around the code that makes up FPOSS. You may be reading this because  you have been asked to fix a bug in the program, or because you need to understand something about the way it is built and released, or because you need to know in detail just what it is doing. You may merely be curious about it.

FPOSS is a relatively large program. This document is intended to help you as you read the code. It tries to give some idea of the structure of the  program and to explain some of the philosophy (and in some cases the history) behind the design.

We start with a description of how to build the system. To understand the code, you need to have used the program and that means you need to build it. Once the program is built and running, if you are not familiar with its operation you should work through the the INS- FPOSS User Manual [12] (the one written for the astronomers needing to prepare configurations for FLAMES), which begins with a step by step walk through the preparation of one such configuration.

Having seen the system in action, the next section of this document describes the overall program structure. FPOSS makes use of a number of reasonably well- separated modules, some of which are general-purpose sub-systems in their own right. This section tries to explain how these different sections of the code fit together, and in particular it describes the data structures at the heart of the program.

The best way to understand a piece of code is to follow its execution as it performs a standard task. You know what it is doing, now you want to open up the black box and watch the parts going round. You can learn a lot about a program by following it through with an interactive debugger. The next section in this document tries to do this for you, talking you through what the  program is doing as it performs the example configuration at the start of the astronomer's User Manual. Obviously, a lot of detail has to be skipped here, but you should end up with a feel for how the program works.

Next, this document has a miscellaneous section that goes deeper into some of those sections of the code that have proven hard to follow at times. For example, the error reporting is quite complex and this is described in considerable detail.

Finally, there is then a list of code files, with brief descriptions of their roles.

I hope this document allows you to understand and to work with FPOSS. Although it is big, it is reasonably well-structured, and I hope this document helps to clarify that structure.

## Scope

FPOSS has its origins in the configuration program written some time ago at AAO for the 2dF fibre positioner. Since then, this program has been extended to support both the AAO's new 6dF fibre positioner and ESO's FLAMES system. The program can be built to support all three of these instruments (in which case the user has to select the instrument in question before doing anything else), or it may be built to support just one instrument. It is when the program is built just to support the ESO FLAMES instrument, and is distributed as part of the P2PP system for FLAMES, that it is known as FPOSS. (At AAO it is usually known just as ‘configure’.)

This document has been written specifically for the FLAMES version of the program. It should be regarded as a FLAMES-specific manual. Although it obviously contains information relevant to the 2dF and 6dF versions of the program, it has been written to simplify the job of ESO maintenance programmers, and contains almost no details about 2dF and 6dF-specific code.

## Applicable Documents

The following documents, of the exact issue shown, form a part of this document to the extent specified herein.

- VLT-SPE-ESO-10000-0011, 2.0 30/09/1992 -- VLT Software Requirements Specification

- VLT-SPE-ESO-17212-0001, 2.0 12/04/1995 -- VLT Software - VLT Instrumentation Software Specification

- VLT-SPE-ESO-17240-0385, 2.1 15/07/1996 -- VLT Software - INS Common Software Specification

- VLT-PRO-ESO-10000-0228, 1.0 10/03/1993 -- VLT Software - Programming Standards

- VLT-MAN-SBI-17210-0001, 3.4 05/05/97 -- VLT Software - LCU Common Software, User Manual

- VLT-MAN-ESO-17210-0690, 3.3 30/10/98 -- VLT Software – Graphical User Interface, User ManualVLT-MAN-ESO-17210-0667, 1.0 03/12/96 -- VLT Software – Guidelines for the Development of VLT Application Software

- VLT-PLA-ESO-00000-0006, 2.0 21/05/92 – VLT Program – Software Management Plan.

- INS-SPE-AUS-13721-0031, 2.0 29/09/1999 -- Fibre Positioner, Software Requirements and Software Functional Analysis

- INS-SPE-AUS-13721-0042, 1.0 29/09/1999 -- Fibre Positioner, Instrument Control Software Design Description

- VLT-SPE-ESO-13730-1816, 1.0 12/04/1999 – Flames Observation Software Design Description

## Reference Documents

The following documents are referenced in this document:

- INS-INS-AUS-13721-0017 Flames Fibre Positioner, Glossary of Terms

- INS-MAN-AUS-13721-0079 --- FPOSS User Manual

## Abbreviations and Acronyms

The following abbreviations and acronyms may be used in this document:

| 2dF | The AAO’s 2 degree Field Facility. |
| --- | --- |
| 6dF | The AAO’s 6 degree Field Facility |
| AAO | Anglo-Australian Observatory |
| ARG | The ARG (argument) set of routines – part of SDS |
| ARGUS | Integral Field Unit (fixed) |
| DRAMA | The AAO’s data acquisition environment (not an acronym). |
| ERS | The Error Reporting System used by DRAMA and FPOSS |
| ESO | European Southern Observatory |
| FACB | Field Acquisition Coherent Bundle |
| FLAMES | The Whole Fibre Facility |
| FP | Fibre Positioner |
| FPIL | Fibre Positioner Interface Library |
| FPCOL | The fibre positioner collision library |
| FPOSS | Fibre Positioner Observer Support Software |
| GUI | Graphical User Interface |
| IMP | DRAMA message passing sub-system. Not used by FPOSS. |
| MESS | Message routines – a DRAMA library used by FPOSS |
| OS | Observation Software |
| OSS | Observer Support Software |
| OZPOZ | The name given to the Fibre positioner |
| PAF | The text file format used internally by VLT software |
| P2PP | Phase 2 proposal preparation |
| SDS | Structured Data System.- a DRAMA library used by FPOSS |
| SLALIB | Starlink Library A (Pat Wallace’s astrometric library). |
| TBD | To Be Defined |
| TCL | Tool Command Language |
| Tk | The Windowing Toolkit.used by Tcl programs |
| VLT | Very Large Telescope |

## Glossary

A glossary of terms specific to the Fibre Positioner is given in the separate document OzPoz Glossary of Terms [11].

## Nomenclature

When a routine is shown as name{} it is a Tcl procedure, although it may turn out to be implemented in C (in which case the C code for it will be found in configure.c). When a routine is shown as name() it is a C routine.

The program described in this document is referred to almost interchangeably as ‘configure’ or as ‘FPOSS’. ‘Configure’ is its traditional name at AAO, where it is usually used to conffigure fields for the 2dF and 6dF instruments. When built for ESO’s FLAMES instrument, it is more correctly called ‘FPOSS’ – Fibre Positioner Observer Support Software. The reader should be aware that both names refer to the same program, which is also sometimes referred to as ‘the configuration program’.

Similarly, the instrument for which FPOSS creates configurations is sometimes referred to as FLAMES, which is the correct ESO name for the complete instrument. However, in places it may occasionally be referred to as ‘OzPoz’, which is the name of the fibre positioner that forms part of FLAMES. Whether one thinks of a configuration as being produced for FLAMES or for OzPoz is really just a matter of one’s point of view.

## Stylistic Conventions

The following styles are used:

bold

in the text, for subroutine and procedure names and for file names.

italic

in the text, for emphasis.

teletype

for code samples

<name>

in the examples, for parts that have to be substituted with the real content before typing

# What Does FPOSS Do?

In order to understand FPOSS, it’s important to understand its place in the scheme of things. It is run as part of the P2PP stage of proposal preparation for the FLAMES instrument. The observer has a list of target objects, possibly with priorities assigned to them. The list should also include guide stars that can be assigned to the FACB guide fibre bundles that are controlled directly by FLAMES, and should also include at least one star that can be used as a VLT guide star. The job of FPOSS is to allocate the various fibres available as part of the FLAMES instrument, in whatever configuration is being used, to these targets.

The resulting list of assignments of fibres to targets is written out as a PAF file and this PAF file is then associated with an observation block generated by the FLAMES P2PP software. Some time later, at the telescope, when the time comes for this observation block to be executed, the VLT system software will arrange for this PAF file to be passed on to the fibre positioner observation software which will work out where on the field plate the targets will be and will position the assigned fibres at the correct locations.

That is a very brief summary of the role of FPOSS. For more detail consult the the the INS- FPOSS User Manual [12]. The following points may be worth noting, since they have implications for the way FPOSS operates and has been coded:

- The FPOSS code is an extension of code originally written for the AAO’s 2dF instrument. This means that it takes advantage of a large body of tried and trusted code, but that this code does not have the overall structure one might expect had this been written from the start as a VLT program. For example, an RTD plug-in might have been used for the display rather than a standalone canvas widget.

- FPOSS is only run as part of the P2PP process. It is not intended to be run at the VLT in any sort of stand-alone mode. If run at all at the telescope, it has to be in the context of a repeat of the P2PP procedure. (This is not the way the AAO runs the equivalent program for 2dF and 6dF, where a batch mode of operation is quite common. This means that although the FPOSS code has a deal of support for a batch mode, this is not used at ESO.)

- FPOSS is a standalone program that is expected to be run by observers on their home systems. It will be downloaded from ESO as an executable (for either HP-UX, Solaris, or Linux).

# Language issues

The structure of FPOSS is that of a relatively straightforward Tcl/Tk program. The majority of the code is in standard C (not C++). Tcl/Tk code is used for the user interface, and once FPOSS has been initialised it operates as an event-driven Tcl/Tk program. The user does something with the GUI, and as a result a Tcl command tied to the interface item in question is invoked. In many cases, these Tcl commands are implemented in C. There is a relatively thin layer of Tcl-specific C code which acts as an interface between the Tcl/Tk GUI code and the C modules that do most of the work. In particular, all the allocation code is written in C.

The C code is almost all portable, ANSI C. It has been compiled under a large number of different compilers. Test versions of FPOSS have been produced for both Macintosh and Windows systems and, although these are not formally supported and will not be made available to observers, the fact that the code works on such different systems suggests that it has been written in a very portable way. With one exception, all the C code will compile without warnings using the GCC compiler with the –Ansi –Wall flags set. (The exception is under Solaris where one module includes some X include files and the version of these include files used at AAO is incompatible with these flags. The fault is with the X include file, not with the FPOSS code.)

The Tcl/Tk version used for the current release of the system is 8.0. There are not believed to be any particular portability issues that relate to Tcl versions, although as always with Tcl/Tk it is important to make sure that the libraries and include files used are for the same Tcl/Tk versions. (You cannot, for example, compile with 8.3 include files and link against 8.0 libraries – the name of the main Tk routine changed between the two versions!).

Although the current version uses Tcl/Tk 8.0, the majority of the Tcl/Tk code was written for much earlier versions, and a number of constructs in the code appear old-fashioned in places. For example, almost no use is made of namespaces, which would have simplified some of the use of global variables. In additon, a number of C routines that implement Tcl commands return values by formatting directly into interp->result, which is no longer the approved method. FPOSS is known to run using Tcl/Tk 8.3. Tcl/Tk version considerations are discussed further in section 8.8.

# Building FPOSS

FPOSS exists as a single module held under source control at ESO. There is a main fposs directory which has a number of sub-directories containing the source for the different sub-systems used. The FPOSS system is entirely self-contained. There is a sub-directory called ‘src’.

To build the program, set your default to the src directory, and type:

Make clean

to remove any intermediate files created from earlier builds (which may have been for different target systems), then type:

make

This will build the default target, which is the configuration program. (There should be no errors and no warnings generated by the compiler or the linker. If there are any, they should be checked carefully.) The executable is called ‘configure’, and the program can be tested in situ using

./configure &

Once a working version has been created, a new release can be created by:

make release

which will create a new gzipped tar file that can be downloaded by observers. Note that a quirk of the way Tcl/Tk is handled means that the DISPLAY environment variable must be set in order for the release target to be built. Nothing is actually displayed, but the build sequence involves starting up Tcl/Tk in order to check on the version numbers for both Tcl and Tk, and this will fail if DISPLAY is not set.

What the ‘release’ target actually creates is a file called ‘fposs-m.n.system.tar.gz’ where ‘m.n’ is the RCS version number of the current version and ‘system’ is the name of the system for which it was built, as returned by the ‘uname’ command. This is the file that a user will expect to download from ESO. Note that it contains an executable built for the current system. This means that to produce a new version for all systems, you should get the new version working and tested (on all systems) then check the new version in to Cmm, then check it out using cmmCopy on all systems in turn and do a ‘make release’ on all systems.

# Program Structure and Data Structures

The overall structure of FPOSS is shown in the diagram at the start of this document. This is a very simplified diagram, but it shows the two main data structures that are central to the program. Both of these are SDS data structures – hierarchical structures of data that contain sufficient meta-data (data about the data they contain) to allow them to be searched for named items, for the size and type of items to be requested. All access to these data structures has to be through the SDS library. From C, the SDS routines can be called directly, from Tcl there are Tcl interfaces provided for all the C Sds calls. All SDS items can be referenced through SDS ‘Id’s, which are SDS-generated identifiers associated with those SDS items.

## Pivot Data

The smaller of the two structures is called ‘Pivot data’. This contains the details of the field plate and the fibres mounted on it. This is read by the routine FpilConFieldId() which is called from the C implementation of the Tcl command Initialise{}. The SDS Id for the top level of this structure is held in the C global variable (in configure.c) called InstFieldDetails. The Tcl command PivotsId{} returns this Id for use by the Tcl level. The following output was generated by temporarily inserting a call to SdsList() after the call to FpilConFieldId() had completed.

field0      Struct

type         Uint  [264]{0,2,8,2,5,2,3,2,7,2,0,2,4,2,0,2,6,2,0,...}

inUse        Ushort [264] {0,1,1,1,1,1,1,1,1,1,0,1,1,1,0,1,1,1,0,...}

xPiv         Int    [264] {460000,459869,459478,458827,457917,...}

yPiv         Int    [264] {0,10946,21887,32816,43725,54610,65464,...}

xPark        Int    [264] {435000,434876,434507,433891,433030,...}

yPark        Int    [264] {0,10351,20698,31032,41349,51642,61906,...}

tPark        Double [264] { 4.71238898, 4.736186674, 4.759988595,...}

UPDATE_TIME  Int    998012454

(Inserting calls to SdsList() at various points in the program can be a very useful diagnostic, if you want to follow what is happening to the SDS structures.)

You can see that there are 264 possible fibre positions around the plate, not all of which are in use. The type codes for each fibre indicate what sort of fibre it is (UVES, Medusa, FACB, etc). The meaning of the type codes is internal to the FPIL routines. Generally, the FPOSS code obtains the type code for a fibre by calling FpilConfWhichSpec(). The important thing is the the FPIL layer knows what fibre type codes are compatible with what target  type codes. The xpiv and ypiv items give the coordinates (in microns from the plate center) of the pivot points for each fibre, and the xpark and ypark items given the coordinates of the park positions for each pivot. The UPDATE_TIME item is the time at which this information was generated (a standard UNIX time value).

The pivot information structure is left unchanged during the running of FPOSS, so its use is relatively easy to follow.

## Configuration Data

The other, more complex, data structure is shown in the diagram as ‘Configuration Data’. This is an SDS structure that changes throughout the running of the program. If you understand what happens to this structure, you understand most of the operation of FPOSS.

When the program starts up, and even when it has been fully initialised, the configuration data does not yet exist. It has to be generated.

The diagram shows the configuration data being read in in one of three ways:

- It can be read from a .fld file. This is the usual way to generate it when FPOSS is being run as part of the P2PP process. The routines in tdFparse.c parse an input text file (usually with a .fld extension) and create the configuration data from it. When this happens, the configuration data will show no fibres currently allocated.

- It can be read from an SDS format file. It is possible to save the configuration data in an SDS file during the running of the program, and then to re-read this when the program is re-run. In this case, the configuration data will be just as it was when it was saved. This is quite common when the configuration program is run at AAO, but is expected to be very rare when it is run as FPOSS at ESO.

- It can be read from a PAF format file. The effect is to reproduce the configuration data as it was (usually in a previous run of the program) when it was saved as a PAF file. Note that information about unallocated targets is not normally saved in a PAF file (although it is when the program is running in ‘expert’ mode), so usually this does not allow the configuration to be changed much. But it can be used to examine a configuration recorded in a PAF file.

The format of the configuration data is as follows. Note that because the SDS structure is dynamic, not all items may exist at any given time. The following list was generated by running the program to completion, ie performing a complete allocation, then saving the configuration data to an SDS format file, then running the sdslist utility on that file to show its contents.

2df_field            Struct

fieldData            Struct

cenRa                Double 2.803704183

cenDec               Double -0.004574992783

appRa                Double 2.80397922

appDec               Double -0.004698394006

appEpoch             Double 52009

configMjd            Double 52009.09078

unallocObj           Short  158

unallocGui           Short  21

unallocSky           Short  -1

label                Char   [80] "F855 Test 10..."

progId               Char   [80] " ..."

mode                 Char   [10] "NORMAL"

allocSky1            Short  0

allocSky2            Short  0

allocSky3            Short  1

allocSky4            Short  0

allocSky5            Short  0

allocSky6            Short  0

allocSky7            Short  0

allocObj             Short  29

allocSky             Short  1

unallocGuide         Struct

name                 Char   [80,21] "F0033..."

ra                   Double [21] { 2.80546842, 2.800149529,...}

dec                  Double [21] { 0.002546435379, 0.002175795319,...}

x                    Int    [21] {900344,857048,610792,558857,266680,...}

y                    Int    [21] {223084,-451282,4306,401737,707788,...}

type                 Char   [21] "FFFFFFFFFFFFFFFFFGGGG"

dhdx                 Double [21] { 1.711851856e-10, -3.219437965e-10,...}

dddx                 Double [21] { 7.218398026e-09, 7.265166148e-09,...}

dhdy                 Double [21] { -7.866951769e-09, -7.707960513e-09,...}

dddy                 Double [21] { -1.714844353e-10, 3.214660539e-10,...}

priority             Short  [21] {9,9,9,9,9,9,9,9,9,9,9,9,9,9,9,9,9,2,...}

magnitude            Double [21] { 9.62, 9.73, 9.9, 10, 9.96, 9.75,...}

pId                  Int    [21] {0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,...}

comment              Char   [80,21] " ..."

unallocObject        Struct

name                 Char   [80,157] "F855_526..."

ra                   Double [157] { 2.804812467, 2.806769417,...}

dec                  Double [157] { 0.00401818427, 0.003566822733,...}

x                    Int    [157] {1108317,1050694,1030725,1028224,...}

y                    Int    [157] {142978,395618,-63364,-36766,273069,...}

type                 Char   [157] "PPPPPPPPPPPPPPPPPPPPPPPPPPPPPPPPPPPP..."

spectrograph         Char   [157] "..."

priority             Short  [157] {2,2,2,2,2,2,2,2,2,2,2,2,2,2,2,2,2,2,...}

magnitude            Double [157] { 19.22, 19.27, 19.35, 19.32, 19.32,...}

pId                  Int    [157] {0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,...}

comment              Char   [80,157] " ..."

objects              Struct

name                 Char   [80,264] "Parked..."

ra                   Double [264] { 0, 2.803212582, 0, 2.803614007, 0,...}

dec                  Double [264] { 0, -0.002833784447, 0,...}

fibre                Short  [264] {0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,...}

x                    Int    [264] {445000,209579,444495,156856,442985,...}

y                    Int    [264] {0,-59168,21173,-10833,42299,98449,...}

theta                Double [264] { 4.71238898, 4.985519, 4.759986834,...}

type                 Char   [264] "XPXPXPXPXPXPXPXXXPXXXPXXXXXXXXXXXPXX..."

priority             Short  [264] {1,2,1,2,1,2,1,2,1,2,1,2,1,2,1,1,1,2,...}

magnitude            Double [264] { 0, 19.23, 0, 19.33, 0, 19.25, 0,...}

spectrograph         Char   [264] "                                    ..."

pId                  Short  [264] {0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,...}

dhdx                 Double [264] { 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,...}

dhdy                 Double [264] { 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,...}

dddx                 Double [264] { 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,...}

dddy                 Double [264] { 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,...}

comment              Char   [80,264] "Parked..."

The configuration data is regarded as being ‘owned’ by the Tcl layer. The SDS Id associated with the top level of the configuration data structure is held in the Tcl global variable Cfid. When the program starts up, this is set to zero, and there are numerous places in the Tcl code where you will find tests checking to see if this is non-zero. If it is non-zero, this means that the configuration data structure has been created. In some cases, the Tcl code will make a complete copy of the configuration data before attempting a change to it. This allows it to revert to the original version if there are any errors during the code that effects the change. When a Tcl procedure implemented in C needs to access the configuration data, the Tcl code will pass either Cfid itself to the C code, or will get an Sds id for one of the sub-structures and pass this to the C code. Note that an SDS Id is just an integer, so there is no problem passing it as an argument to a Tcl procedure.

The fact that the overall structure is called ‘2df_field’ is historical. The top level of the structure has four items, all structures. These are the fieldData, unallocGuide, unallocObject, and the objects sub-structures.

### The fieldData sub-structure

This sub-structure contains the details of the overall target field. It has its central coordinates, the field label, and so on, and it also contains a summary of the configuration to date – the number of guide targets allocated, the number of sky targets allocated, etc.

| cenRa | Double | Mean Field center, RA, in radians |
| --- | --- | --- |
| cenDec | Double | Mean Field center, Dec, in radians |
| appRa | Double | Apparent field center, RA, in radians |
| appDec | Double | Apparent field center, Dec, in radians |
| appEpoch | Double | Epoch of apparent coordinates |
| configMjd | Double | Mean Julian Date for which configuration was made |
| unallocObj | Short | Number of unallocated non-guide objects |
| unallocGui | Short | Number of unallocated guide objects |
| unallocSky | Short | Number of unallocated sky objects |
| label | Char | Label for the field |
| progId | Char | Program Id |
| mode | Char | Not used |
| allocSky1 | Short | Not used |
| allocSky2 | Short | Not used |
| allocSky3 | Short | Not used |
| allocSky4 | Short | Not used |
| allocSky5 | Short | Not used |
| allocSky6 | Short | Not used |
| allocSky7 | Short | Not used |
| allocGui | Short | Total number of allocated guide objects |
| allocObj | Short | Total number of allocated non-guide objects |
| allocSky | Short | Total number of allocated sky targets |

Note that the sdslist output above does not include an entry for allocGui. This is because the allocation did not in fact include any allocated guide objects, and so the allocGui item was never created. This sort of thing is a feature of SDS based programs – some items may simply not exist at any given time, because so far they aren’t needed. The sdslist output should be regarded as representative, but not necessarily comprehensive.

The allocSky1 through allocSky7 items were introduced at one time to try to handle the question of distinguishing between sky targets allocated to different fibre types. To date, this has not been properly implemented.

The fieldData sub-structure is always created when a .fld file is read in. The mean RA and Dec are read from the .fld file. The apparent RA and Dec are calculated once the Mjd is known, and the Mjd is calculated (unless a specific observation time was specified in the .fld file) to put the field on the meridian.

### The unallocGuide sub-structure

The unallocGuide sub-structure contains information for each target classified as a guide target that has not been allocated to a fibre. The size of the arrays in this structure change as the program runs and more guide targets are allocated.

| name | Char | Object name |
| --- | --- | --- |
| ra | Double | Mean Object RA in radians |
| dec | Double | Mean Object Dec in radians |
| x | Int | X position on field plate in microns from center |
| y | Int | X position on field plate in microns from center |
| type | Char | Encoded target type – code depends on instrument |
| dhdx | Double | Change in hour angle per change in plate x position |
| dddx | Double | Change in dec per change in plate x position |
| dhdy | Double | Change in hour angle per change in plate y position |
| dddy | Double | Change in dec per change in plate y position |
| priority | Short | Object priority 1-9 |
| magnitude | Double | Object magnitude |
| pId | Int | Program Id associated with target |
| allocated | Int | Flag set if and when the target is allocated |
| comment | Char | Comment associated with target |

Most of this structure is filled in when the target details are read from a .fld file. The X and Y plate coordinates are calculated once the observation time has been determined. The Tcl procedure ConvertXY{}, which simply invokes ConfConvertXY(), performs all the coordinate conversion. For guide stars, it also records the delta changes in RA and Dec with changes in X and Y. These are needed by the guiding software built in to the positioner, and it’s convenient to have these calculated by the configuration program.

The allocated item is used while the program is running, but is removed before the structure is written out to a file. Although SDS can dynamically resize arrays continuously, it adds overhead to the program and also confuses things, since the meaning of index values changes continually. Instead, the allocated item is used to flag targets that can eventually be removed from this sub-structure.

### The unallocObj sub-structure

The unallocObj sub-structure contains information for each target classified as a non-guide target that has not been allocated to a fibre. The size of the arrays in this structure change as the program runs and more targets are allocated.

| name | Char | Object name |
| --- | --- | --- |
| ra | Double | Mean Object RA in radians |
| dec | Double | Mean Object Dec in radians |
| x | Int | X position on field plate in microns from center |
| y | Int | X position on field plate in microns from center |
| type | Char | Encoded target type – code depends on instrument |
| spectrograph | Char | Spectrograph to which target is assigned |
| priority | Short | Object priority 1-9 |
| magnitude | Double | Object magnitude |
| pId | Int | Program Id associated with target |
| allocated | Int | Flag set if and when the target is allocated |
| comment | Char | Comment associated with target |

Most of this structure is filled in when the target details are read from a .fld file. The X and Y plate coordinates are calculated once the observation time has been determined. The Tcl procedure ConvertXY{}, which simply invokes ConfConvertXY(), performs all the coordinate conversion.

Note that most of the fields here are the same as for the unallocGui sub-structure, except that the delta coordinate change values are no needed. The spectrograph field is intended for cases where it is important to distinguish between one or more spectrographs that an object might be assigned to. This was sufficient for 2dF, where all targets could in principle be allocated to one of two spectrographs. For instruments such as FLAMES, where there are a large number of different target and fibre types, this idea had to be revised. In practice, the way the program now handles targets that have to be allocated to different instruments is to use a much greater variety of target type, and the spectrograph field is much less important. (In 2dF, almost all non-guide targets have a type of ‘P’ (program) with an optional spectrograph suffix of 1 or 2. For FLAMES the target type can be one of a number of different single letters, and the spectrograph field is not used.)

### The objects sub-structure

This structure is not initially created when a .fld file is read in. Only once targets start to be allocated is it needed. The routine ConfMakeAllocated() is used to create this sub-structure. The name (‘objects’) is probably misleading. This sub-structure is in fact tied very closely to the fibres rather than to the target objects. It records the objects allocated to each fibre. The size of its arrays is fixed, there being one entry for each fibre. Initially, every fibre has an entry with a type code set to a special ‘unallocated’ code (usually ‘X’, but in principle instrument-dependent – it needs to be a letter not used as a target type for that instrument).

Once the fibre is allocated to a target, the type code is changed to indicate the type of the target in question, and the other items in the structure are changed to contain the relevant details for the allocated target.

| name | Char | Object name |
| --- | --- | --- |
| ra | Double | Mean Object RA in radians |
| dec | Double | Mean Object Dec in radians |
| fibre | Int | Unused |
| x | Int | X position on field plate in microns from center |
| y | Int | X position on field plate in microns from center |
| theta | Double | Button angle |
| type | Char | Encoded target type – code depends on instrument |
| spectrograph | Char | Spectrograph to which target is assigned |
| dhdx | Double | Change in hour angle per change in plate x position |
| dddx | Double | Change in dec per change in plate x position |
| dhdy | Double | Change in hour angle per change in plate y position |
| dddy | Double | Change in dec per change in plate y position |
| priority | Short | Object priority 1-9 |
| magnitude | Double | Object magnitude |
| pId | Int | Program Id associated with target |
| index | Int | Index into unallocGui or unallocObj structure |
| comment | Char | Comment associated with target |

The index item is not saved when the structure is written to disk. It works with the allocated items in the other sub-structures to reduce the amount of reorganisation that is needed when fibres are allocated and deallocated constantly.

When a configuration is complete – usually at the point just before it is written out to a file, ConfCompressUnalloc() is called. This removes the index and allocated items from the objects and the unallocated target lists, and removes the allocated targets from the unallocated target lists. ConfAddIndex() then has to be called again to correct any indexing issues arising from the changes to the number of items in the various sub-structures. The routine ConfUpdateFibreCounts() is a good place to look for example code that uses these structures.

# Sub-systems

FPOSS has its origins in the configuration program written some time ago at AAO for the 2dF fibre positioner. Since then, this program has been extended to support both the AAO's new 6dF fibre positioner and ESO's FLAMES system.

The program can, in fact, be built to support all of these instruments, but the version released for ESO has been built only with support for FLAMES.

Building the full version is simple - the code needs to be compiled without the FLAMES_ONLY pre-processor variable set, and linked with the 2dF and 6dF instrument-specific libraries as well as the FLAMES-specific library. The Makefile_Solaris makefile has a target 'configure_full' which does  perform such a build, but it needs the additional instrument-specific libraries that are not released with the ESO version of the system. This has some consequences for the structure of the program.

The need to support multiple instruments has led to the introduction of an instrument-independent layer, called FPIL (Fibre Positioner Instrument Library). FPOSS should access all instrument-specific information through this layer. FPIL defines a set of interfaces that instrument-specific libraries have to follow, and can then be linked with any number of such libraries. The library provided for FLAMES is called fpcol (standing for 'Fibre Positioner COLlision, since at one point abstracting the fibre collision routines was its main purpose). See the section called "What routines does Fpil call?" for more details about this aspect of the program.

The fact that the original AAO configuration program was written to run under the AAO's DRAMA instrumentation environment means that a number of  facilities commonly used at AAO were made use of when it was written.  For example, the AAO's SDS (Structured Data System) was used to manipulate the data structures that are central to the operation of the program. Similarly, error reporting was performed through the AAO's Ers (Error Reporting System), and error message codes were translated to text using the 'Mess' sub-system.  There is also a small layer called Arg (for 'Arguments') which provides a simple interface to some SDS functions.

FPOSS still uses these sub-systems. This does not make FPOSS a DRAMA program, because it isn't.  It is a program that uses some libraries developed at AAO that are also used by DRAMA. These four small sub-systems, SDS, Mess, Arg and Ers, are the only such general-purpose libraries used by FPOSS. To enable FPOSS to be built and run outside AAO, the code for the SDS, Arg, and Mess libraries has been supplied with FPOSS, and a stand-alone version of Ers (called ers_imp, since it was originally developed to allow the AAO's IMP message system to be used as a standalone system) has also been included.

Finally, FPOSS, like almost all other professional programs that do astrometry, uses Pat Wallace's SLALIB library.

So, in summary, FPOSS uses the following sub-systems:

| SDS | Structured Data System. Provides access to hierarchical, machine-independent, self-defining data structures, stored either as files or in memory. ('Self-defining' means that each item in the hierarchical structure is stored with embedded 'meta-data' -data about data - that names the item, and gives its type size and other characteristics. This meta-data can be used by enquiry functions to find out about what information is held in a structure.) |
| --- | --- |
| Arg | Argument access. Provides simple access to SDS structures where a number of primitive items are at the same level of the structure.These 'flat' structures are commonly used by DRAMA programs to pass arguments when one task requests another to perform a given action, hence the name. |
| Ers | Error Reporting System. Provides a system where a layer that discovers an error condition (and which therefore knows all the necessary details) can report fully on the error without having to worry that the error is unimportant and will be worked around by higher-level code. It allows errors to be annulled, so that in such cases the errors messages output by lower levels are not seen. |
| Mess | Messages. Provides a simple way of associating error codes with error text and obtaining the text associated with a given error code. |
| FPIL | Fibre Positioner Interface Library. A set of routines that provide an instrument-independent interface to the various instruments supported by FPOSS. |
| fpcol | The FLAMES-specific set of routines used by FPIL to provide information about the instrument. |
| Slalib | Pat Wallace's set of astrometric routines. |

# A Walk through the code

This section attempts to clarify the operation and structure of the FPOSS code by walking the reader through some of its operation, following the code as it starts up and as it goes through a relatively simple configuration.

## Startup

The main() routine is in configure.c. When FPOSS starts, this is executed first. Its role is to perform any basic setup needed and hand over to Tcl.

The key line in main() is:

Tk_Main(argc + 1, newArgv, ConfAddCommands);

where it passes over control to Tcl/Tk, in such a way that ConfAddCommands() is called as part of Tcl initialisation to define all the new C-implemented Tcl commands needed for FPOSS. It has also modified the command line arguments it was passed by inserting the name of the configure.tcl file as the first argument.

So main() starts up Tcl/Tk with a large number of new C-implemented commands (all defined in configure.c - although they make use of code in the other .c files) and with control passing to configure.tcl.

From this point on, FPOSS should be thought of as a Tcl/Tk program. Although a lot of the heavy duty work is done by the C routines, they are all sequenced through Tcl/Tk and all are structured as Tcl commands.

More details about main():

- It initialises the Mess sub-system.

- It sets the environment variables TCL_LIBRARY and TK_LIBRARY in the case where the Tcl/Tk ancilliary files are in the directory from which FPOSS itself is run. (This is important when considering the way the release file structure is created, and is described in more detail in the section on Tcl/Tk ancilliary files. Generally, when the program is being developed, on a machine that has Tcl/Tk installed, the Tcl/Tk ancilliary files are not picked up in this way, but when the program is installed on a some other machine this is the way it determines the location of these files.)

- It looks for the configure.tcl file as follows: If the environment variable CONFIGURE-_SCRIPT is defined, it takes this as the name of the file. If CONFIGURE_SCRIPT is undefined, it looks to see if the environment variable CONFIGURE_DIR is defined. If it is, it looks for the file 'configure.tcl' in the directory given by CONFIGURE_DIR. If neither of these environment variables are defined, it deduces the name of the excutable directory (the directory from which it - the FPOSS program - is being run) and looks for configure.tcl in there. (UNIX programs, strictly speaking, are not supposed to try to work out where they are being run from, since it is possible to run them in ways that defeat the only available mechanism, namely to look in the execution path for a program that matches the program name they are passed as argv[0]. However, in practice, this can be expected to work. If it doesn't, look at how the program is being run.)

- If running in batch mode (which it can deduce by looking at its command line arguments) it calls Tcl_Main() instead of Tk_Main().

## Tcl/Tk startup

The call to Tk_main() starts up Tcl/Tk. During Tcl initialisation it calls the initialisation routine it was passed, ConfAddCommands() (in configure.c), which defines all the C-implemented Tcl commands. We now have a Tcl/Tk program running.

Note that the main() routine in configure.c starts up Tcl/Tk with the command line arguments passed to it, with the full path name of the 'configure.tcl' script insterted as the first argument. This causes configure.tcl to be loaded, and control passes to the main-line code in this file.

This main-line code is found at the end of the file.

The main-line code does a number of things, but the most important are:

- It performs some initialisation of global variables.

- It locates the other tcl scripts that it needs, procs.tcl, mimic.tcl, and   util.tcl and loads them. (These have no main-line code that matters here,   and so loading these just loads a number of additional tcl procedures.)

- It processes the command line arguments, putting any relevant details into   a set of Tcl global variables.

- It looks to see how many instruments are supported, and puts up a dialogue   allowing the user to select the instrument (2dF, 6dF, FLAMES, etc) if support   for more than one has been built-in.

- It calls Initialise{}, which is implemented in C in configure.c as    ConfInitialise(). This is described in more detail below.

- Finally, it calls Setup{}, which starts up the GUI for the program.

The code for the command line arguments is quite long, but in practice  command line arguments are very rarely used, particularly in the ESO context, where 'configure' is used almost exclusively as an interactive program. The command line arguments can be used to specify initialisation details (you can specify the instrument explicitly to bypass the initial instrument-selection dialogue, for example), or they can be used to completely specify the processing to be performed, in which case the program may exit at this stage, before even calling Setup{}. However, for the purposes of this brief tour through the code, we'll assume we are running interactively and Setup{} will be called.

Initialise{} results in ConfInitialise() being called. This performs all the instrument-specific initialisations. It starts up the FPIL routines and these in turn (if FLAMES is being used) call the fpcol initialisation routines which read all the  instrument-specific files such as fpInsStatus.sds.

It is at this stage that one of the important internal data structures at the center of the configuration program is created. The structure describing the pivots - where they are, what their types are - is read from fpInsStatus.sds. Note that the configuration program uses a subtly different structure to that actually in the fpInsStatus.sds file, the contents of which are determined by the needs of the positioner code. So the routines in fpcolFpil.c are used to convert from the SDS structure actually in the file to the structure  required by FPOSS. (An important point is that fpcolFpil.c is the only fpcol file that is NOT used by the fibre positioner code. This makes it the obvious place for any FPOSS-specific operations.)

When 2dF or 6dF are being configured, the user can specify the plate for which the configuration is to be performed, the default plate being plate 0. This allows the pivot positions and types (and broken fibre information) to be that for the actual target plate. For FPOSS it is a a requirement that both plates (actually numbered 1 and 2) should be treated equally, and FPOSS is to create configurations suitable for either plate. So the code restricts the user to the one 'conceptual' plate numbered zero, and the code in  fpcolFpil.c generates a 'super-plate' structure that describes a combination of the two actual plates. In particular, plate 0 is handled by combining the broken fibre information for the two actual plates.

Tcl/Tk initialisation concludes with a call to Setup{}, implemented in configure.tcl. This routine

- Creates the main control panel, with fields to display the current    configuration details and the various menus.

- Calls CreateMimic{} (defined in mimic.tcl) to create the main mimic display.

- If an initial file has been specified, using the command line options (the    details of which it gets from the global variables set by the main-line    configure.tcl code earlier) it opens it.

- Calls Handholder{}, which puts up the step-by-step basic sequence window.

That concludes the setup of the program. Now FPOSS is running under the  normal Tcl/Tk event loop, and what happens next depends entirely on what GUI elements the user chooses to make use of next.

We will follow through the usual basic allocation sequence, selecting in turn each item in the basic sequence window put up by Handholder{}.

## Basic sequence – Magnitude Filter

The ‘magnitude filter’ button in the basic sequence window is tied to the Tcl procedure MagnitudeFilter{}. This puts up a dialogue whose main object is to set the two Tcl global variables LowMagLimit and HighMagLimit. This is all quite conventional dialogue code – when the ‘Apply’ button is pressed, ApplyMagnitudeLimits{} is called to validate the global variables tied to the entry boxes in the dialogue, and if they are reasonable, SetMagnitudeLimits{} is called to set the global variables LowMagLimit and HighMagLimit.

## Basic sequence – Open File

The ‘Open file’ button in the basic sequence window is tied to the Tcl procedure OpenAscii{}. This puts up a standard open file dialogue allowing an ascii input file (usually a .fld file) to be selected. When a file is selected, OpenAsciiOk{} is called. (This is a common convention in FPOSS Tcl code – a dialogue will be put up by a procedure called DoThis{} and there will be a DoThisOk{} procedure tied to the OK or equivalent button in the dialogue.)

OpenAsciiOk{} calls OpenAsciiFile{} to perform the actual file opening, and this in turn calls ConvertAscii{}, passing it the file name and the magnitude filter limits set by the previous step. (Before it does this, it uses CountAsciiObjects{} to do a preliminary pass through the file to see how many objects will be read given the magnitude filter settings.)

ConvertAscii{} does the bulk of the work here. It is implemented in C, causing ConfConvertAscii() to be called. (This is another common FPOSS convention – usually, a Tcl command called MyCommand{}, if implemented in C, will be implemented by a routine called ConfMyCommand(). This is not rigorously adhered to however. The proper way to find out what C routine implements a Tcl command is to look at ConfAddCommands() in configure.c, and search for the name of the Tcl command.)

ConfConvertAscii() calls tdFparseFieldData() to read the field data at the start of the input file, then calls  tdFparseObjectData() to read the details about all the targets into a linked list of structures each of which contains details about one target. TdFconvertToSds() then converts this linked list into the SDS structure containing the configuration data that lies at the heart of the configuration program. (This is the ‘Configuration Data’ shown in the FPOSS structure diagram at the start of this document.).

(You may want to have another look at section 5.2 of this document, which describes the configuration data structures in detail.) At this point, the ‘objects’ sub-structure has not been created, and the X and Y coordinates for the targets have not been calculated. If no time of day was specified in the input file, SetUTMeridian{} is invoked to pick a time of day that will place the field at the meridian, which is the best compromise when the time of day is unknown. MakeAllocated{} is then called to create the ‘objects’ structure, which at this point will shown no allocated objects and all fibres in their parked positions.

ConvertXY{} (implemented in C by ConfConvertXY()) is then used to perform the astrometric calculations needed to set the X and Y plate coordinates for the targets. It runs through all the sub-structures in turn performing the necessary calculations. Note that the code used to do the actual calculation, in ConfDoXYConversion() has to be able to work for all the slightly different sub-structures. For example, it knows that it has to do the delta calculations for guide stars. (There are a number of routines such as this in the C code, which are passed the SDS Id for the top level of one of the configuration data sub-structures, and have to be able to work no matter which sub-structure is actually being used. For example, such code cannot assume that there will be a ‘spectrograph’ item in the structure, since the guide sub-structure does not have one, and so has to use SDS enquiry functions to see just what is in the structure it has been passed.)

We are still in the Tcl procedure OpenAsciiFile{}. Finally, AddIndex{} is called to add the index and allocated items to the structures. Now the configuration data structure is complete. The pivot information structure was read during the program startup, so now FPOSS has both its main data structures present.

Control returns to OpenAsciiOk{}, which now displays the targets and fibres on the mimic display – which up to now has displayed nothing but the concentric circles showing the various field limits. The procedure ZoomMimic{} is always used to redisplay the field in the mimic display, and the code calls it whenever there are changes to be reflected in the display.

DeselectGuidePivot{} clears all the Tcl global variables relating to the VLT guide probe, and control finally returns to the Tcl/Tk event loop and to the user.

## Basic sequence – Select Guide Star

Clicking on the ‘Select guide star’ button in the basic sequence dialogue is unlike clicking on the other buttons in this dialogue. The selection of a guide star is interactive, and all it can do is put up a message reminding the use of what has to be done.

What has to be done is that the user has to double-click on one of the VLT guide stars in the mimic display. Double clicking on any target is tied to the ShowObject{} procedure, which puts up a dialogue box giving details of  the object. If the object is a VLT guide star, then it also works out the two possible pivot positions for the guide probe and draws these as blue circles in the mimic display. Clicking on one of these guide probe pivot positions is tied to SetGuidePivot{}, which then sets the Tcl global variables describing the guide pivot selection to show that this is the selected position.

SetGuidePivot{} also calls DisableShaddowedPivots{}, which handles the business of ensuring that targets under the guide probe are not allocated to fibres. (It should of course have been called DisableShaddowedTargets{}.) This calls VLTGuideShaddow{} for each target in the configuration data. Calling VLTGuideShaddow{} has the important side effect that any shadowed target has its type field modified in the configuration data. A ‘disabled’ bit is set, making this target incompatible with any fibre. (Note that the author of this code has always had a problem with the spelling of the word ‘shadow’, which is why all these routines have two ‘d’s in their name.)

## Basic sequence – Fibre combination

The ‘Fibre combination’ button in the basic sequence dialogue is tied to the procedure SelectFibreCombo{}. This puts up a dialogue that has a radio button for each combination of fibre types that is supported, and allows the user to pick one. When the ‘apply’ button is selected, ApplySelectFibreCombo{} is invoked. There is a global Tcl array called FibreTypeFlags that lets the Tcl layer maintain a list of which fibre types are in use. An equivalent array called UseFibreType is maintained by configure.c, and the C array is set to match the Tcl array by the procedure MakeFibreType{} (implemented by ConfMarkFibreType()). So the main effect of selecting a fibre combination is that the relevant elements of the C global array UseFibreType are set appropriately.

Note that the Tcl routines used here have been written rather abstractly. They have no idea what fibres are used, what combinations are allowed, etc. They have to get all this information from the FPIL layer (and ultimately, for FLAMES, from fpcol). So they do a lot of calling of procedures that are implemented in C and which call Fpil routines to get the necessary information. This makes the code less clear than it might otherwise have been, but allows a variety of instruments to be supported. (It also allows different combinations of fibres to be supported easily – only the underlying FPIL enquiry routines have to be modified; the Tcl code remains unchanged.)

Things are now ready for an allocation.

## Basic sequence – Allocate

This is, of course, the important bit. The allocation is performed by the routines in config.c and config_method.c. The routines in config.c are usually described as the ‘housekeeping’ routines, while those in config_method.c implement the actual allocation algorithm. The separation of the two was intended to make it easy to experiment with different allocation algorithms. There is a clearly defined interface between the two. The housekeeping layer creates a set of C structures that describe the targets and the fibres, and passes these to the actual allocation algorithm routines.

The housekeeping layer is passed the SDS Id of the configuration data structure and processes it to create the data structure shown as ‘Allocation Data’ in the FPOSS structure diagram at the start of this document. This is a number of C data structures. All of this is described in detail in the AAO document “Fibre Instrument Configuration Allocation Routines”, a copy of which is part of the FPOSS documentation.

There are also a number of parameters that can be used to control the details of the allocation algorithm. Since these can differ depending on the implementation of the allocation algorithm, and since some are intended only for use in ‘expert’ mode, the Tcl/Tk code has no idea of the details of these parameters. Instead, it has to obtain them through a series of calls to C routines that make use of the enquiry functions built into the allocation housekeeping layer.

All of this complicates the construction of the dialogue put up by Allocate{}, which is the Tcl procedure tied to the ‘allocate’ button in the basic sequence dialogue. It has to get the parameter details through a call to ParameterDetails{} and then for each parameter it has to handle it depending on its type (logical, numeric, etc). The start of the code in config.c contains a number of declarations of static arrays that contain the algorithm-independent parameter descriptions. Similarly, the start of the code in config_method.c contains a number of declarations of static arrays that contain the algorithm-dependent parameter descriptions.

Finally, AllocateOK{} is invoked when the user accepts the various parameter settings, and this in turn passes control to DoAllocate{}, having set all the various parameters to the specified values. DoAllocate{} is implemented in C in ConfDoAllocate(), and this does very little other than call the basic sequence of allocation routines defined in the interface to the housekeeping layer. It calls ConfigSetTargets(), which handles the basic construction of the allocation data from the configuration data, then calls ConfigSetSpecPivotsFlag(), which allows the various pivot types to be enabled or disabled (depending on the values of the elements of the global C array UseFibreType – see the previous section). It then calls ConfigAllocate(), which performs the allocation.

A report function is passed to ConfigAllocate(), and this provides feedback to the user. This is ConfAllocRpt(), and this will be called by the allocation algorithm whenever a fibre is allocated or deallocated. ConfAllocRpt() executes the Tcl command AllocRpt{} when called, and it is this that allows the mimic display to be updated as the allocation procedes. (The low-level routines that do the actual work here are AllocateFibre{} and DeallocateFibre{}, which are implemented in C in configure.c.) It is also this call that updates the contents of the configuration data structure. (There is no final call that allows the configuration data structure to be updated to reflect the final allocation – it has to be done by monitoring the progress of the allocation through the callback mechanisms.)

## Basic sequence – Write PAF

Finally, the ‘WritePAF’ button in the basic sequence dialogue is tied to the SaveAsPAF{} procedure, which puts up a standard file save dialogue and calls DoSaveAsPAF{} when a file is selected. Most of the real work in DoSaveAsPAF{} is performed by WritePAF{}, implemented as a C routine, ConfWritePAF(). This works through the configuration data structure and writes the PAF file in the standard format.

And that’s that.

# Aspects of the Code

This section focuses on a number of  aspects of the code that may prove confusing to the maintenance programmer. Although FPOSS is a relatively starightforward Tcl/Tk program, it is a large Tcl/Tk program and it has a number of idiosyncracies. If you find yourself having to work in some of the areas covered here, you should make a point of reading the relevant section first. It also includes section sof generally useful information such as a complete list of canvas tags used in the mimic display.

## Tcl/Tk ancillary files

Tcl and Tk are systems that make use of a large number of ancillary files. There are a large number of .tcl files that have to be loaded to provide some of the facilities provided by Tcl/Tk, together with a number of other files. In the FPOSS release, two directories, tcl_dir and tk_dir, are provided that contain these files. It is worth knowing how Tcl and Tk locate these ancillary files, and how this is controlled by FPOSS.

When Tcl is built on a machine, the build procedure notes where in the file system Tcl is being built, and this knowledge is built into the executable images produced and the Tcl/Tk libraries built on that machine. This means that any Tcl program can find these files without needing any other mechanism to be set up - no environment variables need be set, for example.

However, if the ancillary files are moved, Tcl/Tk will no longer be able to find them. More relevantly to FPOSS, if a linked Tcl/Tk executable is moved to another machine, it will look for the ancillary files in the location it used for them on the machine on which it was originally built. Even if the new machine has the same version of Tcl/Tk installed, it will not be able to find the files unless they happen to be in exactly the same location as on the machine used for the original build. This is unlikely. Moreover, if the new machine has a different release of Tcl/Tk, the ancillary files are usually incompatible.

Since FPOSS is released as a ready-built Tcl/Tk executable, it faces this problem. It clearly has to be released together with the ancillary files that go with the version of Tcl/Tk used to build it. There also has to be some mechanism to allow it to find them.

The location used by the Tcl ancillary files can be determined by running up a Tcl program and executing the line:

puts $tcl_library

and the location used by the Tk ancillary files can be determined by running up a Tcl/Tk program and executing the line:

puts $tk_library

The location in which Tcl and Tk look for these libraries can be changed by setting the environment variables TCL_LIBRARY and TK_LIBRARY. It would be asking too much for FPOSS to require that these be set by the user before running it. So instead, the main() routine in configure.c looks explicitly in the directory from which it is run (the 'executable' directory) for the two directories tcl_dir and tk_dir. If it finds them, it sets the TCL_LIBRARY and TK_LIBRARY environment variables to point to these directories.

As an additional check, it looks in tcl_dir for a file called TCL_VERSION_n.m where n.m is the version number for the version of Tcl that it (the FPOSS executable) was linked against. (It gets the version number from the value of the pre-processor variable TCL_VERSION, which is set by the Tcl include files.) Similarly, it looks in tk_dir for a file called TK_VERSION_n.m, n.m in this case being taken from the pre-processor variable TK_VERSION. If FPOSS does not find these check files it outputs a warning message, but continues to run.

Since it does all this before it calls Tk_Main() (or Tcl_Main() in batch mode), when Tk_Main() is eventually called, it will find that TCL_LIBRARY and TK_LIBRARY have been set to the location of the ancillary files and so will find them without any problem.

Usually, when FPOSS is being developed on a machine that has Tcl/Tk installed, these directories are not present, and so TCL_LIBRARY and TK_LIBRARY are  not set, and Tk_Main() finds the ancillary files using the location that has been built in to the Tcl/Tk libraries.

When the release tar file is built, the release process defined by the Makefile involves building the configure executable (the FPOSS program) and then running it twice, first with the -tcl_library flag set and then with the -tk_library flag set. These cause the program to do nothing but execute the puts statements shown above to output the locations used for the ancillary files. The resulting output is used by the makefile to locate the ancillary files and copy them into the tcl_dir and tk_dir directories that are then included in the release tar file. This scheme has the advantage that it ensures that the ancillary files included in the release are those actually used by the executable (because the release procedure has actually involved asking the executable which files it is using!). It has the minor disadvantage that making the release tar file requires that the DISPLAY environment variable be set (because without this, Tk will refuse to start up), even though the procedure doesn't actually involve displaying anything.

The TCL_VERSION_n.m and TK_VERSION_n.m version check files are not included in the Tcl/Tk distribution, and have to be created by the Makefile. To do so, it runs configure twice more, once with the –tk_version flag set and once with the –tcl_version flag set. These flags cause the program to output the Tcl and Tk version numbers that it is using, and these are incorporated into touch statements that create the files. (The –tcl_version flag is implemented in C using a printf() call to output the value of the TCL_VERSION pre-processor variable, and the –tk_version flag is implemented using a puts tcl statement to output the value returned by [info tclversion].

Then, when the release tar file is un-tarred on the user's machine, when FPOSS is run, it will find the tcl_dir and tk_dir directories in the directory from which it is being run, and will set TCL_LIBRARY and TK_LIBRARY properly before starting up Tcl/Tk.

## Error reporting

Error reporting is very necessary, but in FPOSS it is complicated by:

1) The requirement to have error messages displayed nicely as dialogue boxes rather than just as console messages.

2) The fact that errors may occur at the startup phase, or in batch mode, in which case there is no graphical system available and the console will have to be used instead.

3) The number of different ways in which errors can occur. They can be generated by Tcl/Tk itself, or by Tcl code, or by C code.

4) The use of the Ers system to handle error reporting, and the fact that there are C and Tcl implementations of the various Ers routines.

Ers (Error Reporting System) was introduced for the AAO's DRAMA data acquisition environment (for which FP OSS was originally written). The version of Ers distributed with FP OSS is a standalone version written to go with the standalone version of the IMP message system. It provides most of the functionality of the DRAMA IMP system, but does not need the full DRAMA system to be available in order for it to be compiled and used.

The following description of Ers use in FP OSS probably makes for hard reading. It is intended as a relatively compact statement of all the relevant facts about Ers as implemented for FP OSS. It might help make the operation of some of the error handling code a little easier to understand. It forms background information for the 'worked examples' of error reporting that follow.

The general principles behind the use of Ers are that ErsStart() is used to initialise the system. ErsOut() results in an error message being reported immediately. ErsRep() is used to defer reporting of a message until higher level code decides whether to annul it (using ErsAnnul() ), or to flush it out (using ErsFlush()). This allows details of an error to be reported by the part of the code that knows all the details, without worrying the user with error reports generated as the result of failed tentative operations that the higher level code can work around. ErsFlush() outputs any error messages logged using ErsRep() that have not been annulled.

ErsStart() is called from configure.c as part of the execution of the Tcl command Initialise{}. It specifies an output handler ErsHandler() , also defined in configure.c, which from that point on the C routines ErsFlush() and ErsOut() will call to output their messages.

ErsRep() provides deferred error reporting. It allows a routine to make a  tentative call to something that may report an error using ErsRep(), and then decide whether to work around the problem (in which case it can call ErsAnnul() to clear out the message) or to call ErsFlush() to output the message to the user. ErsRep() is only invoked from C - no ErsRep{} is provided. It stacks up the message it is sent, allowing it either to be cleared by a subsequent call to ErsAnnul() , or flushed out to the user through a call to ErsFlush(). The C routine ErsRep() is implemented in sds/ers_imp.c, together with the other Ers C routines such as Configure.tcl, ErsAnnul() , etc.

The Tcl command ErsOut{} is implemented in util.tcl. Normally, it logs the message to the message area in the main command window and pops up a dialogue (using DtclTkDialog) containing the text of the error message. However, in places util.tcl overrides ErsOut{} temporarily to intercept the messages it is called to output. In particular, this happens in bgerror{}. Note that ErsOut{} is not implemented as a simple call to ErsOut().

ErsOut{} is used by any Tcl code that detects an error - such code usually reports it using ErsOut{} and then returns TCL_ERROR.

The Tcl command ErsFlush{} is implemented in C in configure.c and simply calls the C routine ErsFlush(). Since there is no ErsRep{}, the only use of ErsFlush{} is to flush out error messages logged by C routines called from Tcl.

The C routine ErsFlush() is implemented in sds/ers_imp.c. If Configure.tcl has been called it will call ErsHandler() with a package of all the error messages logged to that point. Otherwise (and this only happens for error messages generated before Initialise{} is invoked from Tcl), it will call ErsOut() for each logged message and ErsOut() will print them on the terminal.

The C routine ErsOut() is implemented in sds/ers_imp.c. If Configure.tcl has been called it will call ErsHandler() with the error message in question. Otherwise (and this only happens for error messages generated before Initialise{} is invoked from Tcl), it will print it on the terminal.

ErsHandler() collects all the package of messages it is passed (one if  called from ErsOut(), more if called from ErsFlush()) and invokes the Tcl command ErsOut{} to display them.

To try to make some of this clearer, the next sections work through some of just what happens when an error is detected by the FPOSS code.

### Errors generated by Tcl

To take the simplest case first,  errors generated directly by Tcl are handled as follows:

For example, if you manage to generate a Tcl run-time error, such as by deliberately introducing a reference to a non-existent variable, the following happens:

Say I introduce the line

if { $NON_EXISTENT_VARIABLE } { set NON_EXISTENT_VARIABLE 0 }

into the code and force it to be invoked.

This causes the new bgerror{} defined in util.tcl to be invoked, with the parameter (err) set to

"can't read "NON_EXISTENT_VARIABLE": no such variable"

The new bgerror{} contrives to pick up any messages stored in the Ers system, but since we assume there aren't any we ignore that for the moment. It then invokes the original (Tcl-supplied) bgerror{}. This in turn invokes the tkerror{} routine defined in util.tcl.

Note: The Tcl/Tk documentation says that tkerror{} is called by bgerror, so this will work properly. However, it cautions that, in later versions of Tcl/Tk, this behaviour may change.

tkerror{} will be passed the same err parameter as was passed to bgerror{}, and it also picks up the full error description from the global errorInfo. If the window system has not been started up, it just prints the error details. If the window system is available, it invokes DtclTkDialog{} to display the error to the user.

All DtclTkDialog{} really does is put up a window that displays the arguments it is passed (its messy-looking first argument is just used to create the name of the window, the leading '!' being a special character that is stripped from the name before it is used to generate a class name - too allow all the dialogs to be deleted by the 'clear all' button). Note that although the call specifies the names of the buttons, their functions are hard-coded – the first closes the dialogue, the second displays the stack information, the third clears all windows of the same class and the last puts up some help.

### Errors detected and handled entirely in C.

There are a number of places in the code where a Tcl command implemented in C (in configure.c) detects an error, makes one or more calls to ErsRep() to log error messages, calls ErsFlush() to make sure these messages are output to the user, and then returns TCL_OK. The Tcl/Tk layer is not aware that an error has occurred (usually it doesn't need to know - it has invoked the C code following a menu item selection, and so long as the error message appears there is no need for any Tcl code to take any action).

Assuming that Initialise{} (implemented through ConfInitialise()) has  successfully called Configure.tcl, the output routine used by ErsFlush() will have been set up to be ErsHandler(). In this case, the call to ErsFlush()  causes it to collect all the messages logged through ErsRep() and not yet  output to the user and to pass them to ErsHandler(). ErsHandler() will use Tcl_GlobalEval() to invoke ErsOut{}, passing it the concatenation of all the messages passed to ErsRep(). (The messages are separated by newline characters, which causes them to be formatted reasonably nicely.)

The ErsOut{} that is called is the usual one defined in util.tcl. (In some cases, the util.tcl code overrides the definition of ErsOut{} - see the next section - but this is not one of those cases.) It calls DtclTkDialog{} to put up the messages in a dialogue and also copies them to the message area of the main command window, as in the previous section. And that's all.

### Errors detected by Tcl code

Finally, to look at something more complex, errors detected by Tcl code are handled as follows:

For example, the Setup{} procedure in configure.tcl contains the following lines:

if [catch "OpenAsciiOk $ConfigFile" msg] {

ErsFlush

ErsOut \

"Error reading Initial object file $ConfigFile\n\n$msg"

}

The intention here is to signal an error to the user if the OpenAsciiOk{} procedure fails, but then to carry on. OpenAsciiOk{} calls OpenAsciiFile{}, which invokes ConvertAscii{}, which is a Tcl command implemented in C in configure.c, being handled by the routine ConfConvertAscii().

ConfConvertAscii() can fail in a number of ways. One obvious one is if the file it is called to convert is invalid. This will normally be picked up by tdFparseFieldData() or tdFparseObjectData(). These indicate an error by calling ErsRep() with the error details and finally returning an error code in their inherited status variable, status. The error code returned will be one of the error codes defined by the various modules used.

An SDS error code will be returned as one of the SDS__xxxxx error codes, for example. Any non-zero status value returned indicates an error, and ConfConvertAscii() (which is the lowest-level routine to actually be aware of the Tcl environment in which it runs) picks this up. It adds its own error report, calling  ErsRep() with the file name and the helpful message "Error reading file".Enough details should now have been passed to the Ers system to enable a useful error report to be made, and we just have to get these out to the user. ConfConvertAscii() also calls MessGetMsg(), passing it the status code that indicated the error. MessGetMsg() returns the standard error string associated with this message, which is more information that may be useful.

ConfConvertAscii() now does the standard thing for a C implementation of a Tcl command in FP OSS when a run-time error is detected in C code. It  returns the TCL_ERROR code, and sets the Tcl interpreter result string to a statement of the problem (in this case "Error converting Ascii file") with the error string returned by MessGetMsg() appended to it.

There are now two sets of error information available: the string returned to the Tcl interpreter, and the detailed information that has been passed to Ers.

The error code returned to the Tcl interpreter will cause bgerror{} to be called, and we're almost back where we were with the ordinary Tcl error described in the previous section. In this case, the err parameter will have the message generated in ConfConvertAscii(), ie "Error converting Ascii file", concatenated with the error string from MessGetMsg().

This time, however, before calling tkerror{}, the bgerror{} defined in util.tcl contrives to collect all the error messages passed to the previous ErsRep() calls into the global variable Dtclu(SavedMessage). It does this by temporarily replacing ErsOut{} with a new version that doesn't actually output the message but instead appends it to Dtclu(SavedMessage). It then calls ErsFlush{}, which in turn calls ErsFlush(). Assuming Initialise{} has been called, ErsFlush() will collect all the ErsRep() messages and pass them to its output routine, setup in Configure.tcl to be ErsHandler(). ErsHandler() collects the messages and uses the Tcl interpreter to execute ErsOut{}, passing it all the messages. What gets executed, of course, is the new, temporary, ErsOut{} which puts all the messages into Dtclu(SavedMessage).

At this point, if there were ErsRep() messages queued, they are all in  Dtclu(SavedMessage), and these are pre-pended to the basic error message in err and passed to DtclTkDialog{} which displays them just as in the case of the simple Tcl error.

## Use of canvas tags in the mimic display

FP OSS makes a lot of use of tags in the canvas used for the mimic display.

The following should be a complete list:

| fiducials | Any fiducial drawn in the canvas. Set by: DrawFiducials{} |
| --- | --- |
| frame | Items such as the various circles drawn as part of the overall display. Also the compass that shows N and E. Set by: DrawFrame{}, DrawFrameDecorations{}. |
| pivotcircle | The circle drawn to show the location of the pivots. Used to bind enter and leave actions. Set by: DrawFrame{}. |
| fieldedge | The circle drawn to show the edge of the field. Used to bind enter and leave actions. Set by: DrawFrame{}. |
| 28arcmin | The circle drawn to show the 28 arcmin circle. Used to bind enter and leave actions. Set by: DrawFrame{}. |
| 20arcmin | The circle drawn to show the 20 arcmin circle. Used to bind enter and leave actions. Set by: DrawFrame{}. |
| argus | The black box indicating the location of ARGUS. Used to bind enter and leave actions. Set by: DrawFrame{}. |
| lin[n] | The line representing fibre [n]. Used to bind enter and leave actions, and single and double-clicks. Set by: DrawAFibre{}. |
| fib[n] | The item[s] used to draw the button at the end of fibre [n]. (Note that fib[n] seems to be a misleading name, since it's the button, not the actual fibre.) Used to bind enter and leave actions, and single and double-clicks. Set by: DrawAFibre{}. |
| fibres | Used for anything used to draw a fibre. Comprises all the fib[n] and lin[n] items. Set by: DrawAFibre{}. |
| cone | Only used by the diagnostic (and only half-baked) routine. Cone{} to tag a cone drawn to show aspects of the Oxford allocation algorithm's operation. Set by: Cone{}. |
| obj[n] | Allocated object [n]. Used to bind enter and leave actions, and single and double-clicks. Can be a guide star or a program object. Set by: DrawOneObject{}. |
| uobj[n] | Unallocated object [n]. Used to bind enter and leave actions, and single and double-clicks. Only used for program objects. Set by: DrawOneObject{}. |
| uguide[n] | Unallocated guide star [n]. Used to bind enter and leave actions, and single and double-clicks. Set by: DrawOneObject{}. |
| obj | All the obj[n] items are also tagged as obj. Set by: DrawOneObject{}. |
| uobj | All the uobj[n] items are also tagged as uobj. Set by: DrawOneObject{}. |
| uguide | All the uguide[n] items are also tagged as uguide. Set by: DrawOneObject{}. |
| bluearrow | The arrow bitmap used to highlight a target if arrow highlighting is turned on. Set by: HighlightByCriteria{}. |
| redarrow | The arrow bitmap used to highlight a point in the mimic display clicked-on by the user. (The same bitmap is used for both the red and blue arrows.) Set by: SetClickPosition{}. |
| guidepivot1 | The pivot position for the VLT guide probe, in the first of its two possible positions. This is drawn in blue. Set by: ShowObject{}. |
| guidepivot2 | The pivot position for the VLT guide probe, in the second of it two possible positions. This is drawn in blue. Set by: ShowObject{}. |
| guidepivot | The selected pivot position for the VLT guide probe. This is drawn in red. Set by: ShowGuidePivot{}. |
| guideshaddow | The shadow used to indicate the area obsucred by the VLT guide probe. Set by: ShowGuidePivot{}. |
| selected | Applied to the selected fibre (both the line for the fibre and the button). Set by: SelectFibre{}. |
| oselected | Applied to the selected target object. Set by: SelectObject{}. |
| objects | All the target objects are tagged as 'objects', to allow them to be deleted in one go. Set by: DrawOneObject{}. |
| unhighlit | All objects that are not to be high-lighted are tagged as 'unhighlit'. Set by: DrawOneObject{}, HighlightByCriteria{}. |
| highlit | All objects that are to be high-lighted are tagged as 'highlit'. Set by:HighlightByCriteria{}. |
| highlitfib | All fibres that are to be high-lighted are tagged as 'highlitfib'. Since this is associated with the appropriate fib[n] tag, it is actually only the button that is highlghted, not the whole fibre. Set by:HighlightByCriteria{}. |

## What Routines Does FPIL call?

A number of new Fpil routines were defined in the process of adding FLAMES support to the configuration program, and these were placed in the file FpilExtras.c. In principle, the FPIL abstract interfaces should have been extended to cover these, and the actual code in FpilExtras.c should be spread between the fpcol routines and the equivalents for 2dF and 6dF. So far, this has not been done. Since it can be hard to follow the flow of control through the FPIL abstract interface, this is not seen as a particular disadvantage from the point of view of the maintenance programmer. But it does mean that the existence of FpilExtras.c has broken what should be a clear distinction between the FPIL layer and the instrument-dependent libraries such as fpcol.

For example, in configure.c you can find a calls to FpilEncodeTargetType() and to FpilColButFib(). Of these, FpilEncodeTargetType() has been added for FLAMES, and FpilExtras.c contains a routine called FpilEncodeTargetType(), which contains the complete code to perform the required function for all supported instruments.

On the other hand, FpilColButFib(), which is implemented properly in fpil.c, calls a routine of the correct type a pointer to which is supplied by the fpcol library. This is a pointer to the routine fpcolBaseColButFib(), which is defined in fpcolBase.c and does the actual work in the case where the  instrument in use is FLAMES.

The trick to following this through is to know that routines in fpil.c access the instrument-specific routines through a structure called routines. So FpilColButFib() will make use of a pointer called routines->ColButFib.  This means that you can grep for "ColButFib" in the instrument-specific code, and you will find that in its initialisation phase fpcolFpil.c executes a line:

routines.ColButFib  = fpcolBaseColButFib;

You might have guessed that the routine would be called fpcolColButFib(), but it isn't. With luck, having seen this here will make it relatively easy to navigate through the FPIL layer. Note that the name of the item in the routines structure is not always as obvious as in this case, and you may need to look at the fpil.c code or the FPIL documentation to see which is used.

## Calling Tcl from C

The structure of FP OSS is such that Tcl code frequently invokes C code, using the Tcl procedures implemented in C, but there are only a limited number of places where C code invokes Tcl code:

The progress reports (the bar showing percentage of an operation complete) are drawn by C callback routines that use Tcl_Eval() to execute an AllUpdate{}, AllocRpt{} or AllUpdateBatch{} command.

There are two places where fibre deallocation is handled from C code by invoking the Tcl routine DummyDeallocateFib{}.

The Xerror() routine attempts to use a call to Tcl_Eval() to display information about the details of an error, and may also attempt to execute the Tcl statement ‘exit 1’.

The routine Tcl_GlobalEval() is used in three places in the code (two in ErsHandler() and one in ConfMessage()). In all three cases, this is done in order to arrange for a message generated by a  C routine to be displayed using the Tcl/Tk GUI. ErsHandler() invokes ErsOut{} and ConfMessage() invokes MsgOut{}.

## Allocation tolerances

When FPOSS allocates fibres, it makes use of a number of tolerances. These control such things as the minimum necessary clearance between two fibres, the maximum bend angle for a fibre, etc. When FPOSS checks a field for validity, it is important that the values used for these tolerances are the same as (or at least, no less tolerant than) those used by the actual positioner. If this were not the case, FPOSS would be capable of generating configurations that would be rejected by the positioner.

However, if FPOSS were to use exactly the same tolerances as the positioner when allocating fibres, there would inevitable be some fibres that were right on the limit of the allowed tolerances. Any change in hour angle or other observing parameters would then risk invalidating the configuration. Remember that FPOSS configures assuming a specific hour angle and a specific set of observing parameters such as wavelength. It is also important that a configuration be valid over a range of observing wavelengths.

For these reasons, FPOSS uses a more constraining set of tolerances when it configures a field. The intent is that a configuration should be valid over the full wavelength range, and over a range of +/- four hours in hour angle. It is not possible to determine the actual values for the more constraining tolerances analytically, and the values used by FPOSS have been determined purely by trial and error over a number of fields. Note that if the tolerances are set to be unnecessarily constraining, the percentage of fibres that can be allocated will be reduced.

The main code that handles the tolerances is in config.c. The code in here defines a constant called CONFIG_PERCENT, currently set at 15. The code that uses this applies it so that all the value for all the various clearance parameters are set to be more constraining by that percentage. Fortunately for the code, it seems that one single percentage is quite satisfactory. The code that applies this makes use of the various Fpil routines, such as FpilGetButClear() to access the positioner tolerances for the instrument in use. It then applies CONFIG_PERCENT to get the more constraining tolerances.

The code that provides the values returned by such routines as FpilGetButClear() is found in the instrument-dependent modules called by the Fpil routines. For FLAMES, the relevant definitions are found in the file fpcolInstDescr.h. This defines the following items:

| fpcolInstDescr.h name | Fpil routine | Current value | Meaning |
| --- | --- | --- | --- |
| FpcolINST_BUTTON_CLEAR | FpilGetButClear | 160 | Maximum button/button clearance in microns |
| FpcolINST_FIBRE_CLEAR | FpilGetFibClear | 2560 | Maximum button/fibre clearance in microns |
| FpcolINST_MAX_PIV_ANGLE | FpilGetMaxPivAngle | 26 deg | Maximum bend angle as fibre leaves the pivot. |

The code also allows for an additional bend angle, which is the maximum angle at which the button can be bent relative to the fibre. This is only important for instruments that have rectangular buttons, where the concept is meaningful. For FLAMES this is not a meaningful concept and the maximum bend angle is zero.

## Makefiles

This section covers some of the more unusual aspects of the makefiles that are used to build FPOSS.

### Targets

The main Makefile has three targets:

configure	is the default target, and builds the executable FPOSS program, ‘configure’.

clean	cleans up any intermediate files created by the build process.

release	builds the release tar file that is to be distributed to observers wishing to use FPOSS.

### Sub-directories

The various sub-directories such as sds_src do not have their own makefiles. Although each such sub-directory contains the code for a relatively stand-alone system, since they are only supplied here as part of the overall FPOSS program, the only makefiles used are in the main src directory. These makefiles get their source files from the sub-directories, but place all the intermediate object files in the src directory. Only the SDS sub-system is handled using a separate library, and even this is placed in the main src directory. This is simple, but does not make the best use of the structure of the various sub-systems. It makes sense if the aim of the system is thought of as simple being to build FPOSS, rather than to provide the various sub-systems.

### System-dependent Makefiles

The main makefile, Makefile, does most of the work, particularly for creating the release tar files. However, the code used in FPOSS is such that there is not one simple set of  compiler options that works for all systems, so the actual complilation process is handled by one of a set of system-dependent makefiles. This is done quite simply. The rule to build the main executable, configure, is given simply as:

configure:

make –f Makefile_`uname` configure

where the `uname` part of the rule expands to one of ‘HP-UX’, ‘SunOS’, or ‘Linux’, depending on the system being used. Control therefore passes over to one of the system-dependent makefiles, ‘Makefile_HP-UX’, ‘Makefile_SunOS’ (note that under Solaris uname returns ‘SunOS’ ), or ‘Makefile_Linux’. This trick allows FPOSS to have system-dependent makefiles, while at the same time conforming to the ESO requirement that a simple ‘make’ command should make the required program. (It might be arguable that the ‘make’ should make the release tar file rather than the program, but for the moment the separate ‘release’ target has to be used for this.)

### RCS version number

The main Makefile contains some rather convoluted code to obtain the RCS version number for the system. How this works is described in detail in the comments in Makefile. The scheme used is inelegant, but does work under all the systems used.

## Changing Tcl and Tk versions

FPOSS was originally built using Tcl/Tk version 8.0. (The original 2dF configuration program was built using a much earlier version, and in some places there is conditional code that tests the TCL version and allows for differences between version 8.0 and earlier code.)

In principle, moving to a later version of Tcl/Tk should not cause any problems. There is no explicit code that assumes version 8.0 is being used. Most of the Makefiles do not refer to the Tcl/Tk version explicitly either. The one exception is the Makefile_HP-UX makefile, where the ESO distribution was found to need the makefile to refer explicitly to the libtcl8.0 and libtk8.0 files. Accordingly, it contains two Makefile variables called TCL_VERSION and TK_VERSION which will need to be changed.

In practice, moving to a new version of Tcl/Tk often causes problems. In the course of its development, FPOSS was also tested on a Linux system running version 8.3.  At this point it became clear that the Tcl routine DtcluFDialog{}, in util.tcl,  made use of some internal Tcl/Tk features that were no longer supported in version 8.3. This particular routine, introduced in DRAMA to avoid the use of tkwait in a real-time system, was not really needed in FPOSS and was replaced by more conventional tk_getSaveFile{} and tk_getOpenFile{} calls. There may other such version-dependent aspects of the code waiting to be found by even later versions of Tcl, but by now there should be very few of these.

## Source files

The files needed to build FPOSS are held in a simple directory structure that follows normal ESO practice. There is an overall fposs directory that contains a number of sub-directories each containing the source files for a different sub-system. There is an src directory that is the main working directory for building the system. This contains all the FPOSS-specific source code together with the necessary files for building and running the program. Following normal ESO practice, the program can be built by setting the default to the src directory and typing ‘make’.

The source files comprise .h files, .c files and .tcl files, an overall Makefile and three system-dependent Makefiles that are called Makefile_<system>.

The section headed "Sub-Systems" describes the different sub-systems used by FPOSS, and you should read that to understand some of the following description.

## DRAMA files

The following files are those held in the ‘drama_src’ sub-directory. These are files that implement sub-systems that are provided by the AAO’s DRAMA data acquisition system.

### General-purpose files

| drama.h | This is a file taken from the AAO's DRAMA distribution. It defines a number of items that are used by DRAMA code to work around differences between compilers and systems. For example, on compilers that don't support 'void', DVOID is defined as a blank. Most modern compilers support all these constructs, but the use of this file means that DRAMA code doesn't depend on this while being able to make use of the constructs on systems provide them. DUNUSED is an example of a less common construct - it allows a routine parameter to be flagged as purposely unused. |
| --- | --- |
| status.h | This is a file taken from the AAO's DRAMA distribution. Its role may seem trivial - it has to produce the proper typedef so that StatusType is defined as the smallest integer of at least 32 bits. Having this exactly right is important for DRAMA, which passes status information around between tasks written in different languages and running on different systems. It doesn't matter so much for FPOSS, which could probably manage quite well having it just defined as 'int', but so much of the SDS and Ers code uses StatusType that it's worth having it defined properly. It also defines STATUS__OK which everyone who uses it knows is always zero, but we try not to make use of that. |

These files should be copies of those in the DRAMA distribution.

### Ers files

These are the files needed to build the stand-alone implementation of the Ers error reporting system used by FPOSS.

| Ers.h | Include file describing the Ers routines. |
| --- | --- |
| ers_imp.c | Code providing a stand-alone implementation of the main Ers routines. |
| erssprintf.c | The code for the utility routine ErsSPrintf(), which is just an implementation of sprintf() that checks for overflow of the output buffer. |
| ersvsprintf.c | The code for the utility routine ErsVSPrintf(), which is just an implementation of vsprintf() that checks for overflow of the output buffer. |

Of these files, Ers.h and ers_imp.c are copies of the same files distributed by AAO with the IMP messaging system (hence the name of the .c file), while erssprintf.c and ersvsprintf.c are copies of the DRAMA versions of these files.

### Mess files

These are the files needed to build the MESS message translation sub-system used by FPOSS.

| mess.h | Include file describing the MESS routines. |
| --- | --- |
| mess.c | C code implementing the MESS routines. |

These files should be copies of those in the DRAMA distribution.

## SDS Files

These files are all found in the ‘sds_src’ sub-directory.

These are the files that are needed to build the libsds.a library. SDS has two layers: there is the underlying SDS core system, all of whose routines are called SdsXxxx and then there is a small set of routines called ARG which provides easy access to SDS sub-structures where there are a number of items all present at the same level of the hierarchy.

| arg.h | Include file that defines all the ARG routines. |
| --- | --- |
| arg.c | The code for all the ARG routines. |
| arg_err.h | An include file that defines all the error codes returned by ARG routines. |
| arg_err_msgt.h | An include file that defines a structure that can be used by the mess library to convert an ARG error code into a text string. |
| sds.h | Include file that defines all the SDS routines. |
| sds.c | The code for all the core SDS routines. |
| sds_err.h | An include file that defines all the error codes returned by SDS routines. |
| sds_err_msgt.h | An include file that defines a structure that can be used by the mess library to convert an SDS error code into a text string. |
| sdstcl.h | Include file that defines the routines in sdstcl.c. |
| sdstcl.c | Code for routines that make SDS functionality available in Tcl. Each SDS routine has a Tcl equivalent, implemented in C in this file. This routine also implements the ARG routines for Tcl. |
| sdstest.c | Not actually used by FPOSS. This is the code for a test program that exercises SDS. |
| sdslist.c | Not actually used by FPOSS. This is the code for a program that lists the contents of an SDS structure that has been written to a file. It can be a useful diagnostic, for example it can be used to show the contents of the instrument configuration file used by FPOSS. |
| sdstclinit.c | The code that adds the SDS and ARG commands defined in sdstcl.c to a Tcl interpreter. This is separate from sdstcl.c because DRAMA performs this function in a slightly different way and this file is intended for use with non-DRAMA systems. |
| sdsutil.c | A number of additional SDS routines that are regarded as additional to the 'core' routines included in sds.c. This includes SdsRead() and SdsWrite(), which read and write SDS structures to and from files, and the useful utility SdsList(), which can be inserted as a diagnostic in code to dump the contents of an SDS structure. |

All of these files should be exact copies of those in the AAO's DRAMA release. The files arg_err.h, arg_err_msgt.h, sds_err.h and sds_err_msgt.h are all generated automatically from two files (arg_err.msg and sds_err.msg) which are not includes in this release. When a new error code is added to a DRAMA sub-system, it is added to its .msg file and a utility program called messgen is run to produce the corresponding two .h files. Since maintenance of FPOSS should not include the need to add error codes to SDS or ARG, and since it complicates the build process, this has not been included in the FPOSS source release. (If necessary, error codes could be added by hand to the generated files, but if this does prove necessary AAO would appreciate being told. Indeed, AAO would appreciate being told of any problems found in SDS or other DRAMA software. Bug fixes are particularly welcome - although these are packages that are now regarded as well-proven, so they should give few problems. No SDS bugs were found in the development of FPOSS.)

## SLALIB files

These files are all found in the ‘sla_src’ sub-directory.

A copy of the SLALIB code, all concatenated into one .c file, is included in the FPOSS files, but only for completeness to allow the code to be worked on conveniently on AAO machines that do not normally have Slalib included. Note that the C version of SLALIB is copyright and is not for general distribution. It was also needed because for some time the version of SLALIB commonly in use at AAO did not support the VLT.

| slalib.h | Include file describing the SLALIB routines |
| --- | --- |
| slamac.h | Include file defining macros used by the SLALIB routines |
| sla.c | A concatenation of all the SLALIB C code. |

## FPCOL files

These files are all found in the ‘fpcol_src’ sub-directory.

These files contain the code for the fpcol routines - the device dependent code  that provided details of the FLAMES fibre positioner. This library is used  in its own right, directly, by the OZPOZ control code, and is also able to  provide the interfaces needed to allow its use by FPIL.

| fitsio.h | A dummy file. The file fpcolMdlCalLibs.h uses the fitsio library in some circumstances and so includes fitsio.h. For FPOSS this is not needed, but rather than modify the standard fpcol source it was easier to include this dummy file to allow compilation to procede. |
| --- | --- |
| fpDefines.h | Include file defining a number of constants relating to the FLAMES fibre positioner, in particular the number and type of the fibres. |
| fpcolInstDescr.h | Include file defining a number of additional constants relating to the FLAMES fibre positioner, in particular to the button dimensions. |
| fpcolBase. | Describes the routines contained in fpcolBase.c |
| fpcolBase.c | The code for the basic fpcol routines. |
| fpcolFpil.h | Describes the routines contained in fpcolFpil.c |
| fpcolFpil.c | The code used to provide the interface between FPIL and fpcol. Note that this file is not used by the OzPoz fibre positioner code. |
| fpcolMdl.h | Describes the routines contained in fpcolMdl.c |
| fpcolMdl.c | The code for the astrometric modeling routines used by fpcol. |
| fpcolMdlCalLibs.h | Describes the routines found in fpcolMdlCalLibs.c. Although this .c file is not needed by FPOSS, the include file is referenced by other files. |
| fpcolMdlCalModel.c | The astrometric modeling code. |
| fpcolMdlCalModel.h | Describes the routines found in fpcolMdlCalModel.c |
| fpcolMdlFit.c | Routines that do a least squared fit of a model to OzPoz observations. |
| fpcolMdl_err.h | An include file that defines all the error codes returned by the fpcolMdl routines. |

These files should be copies of those used by the OzPoz fibre positioner. For additional details about keeping these synchronised with the fibre positioner code, see the section on ‘Keeping FPOSS and FP in synch’.

## Configure (FPOSS) files

These are the files contained in the main ‘src’ sub-directory.

These files contain the code that is specific to the configuration program (FPOSS). In some cases the names reflect the code's 2dF origins. The files connected with the actual allocation algorithm are listed separately.

| conf_err.h | An include file that defines all the error codes returned by the high level configuration routines. |
| --- | --- |
| conf_err_msgt.h | An include file that defines a structure that can be used by the mess library to convert an error code defined in conf_err.h into a text string. |
| configure.c | The main C code for configure (FPOSS). This contains the main() routine, the C code that implements the various new Tcl commands needed, and a number of additional routines that these need. |
| configversion.c | A small file that provides the version number for the program, based on the pre-processor variables CONFIG_VER and CONFIG_DATE. |
| convertPAF.h | Include file defining the routines in convertPAF.c. |
| convertPAF.c | The code that reads in a PAF file and creates the internal SDS structures used by FPOSS. Note that configure.c contains the code that writes a PAF file. |
| tdFconvert.h | Include file defining structures and routines for both tdFparse.c and tdFconvertSds.c. |
| tdFconvertSds.c | Code that converts the internal structures returned by the routines in tdFparse.c into the SDS structures used by the main configuration code. Note that this converts |
| tdFparse.c | Code that reads the initial text files produced by users and converts them into internal C structures that can be read by the routines in tdFconvertSds.c. |
| configure.tcl | The main Tcl code for FPOSS. This file is loaded from during execution of the main() routine in configure.c. |
| mimic.tcl | Tcl code for controlling the mimic display that shows the location of the targets and fibres. This file is loaded by the main-line code in configure.tcl. |
| procs.tcl | A set of procedures that were originally common to both the configuration program and to the the AAO 2dF control program. Almost all of these are connected with the use of dialogue boxes. |
| util.tcl | This is a stand-alone version of a set of Tcl procedures used by DRAMA programs. Amongst other things, it provides the error reporting procedures and a number of utilities for putting up message regions such as that used by the main control panel. |

The following four files are not strictly used by FPOSS, but are needed by the configuration program when it is built with support for 2dF and 6dF. Rather than introduce conditional compilation code to eliminate the need for them when FPOSS is being built (this is one place in this manual where we differentiate between the programs 'configure' and 'FPOSS'), we simply include them in the set of files anyway.

| sixdf_util_err_msgt.h | An include file that defines a structure that can be used by the mess library to convert a 6dF error code into a text string. |
| --- | --- |
| sixdffpil.h | An include file that defines the 6dF interfaces to FPIL. |
| tdffpil.h | An include file that defines the 2dF interfaces to FPIL. |
| tdfxy_err_msgt.h | An include file that defines a structure that can be used by the mess library to convert a 2dF error code into a text string. |

The code files that implement the routines defined in sixdffpil.h and tdffpil.h are not included with the FPOSS release. When Makefile_Solaris is  used to generate the version of configure that supports all systems, it picks  the 2dF and 6dF code up from separate directories. The structure of the FPIL  layer, which refers to all these routines indirectly through pointers in a   structure, means that none of these are referred to in the code unless  a call to TdfFpilFullInit() and sixdfFpilFullInit() are included in the code.  When FPOSS is built, the FLAMES_ONLY pre-processor variable is defined and  this prevents the references to these routines being compiled.

### FPIL files

These files are the source code for the FPIL (Fibre Positioner Interface Library) sub-system. This provides the instrument-independent interface between the configuration code and the OzPoz-specific code module, fpcol.

FPIL is a DRAMA sub-system, although to date its main use has been for OzPoz. The original version of the configuration program (for 2dF) did not use FPIL, which was introduced when it was realised that the need to support an increasing number of instruments meant that an instrument-independent layer was going to be needed. Since they are in practice FPOSS-specific files, they have been included in the main ‘src’ sub-directory.

| fpil_err.h | An include file that defines all the error codes returned by FPIL routines. |
| --- | --- |
| fpil_err_msgt.h | An include file that defines a structure that can be used by the mess library to convert an FPIL error code into a text string. |
| fpil.h | Include file that defines all the FPIL routines. |
| fpil.c | The code implementing the 'core' FPIL routines. |
| FpilExtras.c | Code implementing a number of additional FPIL routines that were found to be necessary to support FLAMES. These should eventually be included in the 'core' routines. Note that this file includes the instrument-specific code for these routines as well as the instrument-independent interface, as noted in the sections describing FPIL in more detail. Some of this code should therefore really be in fpcol, some in fpil.c, and some in the 2dF and 6dF equivalents of fpcol. |
| fpilmodel.c | The code for all the FPIL routines concerned with access to the telescope model. |
| fpilcons.c | The code for all the FPIL routines concerned with access to the instrument constants. |

All these files should be copies of the files in the DRAMA version of FPIL. Obviously, a number of changes were made to FPIL during the development of FPOSS, and it is quite likely that anyone maintaining the FPOSS code will find the need to make further changes. If so, AAO would appreciate being informed so that the changes can be reflected in the AAO version.

### Allocation algorithm files

These files contain the code used by the allocation algorithms. These are  split into two parts, the algorithm-independent housekeeping code, and the  algorithm-specific code. Although FPOSS supports both the 'Oxford' and the  'Taylor' allocation, these are both implemented in the same algorithm-specific  code. (Which one is used depends on one of the parameters of what the  the housekeeping layer sees as just one algorithm.)

| tdFconfig.h | Include file defining all the structures and routines associated with the algorithm-dependent housekeeping code. |
| --- | --- |
| tdFconfigMethod.h | Include file defining all the structures and routines associated with the algorithm-specific code. |
| config_err.h | An include file that defines all the error codes returned by the allocation routines. |
| config_err_msgt.h | An include file that defines a structure that can be used by the mess library to convert an error code generated by the allocation routines into a text string. |
| config.c | The code for the algorithm-dependent housekeeping routines. |
| config_method.c | The code for the algorithm-specific allocation routines. |

## Additional files

These are a set of additional files that are needed for FPOSS to be useful, but which are not source code files.

| aao_logo.bmp | The bit map used to generate the AAO logo in the top-right corner of the main control panel. |
| --- | --- |
| eso-logo.bmp | The bit map used to generate the ESO logo in the top-left corner of the main control panel. This file was generated by hand for FPOSS from a standard ESO logo .gif file – normal scaling of images to such a small size using image handling programs produced unacceptable artifacts. |
| fpInsStatus.sds | The OzPoz instrument status file. This is what is read to determine the details for each of the fibres in the system. It lists for each plate the different fibre types, pivot and park positions. |
| N3201-00.fld | A test field supplied by ESO and used for the basic walk through the program described in the user manual. |
| sixdFconstants.sds | The 6dF equivalent of fpInsStatus.sds. Not needed when the configuration program is used in its FLAMES-only mode as FPOSS. (The 2dF equivalent file is always found in a standard location at AAO, so is not usually included in a release of the program anyway.) |
| M67.fld | A second test field, also supplied by ESO. |
| vlt1.fld | A third test field, with a very limited number of targets. |

Of these files, the only really important one is fpInsStatus.sds, which needs to be the most recent version of the file, reflecting the current state of the instrument as far as is possible.

## Documentation files

The following documentation files are included with the source distribution:

| fposs_user_manual.doc | The user manaul for FPOSS. This is a Microsoft Word file. |
| --- | --- |
| fposs_internals.doc | The internals manual – this very manual, in fact. This is also a Microsoft Word file. |

## Makefiles

The following makefiles are used by the build process:

| Makefile | The main makefile for the system. |
| --- | --- |
| Makefile_SunOS | The Sun-specific makefile used for any system-dependent parts of the build process |
| Makefile_Linux | The Linux-specific makefile used for any system-dependent parts of the build process |
| Makefile_HP-UX | The HP-UX-specific makefile used for any system-dependent parts of the build process |

# Keeping FP and FPOSS in sync

It is vitally important that the configuration program, FPOSS, and the main positioner code, FP, agree completely about all aspects of the positioner. This includes the current state of the instrument, the dimensions of such things as buttons and fibres, and the astrometric model and its parameters. Fortunately, the number of things that need to be kept synchronised is relatively small.

The current state of the instrument – which fibres are in use, exactly where their pivot positions are, etc, is kept in the instrument status file, fpInsStatus.sds. This also contains the latest values of the astrometric model parameters for each plate. The latest version of this file should normally be distributed with FPOSS. (It is possible that there are circumstances where one might not really want to use the latest version of the file – for example, the instrument may be temporarily in an unusual state, perhaps with most of the fibres disabled for some reason connected with maintenance. What should really be distributed is an ‘ideal’ file that represents the way the instrument can be expected to look when the observation is actually going to be made. In most cases, this is not going to be an issue.)

The basic dimensions of the instrument, including button sizes etc, are contained in the file fpDefines.h, which FPOSS keeps in its fpcol_src sub-directory. This file comes from the main fp module in the positioner code.

The code for the astrometric model used by both the positioner and FPOSS comes from routines in the fpcol module of the positioner code. What FPOSS keeps in its fpcol_src directory is the fpDefines.h file from the fp module, and the necessary subset of files from the positioner’s fpcol module. Section 8.12 describes the contents of the FPOSS fpcol_src directory in detail.

From this, you can see that keeping the positioner and FPOSS in sync is just a question of a) making sure FPOSS has the correct version of fpInsStatus.sds, and b) making sure that the files in FPOSS’s fpcol_src directory match the versions in the positioner code. Since the fpcol_src files come from more than one place in the positioner code, I have used the following shell script to collect up the required positioner files for FPOSS.

#!/bin/csh

#

# Copies the fpcol and fp files needed by FPOSS into a tar

# file.

#

mkdir fposs_fpcol_src

cp fp/include/fpDefines.h fposs_fpcol_src

cp fpcol/ws/include/fpcolBase.h fposs_fpcol_src

cp fpcol/ws/include/fpcolFpil.h fposs_fpcol_src

cp fpcol/ws/include/fpcolInstDescr.h fposs_fpcol_src

cp fpcol/ws/include/fpcolMdl.h fposs_fpcol_src

cp fpcol/ws/include/fpcolMdl_err.h fposs_fpcol_src

cp fpcol/ws/include/fpcolMdlCalLibs.h fposs_fpcol_src

cp fpcol/ws/include/fpcolMdlCalModel.h fposs_fpcol_src

cp fpcol/ws/src/fpcolBase.c fposs_fpcol_src

cp fpcol/ws/src/fpcolFpil.c fposs_fpcol_src

cp fpcol/ws/src/fpcolMdl.c fposs_fpcol_src

cp fpcol/ws/src/fpcolMdlFit.c fposs_fpcol_src

cp fpcol/ws/src/fpcolMdlCalModel.c fposs_fpcol_src

tar -cvf fposs_fpcol_src.tar fposs_fpcol_src

gzip fposs_fpcol_src.tar

If you go to a suitable directory and do ‘cmmCopy fp’ and then ‘cmmCopy fpcol’ you will get sub-directories fp and fpcol with the latest versions of the necessary files. Running the above script will then collect up the files needed by FPOSS and put them into a file called fposs_fpcol_src.tar.gz. This file can then be uncompressed and un-tar’ed into the FPOSS fpcol_src directory. This is the technique I used at AAO where I did the original FPOSS work on a number of machines, not all of which had cmm installed. It should be possible to work out a more satisfactory procedure more suited to the ESO development systems, but the file above shows what needs to be done.

# Expert Mode

FPOSS has an ‘expert’ mode. A normal user of the program should not need to use ‘expert’ mode. It provides a number of additional menu items, additional options in dialogues (particularly the allocation dialogue), and removes a number of restrictions. It is possible in expert mode to generate configurations that will be unacceptable to the observing system. For example, it allows combinations of fibres that are not valid for observing. Expert mode is the only way the test fibres used to set-up the instrument can be included in a configuration.

This section was originally included in the user manual, but this was felt to be inappropriate, so it has been moved over to this internals manual. If this section seems to have a slightly different style to the rest of the manual, this is because it was originally intended as user level documentation.

## Invoking expert mode

Expert mode is invoked through an ‘Expert mode’ item in the ‘Options’ menu. This is a toggle that takes the program in and out of expert mode. Normally, this item is not available in the ‘Options’ menu. It appears only if the program is being run from an executable file called ‘configure_expert’. (The program is the same – when it starts up it looks at its first argument to see under what name it is being run.) This can be a copy of, or more usually a soft link to, the ususal ‘configure’ executable.

## Menus in expert mode

This section shows all the control panel menus as they appear when expert mode is selected. In expert mode, new options appear at the bottom of all the menus (except for zoom, which is unchanged in expert mode). This section only provides explanations for those items that are unique to expert mode.

### The  File menu in expert mode

Figure 2 shows the File menu in expert mode. Note that this is shown when the configuration program is being used to configure FLAMES. For other instruments, SDS file format is the default rather than PAF, and this is reflected in the menu options.

Figure 2 The File menu in expert mode

The additional options that are available in expert mode are:

Open SDS	Puts up a file selection dialogue to open an SDS format file (usually a file saved as the result of a previous run of FPOSS). This will put the program back to the state it was in when that file was saved. If a file is already open and has not yet been saved, you will be asked if you want to save it before opening the new file.

Merge…	Puts up a dialogue that allows you to specify the name of an existing SDS format file (usually one saved during a previous run of the configuration program). The lists of unallocated targets in the current configuration (both guide and non-guide targets) are replaced by those in the specified file.

Save (^S)	Puts up a file selection dialogue that allows you to save the current configuration a file in the default format for the instrument. So if the program is producing configurations for FLAMES the save will be to a PAF format file. If FLAMES is not being used the save will be to a file in SDS format.

Save as SDS file	Puts up a file selection dialogue that allows you to save the current configuration as an SDS format file. This is the default file format used when configuring the 2dF or 6dF instruments.

Save Alloc to UnAlloc	This command is used to prepare a currently configured field for reconfiguration on the other plate. As such, it will not normally be used when configuring for FLAMES, where FPOSS ignores distinctions between plates. It copies the list of allocated targets into a new file as a list of unallocated targets. Configuring using this new file will ensure that only those targets originally configured will participate in the new configuration for the new plate.

### The View menu in expert mode

Figure 3 shows the View menu in expert mode.

Figure 3 The View menu in expert mode

The additional options that are available in the view menu in expert mode are:

Show empty fibre positions	In the mimic display, only fibres that are directly participating in the current configuration are normally shown. In additon, there are a number of fibre positions that are given numbers, but do not actually have fibres. These are also usually not shown. If ‘show empty fibre positions’ is selected, all fibres (even those that are only placeholders for non-existent fibres) are shown in the display. This option is only active if empty fibre positions are not being shown.

Hide empty fibre positions	Reverts to the default mode where only only fibres that are directly participating in the current configuration are shown in the mimic display. This option is only active if empty fibre positions are being shown.

### The Options menu in expert mode

Figure 4 shows the Options menu in expert mode.

Figure 4 The Options menu in expert mode

The additional options that are available in the options menu in expert mode are:

Buffer Display Update	Puts the program into a mode where changes to the mimic display are buffered and applied only when the buffer is full. This makes the program run faster, but at the expense of a rather jerky display. In practice, the buffering code has changed recently and there is now little difference between the two buffering modes.

Full Display Update 	Puts the program into a mode where changes to the mimic display are applied as soon as the underlying configuration changes. This makes the program display smoothly, but at the expense of some additional execution time, particularly on X-terminals. In practice, the buffering code has changed recently and there is now little difference between the two buffering modes

Change Observation Date	Allows the date of the observation to be changed. This may invalidate any existing configuration, and so if fibres have already been allocated the configuration should be checked using the Check Allocation option in the Commands menu.

Select Fibre Type	Puts up a dialogue that allows the various fibre types to be selected independently. This allows non-standard fibre combinations to be selected, and so should normally be avoided. This is the only way to create a configuration that uses test fibres.

Refraction parameters	Puts up a dialogue that allows the refraction parameters built into the astrometric model to be modified.

Telescope Parameters	Puts up a dialogue that allows the telescope parameters built into the astrometric model to be modified.

### The Commands menu in expert mode

Figure 5 shows the Commands menu in expert mode.

Figure 5 The Commands menu in expert mode

The additional options that are available in the commands menu in expert mode are:

Set Wavelength	Puts up a dialogue box allowing you to specify the central wavelength of the observation in nm. This will change the plate positions of the various targets very slightly, and you should check the validity of the allocation (if there is one) using Check Allocation.

Rotate button	Allows a button to be rotated – this is only meaningful for instruments such as 2dF where button rotation is supported. For FLAMES and 6dF it has no effect and should not be used.

Import allocations	Puts up a dialogue allowing you to select a file that contains a list of fibre/target allocations and apply those.

Tcl command	Puts up a dialogue that allows direct Tcl commands to be entered. This is a very powerful facility, but needs intimate knowledge of the program’s internal structure and Tcl code to be useful. Anyone without such knowledge should avoid this command. Anyone with such knowledge will already know to avoid using this command. (It is mostly for the convenience of program developers, allowing features to be exercised for which no GUI control is available yet.)

## Additional target type codes

The following additional target types are accepted in an input text file, but ‘P’ is not expected to be used for FLAMES, and ‘T’ is strictly for experts only (‘T’ fibres are not included in any standard configuration and need to be selected in expert mode using the ‘Select fibre type’ item in the ‘Options’ menu. These types are accepted in both expert and non-expert mode, but are included here since the ‘T’ type is only useful in expert mode.

T	Test. This indicates a position used for testing. Normal observations will not include such objects.

P	Program object. A target that should be assigned to a single fibre (as opposed to a fibre bundle such as an IFU). For FLAMES this is equivalent to ‘M’ and ‘M’ should be used instead of ‘P’.

## The Allocation Dialogue in Expert mode

When the program is running in expert mode, a number of additional options are available in the allocation dialogue. The expert mode allocation dialogue is shown below in Figure 6.

Figure 6 The Allocate dialogue in expert mode

Number of fibres to leave for sky controls the number of fibres, Ns left parked during normal allocation. They can then be placed on sky at a subsequent step. If Ns > 0 no sky positions will be allocated during the initial object allocation stage. The value of Ns = 0 is special = in this case all targets in the catalog (both object and sky) will be allocated according to priority. This saves the extra sky allocation pass but may result in too many or insufficient sky fibres depending on the balance of number versus priority in the input catalog. Note that if more than one spectrograph is in use at once it is important to make sure that enough sky fibres are allocated to each. This information can be obtained from the Select Targets… option in the View menu.

___oOo___